APPL$ROOT:[DOCUMENTS.DOCUMENTATION]

GF3_MEDS_CODES.DOC

GF3 parameter code tables

Author: M. Jones, IOS.BIDSTON

Last updated: 7 Jan, 1988

Last MEDS additions (J.R. Keeley):  8 Jul, 1996

Last MEDS additions (Don Spear): 8 Sept, 1998

Last MEDS additions (Andre Bolduc): 24 Feb, 1999

Last MEDS additions (Bob Keeley): 11 Oct, 2000

Contents:

---------

1.0 GF3 codes sorted alphabetically

2.0 GF3 codes sorted alphabetically within GF3 tables

3.0 MEDS codes sorted alphabetically

4.0 MEDS codes sorted alphabetically within GF3 tables

1.0 GF3 Parameter Code Table sorted alphabetically

--------------------------------------------------

PPPP  KMMS       Parameter Name                          Units

ABSH  7XXA   ABSOLUTE HUMIDITY                          g/m**3

ALKY  7XXD   TOTAL ALKALINITY                           mmol/m**3

ALTG  7XXN   HEIGHT/ALTITUDE ABOVE GROUND LEVEL         metres

ALTS  7XXN   HEIGHT/ALTITUDE ABOVE MEAN SEA LEVEL       metres

AMON  7--D   AMMONIUM (NH4-N) CONTENT                   mmol/m**3

APER  7XXN   AVERAGING PERIOD FOR MEASUREMENT           CODE

(WMO CODE TABLE 2604)

ASTD  7XXB   AIR-SEA TEMPERATURE DIFFERENCE             degrees C

ATCK  7XXN   ANGLE OF ATTACK (AIRCRAFT)                 degrees

ATMP  7XXA   ATMOSPHERIC PRESSURE AT ALTITUDE           hPa

ATMS  7XXA   ATMOSPHERIC PRESSURE AT SEA LEVEL          hPa

ATPT  7XXA   ATMOSPHERIC PRESSURE TENDENCY              hPa/hr

ATRK  7XXN   ALONG TRACK DISPLACEMENT                   metres

BAND  7XXN   BANDWIDTH OF SPECTRAL ANALYSIS             hertz

BATH  7--N   BATHYMETRIC DEPTH                          metres

BDIR  7XXN   BANDWIDTH OF DIRECTIONAL ANALYSIS          degrees

BEST  7XXN   BANDWIDTH OF SPECTRAL COMPONENT            hertz

BRIT  7XXA   BRIGHTNESS (RADIATIVE) TEMPERATURE         degrees C

CALK  7XXD   CARBONATE ALKALINITY                       mmol/m**3

CAST  7--N   HYDROGRAPHIC CAST TYPE                     code

CCCC  7--N   DATA CYCLE OVERFLOW INDICATOR              code

CCVR  7XXA   TOTAL CLOUD COVER IN TENTHS OF SKY         ----

CFLG  7--N   DATA CONTINUATION FLAG                     code

CHAN  7XXN   NUMBER OF SENSOR CHANNELS                  ----

CHCH  7XXA   TYPE OF HIGH ALTITUDE CLOUD                code

(WMO CODE 0509)

CHLR  7--D   CHLORINITY (PARTS/THOUSAND)                g/kg

CHLS  7--D   CHLOROSITY                                 kg/m**3

CLCL  7XXA   TYPE OF LOW ALTITUDE CLOUD                 code

(WMO CODE 0513)

CLCM  7XXA   AMOUNT OF LOW/MEDIUM ALTITUDE CLOUD        code

(OKTAS - WMO CODE 2700)

CLDA  7XXA   TOTAL CLOUD AMOUNT (OKTAS-WMO CODE 2700)   code

CLDB  7XXA   CLOUD BASE ALTITUDE                        metres

CLDH  7XXA   CLOUD BASE HEIGHT (WMO CODE 1600)          code

CLDT  7XXA   CLOUD TYPE (WMO CODE 0500)                 code

CMCM  7XXA   TYPE OF MEDIUM ALTITUDE CLOUD              code

(WMO CODE 0515)

CNDC  7XXD   ELECTRICAL CONDUCTIVITY                    mhos/m

CNQF  7--N   CHEMICAL CONTENT QUALIFIER FLAG            code

CORG  7--D   ORGANIC CARBON CONTENT                     mmol/m**3

CPHL  7XXD   CHLOROPHYLL-A CONTENT                      mg/m**3

CTOT  7--D   TOTAL CARBON (C) CONTENT                   mmol/m**3

CVTQ  7--A   CO-VARIANCE OF AIR TEMPERATURE AND         degC.g/kg

SPECIFIC HUMIDITY

CVWQ  7--A   CO-VARIANCE OF WIND SPEED AND SPECIFIC     (m/s).g/kg

HUMIDITY

CVWS  7--A   CO-VARIANCE OF WIND SPEED COMPONENTS       (m/s)**2

CVWT  7--A   CO-VARIANCE OF WIND SPEED AND AIR          degC.m/s

TEMPERATURE

DATE  7--N   DATE WITHIN YEAR IN FORMAT MMDD            ----

DATM  7--N   BATHYMETRIC DATUM                          code

DAYS  7--N   DAY NUMBER WITHIN YEAR (Jan 1st = 1)       days

DENS  7--D   SEA DENSITY                                kg/m**3

DEPH  7--N   SENSOR DEPTH BELOW SEA SURFACE             metres

DEWD  7XXA   DEW POINT DEPRESSION                       degrees C

DEWT  7XXA   DEW POINT TEMPERATURE                      degrees C

DIRM  7XXN   BEARING OF OBJECT FROM REFERENCE POINT     degrees

(relative to magnetic North)

DIRT  7XXN   BEARING OF OBJECT FROM REFERENCE POINT     degrees

(relative to True North)

DISE  7XXN   DISTANCE OF OBJECT IN DIRECTION TRUE       metres

EAST FROM REFERENCE POINT

DISN  7XXN   DISTANCE OF OBJECT IN DIRECTION TRUE       metres

NORTH FROM REFERENCE POINT

DOXY  7--D   DISSOLVED OXYGEN                           mmol/m**3

DPSF  7XXN   DEPTH BELOW SEA FLOOR                      metres

DRDP  7XXD   DEPTH OF DROGUE                            metres

DRHR  7--N   DURATION (HOURS)                           hours

DRMN  7--N   DURATION (MINUTES)                         minutes

DRSC  7--N   DURATION (SECONDS)                         seconds

DRYT  7XXA   DRY BULB TEMPERATURE                       degrees C

DTDZ  7--A   VERTICAL AIR TEMPERATURE GRADIENT          degC/m

EAZM  7XXN   AZIMUTH OF MAJOR AXIS OF NAVIGATION        degrees

ERROR ELLIPSE

EEEE  7XXN   DECIMAL EXPONENT                           ----

ELEV  7XXN   ELEVATION ANGLE OF OBJECT FROM             degrees

REFERENCE POINT

EMAJ  7XXN   LENGTH OF SEMIMAJOR AXIS OF NAVIGATION     metres

ERROR ELLIPSE

EMIN  7XXN   LENGTH OF SEMIMINOR AXIS OF NAVIGATION     metres

ERROR ELLIPSE

ETHR  7XXN   ELAPSED TIME (HOURS)                       hours

ETMN  7XXN   ELAPSED TIME (MINUTES)                     minutes

ETSC  7XXN   ELAPSED TIME (SECONDS)                     seconds

EWCM  7--D   EAST (MAGNETIC) COMPONENT OF CURRENT       m/s

EWCT  7--D   EAST (TRUE) COMPONENT OF CURRENT           m/s

FFFF  7--N   QUALITY CONTROL FLAG                       code

FIXF  7--N   PRIME NAVIGATION AID FIX FLAG              code

FREQ  7--N   FREQUENCY                                  hertz

GBGA  7XXN   BOUGER GRAVITY ANOMALY                     milligals

GDIR  7XXA   DIRECTION FROM WHICH GUST WIND IS          degrees

BLOWING (relative to True North)

GEOT  7--N   GRAVITY EOTVOS CORRECTION                  milligals

GFAA  7--N   FREE AIR GRAVITY ANOMALY                   milligals

GGCD  7XXN   PERIOD OF CURRENT MEASUREMENT (DRIFT)      code

(WMO CODE 2265)

GGCM  7XXN   INDICATOR FOR METHOD OF CURRENT            code

MEASUREMENT (WMO CODE 2266)

GGDI  7XXN   INDICATOR FOR DIGITIZATION                 code

(WMO CODE 2262)

GGEC  7XXN   DURATION AND TIME OF EULERIAN CURRENT      code

MEASUREMENT (WMO CODE 2264)

GGIN  7--N   IGOSS BATHY/TESAC INDICATORS               code

GGLC  7XXN   QUALITY OF LOCATION (WMO CODE 3311)        code

GGMS  7--N   IGOSS MESSAGE IDENTIFIER                   code

GGQF  7--N   QUALITY CONTROL FLAGS FOR DATE, TIME,      code

POSITION AND SEA FLOOR DEPTH

GGSL  7XXN   METHOD OF SALINITY/DEPTH MEASUREMENT       code

(WMO CODE 2263)

GGST  7XXN   QUALITY OF BUOY SATELLITE TRANSMISSION     code

(WMO CODE 3313)

GGWI  7XXN   WIND SPEED INDICATOR (WMO CODE 1853)       code

GRAV  7XXN   OBSERVED GRAVITY                           milligals

GSPD  7XXA   GUST WIND SPEED                            m/s

HCDM  7--D   DIRECTION TO WHICH HORIZONTAL CURRENT      degrees

IS FLOWING (relative to Magnetic North)

HCDT  7--D   DIRECTION TO WHICH HORIZONTAL CURRENT      degrees

IS FLOWING (relative to True North)

HCSP  7--D   HORIZONTAL CURRENT SPEED                   m/s

HEAD  7XXN   PLATFORM HEADING                           degrees

(relative to True North)

HGHT  7XXN   HEIGHT/ALTITUDE ABOVE SEA SURFACE          metres

HHMM  7--N   TIME WITHIN DAY IN FORMAT HHMM             ----

HIGF  7XXN   HIGH FREQUENCY CUT OFF FOR INTEGRATION     hertz

UNDER SPECTRUM

HMXR  7XXA   HUMIDITY MIXING RATIO                      g/kg

HOUR  7--N   HOURS WITHIN DAY                           hours

HSUL  7--D   HYDROGEN SULPHIDE (H2S-S) CONTENT          mmol/m**3

HTSF  7XXN   HEIGHT ABOVE SEA FLOOR                     metres

ICEF  7--N   FLAG FOR ICE IN VICINITY OF HYDROGRAPHIC   code

OBSERVATIONS

IDEN  7XXN   DATA IDENTIFIER                            ----

LATD  7--N   LATITUDE DEGREES (North +ve)               degrees

LATM  7--N   LATITUDE MINUTES WITHIN DEGREE             mins arc

(North +ve)

LINC  7XXA   LONG-WAVE INCOMING RADIATION               W/m**2

LOND  7--N   LONGITUDE DEGREES (East +ve)               degrees

LONM  7--N   LONGITUDE MINUTES WITHIN DEGREE            mins arc

(East +ve)

LOUT  7XXA   LONG-WAVE OUTGOING RADIATION               W/m**2

LOWF  7XXN   LOW FREQUENCY CUT OFF FOR INTEGRATION      hertz

UNDER SPECTRUM

LVLS  7--N   SELECTION OF DEPTH LEVELS                  code

LWCT  7XXA   LIQUID WATER CONTENT                       g/m**3

MAGC  7XXN   MAGNETIC FIELD CORRECTION                  nanoteslas

MAGN  7XXN   MAGNETIC VARIATION FROM TRUE NORTH         degrees

MAGR  7--N   RESIDUAL MAGNETIC FIELD                    nanoteslas

MAGT  7XXN   TOTAL MAGNETIC FIELD                       nanoteslas

MINS  7--N   MINUTES WITHIN HOUR                        minutes

MMFX  7XXN   METHOD CODE FOR POSITION FIXING            code

MMMM  7--N   METHOD CODE IN USER DEFINED AREA           code

MNTH  7--N   CALENDAR MONTH (MM) WITHIN YEAR            ----

NETR  7XXA   NET RADIATION                              W/m**2

NIRR  7XXA   NEAR-INFRARED RADIATION                    W/m**2

NORG  7--D   ORGANIC NITROGEN CONTENT                   mmol/m**3

NSCM  7--D   NORTH (MAGNETIC) COMPONENT OF CURRENT      m/s

NSCT  7--D   NORTH (TRUE) COMPONENT OF CURRENT          m/s

NTHR  7--N   INTERVAL (HOURS)                           hours

NTMN  7--N   INTERVAL (MINUTES)                         minutes

NTOT  7--D   TOTAL NITROGEN (N) CONTENT                 mmol/m**3

NTRA  7--D   NITRATE (NO3-N) CONTENT                    mmol/m**3

NTRI  7--D   NITRITE (NO2-N) CONTENT                    mmol/m**3

NTRZ  7--D   NITRATE + NITRITE CONTENT                  mmol/m**3

NTSC  7--N   INTERVAL (SECONDS)                         seconds

PAIR  7XXN   COUNT OF PARAMETER PAIRS IN DATA CYCLE     ----

PHOS  7--D   PHOSPHATE (PO4-P) CONTENT                  mmol/m**3

PHPH  7XXD   HYDROGEN ION CONCENTRATION (pH)            ----

PLAT  7--N   PLATFORM IDENTIFIER                        code

POTM  7--D   POTENTIAL TEMPERATURE                      degrees C

POTT  7XXA   POTENTIAL AIR TEMPERATURE                  degrees C

PRES  7--D   SEA PRESSURE (sea surface = 0)             decibars

PRHB  7--N   PROBE HIT BOTTOM INDICATOR                 code

PRRT  7XXA   PRECIPITATION RATE                         mm/hr

PRTN  7XXA   PRECIPITATION AMOUNT                       mm

PSAL  7--D   PRACTICAL SALINITY                         ----

PTCH  7XXN   PITCH ANGLE                                degrees

PVAR  7XXN   VARIANCE OF PRECEDING PARAMETER            ----

QPOS  7--N   QUALITY CONTROL FLAG FOR GEOGRAPHIC        code

POSITION

QSOL  7XXG   GROUND HEAT FLUX                           W/m**2

QTIM  7--N   QUALITY CONTROL FLAG FOR SERIES            code

DATE/TIME

RADD  7XXN   HORIZONTAL DISTANCE OF OBJECT FROM         metres

REFERENCE POINT

RANG  7XXN   DIRECT DISTANCE OF OBJECT FROM             metres

REFERENCE POINT

RELH  7XXA   RELATIVE HUMIDITY                          percent

RELP  7--D   RELATIVE TOTAL PRESSURE                    decibars

ROLL  7XXN   ROLL ANGLE                                 degrees

SALD  7--N   SALINITY UNITS FLAG                        code

SCDT  7--D   DIRECTION TO WHICH SEA SURFACE CURRENT     degrees

IS FLOWING (relative to True North)

SCSP  7--D   SEA SURFACE CURRENT SPEED                  m/s

SDAT  7--A   STANDARD DEVIATION OF AIR TEMPERATURE      degrees C

SDEV  7XXN   STANDARD DEVIATION OF PRECEDING            ----

PARAMETER

SDHU  7XXA   STANDARD DEVIATION OF SPECIFIC HUMIDITY    g/kg

SDIF  7XXA   SHORT-WAVE DIFFUSE RADIATION               W/m**2

SDIR  7XXA   SHORT-WAVE DIRECT RADIATION                W/m**2

SDWS  7--A   STANDARD DEVIATION OF WIND SPEED           m/s

SEAS  7XXD   SEA STATE (WMO CODE 3700)                  code

SECC  7XXD   SECCHI DISC DEPTH                          metres

SECS  7--N   SECONDS WITHIN MINUTE                      seconds

SIDE  7XXN   ANGLE OF SIDESLIP (AIRCRAFT)               degrees

SINC  7XXA   SHORT-WAVE INCOMING RADIATION              W/m**2

SLCA  7--D   SILICATE (SIO4-SI) CONTENT                 mmol/m**3

SLEV  7XXD   OBSERVED SEA LEVEL                         metres

SOLT  7XXG   GROUND (SOIL) TEMPERATURE                  degrees C

SOUT  7XXA   SHORT-WAVE OUTGOING RADIATION              W/m**2

SPCF  7XXN   FREQUENCY OF SPECTRAL COMPONENT            hertz

SPDG  7XXN   TRUE PLATFORM SPEED ACROSS THE GROUND      m/s

SPDI  7XXN   INDICATED PLATFORM SPEED (AIRCRAFT)        m/s

SPDR  7XXN   RELATIVE PLATFORM SPEED THROUGH THE        m/s

AIR/WATER

SPDV  7XXN   VERTICAL PLATFORM SPEED                    m/s

SPEH  7XXA   SPECIFIC HUMIDITY                          g/kg

SSAL  7--D   SALINITY (PRE-1978 DEFINITION)             g/kg

(PARTS/THOUSAND)

SSPS  7--D   SEA SURFACE PRACTICAL SALINITY             ----

SSSL  7--D   SEA SURFACE SALINITY (PRE-1978             g/kg

DEFINITION) (PARTS/THOUSAND)

SSTP  7--D   SEA SURFACE TEMPERATURE                    degrees C

STAG  7XXA   STAGNATION TEMPERATURE                     degrees C

SVCZ  7XXN   BATHYMETRY SOUND VELOCITY CORRECTION       ----

AREA

SVEL  7--D   SOUND VELOCITY                             m/s

SWDR  7--D   DIRECTION FROM WHICH SWELL IS COMING       degrees

(relative to True North)

SWHT  7--D   SWELL HEIGHT                               metres

SWPR  7--D   SWELL PERIOD                               seconds

TDFL  7--D   TRACE DIRECTION FLAG                       code

TDIF  7--A   AIR TEMPERATURE DIFFERENCE BETWEEN TWO     degrees C

LEVELS (UPPER-LOWER)

TEMP  7--D   SEA TEMPERATURE                            degrees C

TEXT  7XXN   PLAIN LANGUAGE TEXT                        ----

TGRD  7--D   SEA TEMPERATURE GRADIENT                   degC/m

TIME  7--N   TIME WITHIN DAY IN FORMAT HHMMSS           ----

TOTP  7--D   TOTAL PRESSURE (atmosphere+sea pressure)   decibars

TPHS  7--D   TOTAL PHOSPHOROUS (P) CONTENT              mmol/m**3

TWCT  7XXA   TOTAL WATER CONTENT                        g/m**3

TWTT  7XXN   BATHYMETRIC TWO WAY TRAVEL TIME            seconds

ULTR  7XXA   ULTRA-VIOLET RADIATION                     W/m**2

USAL  7--D   UNDEFINED SALINITY (Prac. Salin or         ----

parts/thousand) - see also SALD7AAN

VAPP  7XXA   ACTUAL WATER VAPOUR PRESSURE               hPa

VAVH  7--D   AVERAGE HEIGHT HIGHEST ONE THIRD WAVES     metres

VBRF  7--D   WAVES SPECTRAL WIDTH (BROADNESS)           ----

VCAR  7--D   CHARACTERISTIC WAVE HEIGHT (4*RMS)         metres

VCMX  7--D   MAXIMUM CREST TO TROUGH WAVE HEIGHT        metres

VCXX  7--D   AUTO-SPECTRUM OF NORTH-SOUTH TILT          deg**2/Hz

VCXY  7--D   CO-SPECTRUM OF NORTH-SOUTH AND             deg**2/Hz

EAST-WEST TILTS

VCYY  7--D   AUTO-SPECTRUM OF EAST-WEST TILT            deg**2/Hz

VCZX  7--D   CO-SPECTRUM OF HEAVE AND NORTH-SOUTH       m.deg/Hz

TILT

VCZY  7--D   CO-SPECTRUM OF HEAVE AND EAST-WEST TILT    m.deg/Hz

VDEP  7XXN   SPECIFIED DIRECTION OF WAVE ENERGY         degrees

PROPAGATION

VDIR  7--D   VISUAL ESTIMATE OF DIRECTION FROM WHICH    degrees

WAVES ARE COMING(relative to True North)

VDSD  7--D   DIRECTIONAL WAVE SPECTRUM DENSITY          m**2/Hz

VERT  7XXN   VERTICAL DISTANCE OF OBJECT ABOVE          metres

REFERENCE POINT

VEST  7--D   VISUAL AVERAGE WAVE HEIGHT                 metres

VIRT  7XXA   VIRTUAL AIR TEMPERATURE                    degrees C

VISB  7XXA   HORIZONTAL VISIBILITY                      metres

VMED  7--D   WAVE SPECTRUM MEAN ENERGY DIRECTION        degrees

VMNL  7--D   MINIMUM WAVE LEVEL                         metres

VMTA  7--D   ZEROETH MOMENT OF WAVE SPECTRUM            m**2

VMTB  7--D   FIRST MOMENT OF WAVE SPECTRUM              m**2.Hz

VMTC  7--D   SECOND MOMENT OF WAVE SPECTRUM             (m.Hz)**2

VMTD  7--D   THIRD MOMENT OF WAVE SPECTRUM              m**2.Hz**3

VMTE  7--D   FOURTH MOMENT OF WAVE SPECTRUM             m**2.Hz**4

VMTM  7--D   FIRST NEGATIVE MOMENT OF WAVE SPECTRUM     m**2/Hz

VMTN  7--D   SECOND NEGATIVE MOMENT OF WAVE SPECTRUM    (m/Hz)**2

VMWD  7--D   MEAN WAVE DIRECTION FROM CROSS SPECTRA     degrees

VMXL  7--D   MAXIMUM WAVE LEVEL                         metres

VNUM  7--D   WAVE NUMBER FROM CROSS SPECTRA             degrees/m

VPED  7--D   WAVE SPECTRUM PEAK ENERGY DIRECTION        degrees

VPER  7--D   VISUAL ESTIMATE OF WAVE PERIOD             seconds

VQXY  7--D   QUAD-SPECTRUM OF NORTH-SOUTH AND           deg**2/Hz

EAST-WEST TILTS

VQZX  7--D   QUAD-SPECTRUM OF HEAVE AND NORTH-SOUTH     m.deg/Hz

TILT

VQZY  7--D   QUAD-SPECTRUM OF HEAVE AND EAST-WEST       m.deg/Hz

TILT

VRAT  7--A   VARIANCE OF AIR TEMPERATURE                degC**2

VRHU  7XXA   VARIANCE OF SPECIFIC HUMIDITY              (g/kg)**2

VRMS  7--D   RMS WAVE DISPLACEMENT                      metres

VRWS  7--A   VARIANCE OF WIND SPEED                     (m/s)**2

VSDN  7--D   WAVE VARIANCE SPECTRAL DENSITY : S(f)      m**2/Hz

VSMA  7--D   SPECTRAL MOMENTS (-1,0) WAVE PERIOD        seconds

VSMB  7--D   SPECTRAL MOMENTS (0,1) WAVE PERIOD         seconds

VSMC  7--D   SPECTRAL MOMENTS (0,2) WAVE PERIOD         seconds

VSMD  7--D   SPECTRAL MOMENTS (2,4) WAVE PERIOD         seconds

VSPR  7--D   WAVE DIRECTIONAL SPREAD FROM CROSS         degrees

SPECTRA

VSWD  7--D   WAVES SPECTRAL WIDTH FROM MOMENTS          ----

VTCA  7--D   AVERAGE WAVE CREST PERIOD                  seconds

VTDH  7--D   TUCKER DRAPER SIGNIFICANT WAVE HEIGHT      metres

VTKC  7--D   SECOND HIGHEST WAVE CREST                  metres

VTKD  7--D   SECOND LOWEST WAVE TROUGH                  metres

VTPK  7--D   WAVE SPECTRUM PEAK PERIOD                  seconds

VTZA  7--D   AVERAGE ZERO CROSSING WAVE PERIOD          seconds

VTZM  7--D   PERIOD OF MAXIMUM ZERO CROSSING WAVE       seconds

VWSA  7--D   INSTANTANEOUS HEAVE ACCELERATION           m/s**2

VWSE  7--D   INSTANTANEOUS WATER SURFACE ELEVATION      metres

VWSH  7XXA   VERTICAL WIND SHEAR                        (m/s)/km

VWTE  7--D   WATER SURFACE EAST-WEST TILT ANGLE         degrees

VWTN  7--D   WATER SURFACE NORTH-SOUTH TILT ANGLE       degrees

VZMX  7--D   MAXIMUM ZERO CROSSING WAVE HEIGHT          metres

WCLR  7XXD   WATER COLOUR (FOREL-ULE SCALE)             code

WDIR  7XXA   DIRECTION FROM WHICH WIND IS BLOWING       degrees

(relative to True North)

WETT  7XXA   WET BULB TEMPERATURE                       degrees C

WFBS  7XXA   WIND FORCE ON BEAUFORT SCALE               code

WMDP  7--D   MEAN WATER DEPTH                           metres

WRDP  7--D   MEAN RECORD WATER DEPTH                    metres

WSDP  7--D   STILL WATER DEPTH                          metres

WSPD  7XXA   HORIZONTAL WIND SPEED                      m/s

WSPE  7XXA   EASTWARD (TRUE) COMPONENT OF WIND SPEED    m/s

WSPN  7XXA   NORTHWARD (TRUE) COMPONENT OF WIND SPEED   m/s

WTHA  7XXA   PAST WEATHER (WMO CODE 4561)               code

WVER  7XXA   VERTICAL WIND SPEED                        m/s

WWCD  7XXA   PRESENT WEATHER (WMO CODE 4677)            code

XTRK  7XXN   ACROSS TRACK DISPLACEMENT                  metres

(starboard +ve)

YEAR  7--N   CALENDAR YEAR                              years

ZNTH  7XXN   ZENITH ANGLE OF OBJECT FROM REFERENCE      degrees

POINT

ZONE  7XXN   TIME ZONE CORRECTION                       hours

2.0 GF3 Parameter Code Table sorted alphabetically within GF3 tables

General purpose codes

---------------------

PPPP  KMMS       Parameter Name                          Units

----  ----   ---------------------------------------   --------

CCCC  7--N   DATA CYCLE OVERFLOW INDICATOR              code

CFLG  7--N   DATA CONTINUATION FLAG                     code

CHAN  7XXN   NUMBER OF SENSOR CHANNELS                  ----

CHCH  7XXA   TYPE OF HIGH ALTITUDE CLOUD                code

(WMO CODE 0509)

EEEE  7XXN   DECIMAL EXPONENT                           ----

FFFF  7--N   QUALITY CONTROL FLAG                       code

GGQF  7--N   QUALITY CONTROL FLAGS FOR DATE, TIME,      code

POSITION AND SEA FLOOR DEPTH

IDEN  7XXN   DATA IDENTIFIER                            ----

MMMM  7--N   METHOD CODE IN USER DEFINED AREA           code

PAIR  7XXN   COUNT OF PARAMETER PAIRS IN DATA CYCLE     ----

PLAT  7--N   PLATFORM IDENTIFIER                        code

PVAR  7XXN   VARIANCE OF PRECEDING PARAMETER            ----

QPOS  7--N   QUALITY CONTROL FLAG FOR GEOGRAPHIC        code

POSITION

QTIM  7--N   QUALITY CONTROL FLAG FOR SERIES            code

DATE/TIME

SDEV  7XXN   STANDARD DEVIATION OF PRECEDING            ----

PARAMETER

TEXT  7XXN   PLAIN LANGUAGE TEXT                        ----

Date and time within day codes

------------------------------

PPPP  KMMS       Parameter Name                          Units

----  ----   ---------------------------------------   --------

DATE  7--N   DATE WITHIN YEAR IN FORMAT MMDD            ----

DAYS  7--N   DAY NUMBER WITHIN YEAR (Jan 1st = 1)       days

HHMM  7--N   TIME WITHIN DAY IN FORMAT HHMM             ----

HOUR  7--N   HOURS WITHIN DAY                           hours

MINS  7--N   MINUTES WITHIN HOUR                        minutes

MNTH  7--N   CALENDAR MONTH (MM) WITHIN YEAR            ----

SECS  7--N   SECONDS WITHIN MINUTE                      seconds

TIME  7--N   TIME WITHIN DAY IN FORMAT HHMMSS           ----

YEAR  7--N   CALENDAR YEAR                              years

ZONE  7XXN   TIME ZONE CORRECTION                       hours

Time and frequency codes

------------------------

PPPP  KMMS       Parameter Name                          Units

----  ----   ---------------------------------------   --------

BAND  7XXN   BANDWIDTH OF SPECTRAL ANALYSIS             hertz

BEST  7XXN   BANDWIDTH OF SPECTRAL COMPONENT            hertz

DRHR  7--N   DURATION (HOURS)                           hours

DRMN  7--N   DURATION (MINUTES)                         minutes

DRSC  7--N   DURATION (SECONDS)                         seconds

ETHR  7XXN   ELAPSED TIME (HOURS)                       hours

ETMN  7XXN   ELAPSED TIME (MINUTES)                     minutes

ETSC  7XXN   ELAPSED TIME (SECONDS)                     seconds

FREQ  7--N   FREQUENCY                                  hertz

HIGF  7XXN   HIGH FREQUENCY CUT OFF FOR INTEGRATION     hertz

UNDER SPECTRUM

LOWF  7XXN   LOW FREQUENCY CUT OFF FOR INTEGRATION      hertz

UNDER SPECTRUM

NTHR  7--N   INTERVAL (HOURS)                           hours

NTMN  7--N   INTERVAL (MINUTES)                         minutes

NTSC  7--N   INTERVAL (SECONDS)                         seconds

SPCF  7XXN   FREQUENCY OF SPECTRAL COMPONENT            hertz

Position and navigation codes

-----------------------------

PPPP  KMMS       Parameter Name                          Units

----  ----   ---------------------------------------   --------

ALTG  7XXN   HEIGHT/ALTITUDE ABOVE GROUND LEVEL         metres

ALTS  7XXN   HEIGHT/ALTITUDE ABOVE MEAN SEA LEVEL       metres

ATCK  7XXN   ANGLE OF ATTACK (AIRCRAFT)                 degrees

ATRK  7XXN   ALONG TRACK DISPLACEMENT                   metres

DEPH  7--N   SENSOR DEPTH BELOW SEA SURFACE             metres

DIRM  7XXN   BEARING OF OBJECT FROM REFERENCE POINT     degrees

(relative to magnetic North)

DIRT  7XXN   BEARING OF OBJECT FROM REFERENCE POINT     degrees

(relative to True North)

DISE  7XXN   DISTANCE OF OBJECT IN DIRECTION TRUE       metres

EAST FROM REFERENCE POINT

DISN  7XXN   DISTANCE OF OBJECT IN DIRECTION TRUE       metres

NORTH FROM REFERENCE POINT

DPSF  7XXN   DEPTH BELOW SEA FLOOR                      metres

EAZM  7XXN   AZIMUTH OF MAJOR AXIS OF NAVIGATION        degrees

ERROR ELLIPSE

ELEV  7XXN   ELEVATION ANGLE OF OBJECT FROM             degrees

REFERENCE POINT

EMAJ  7XXN   LENGTH OF SEMIMAJOR AXIS OF NAVIGATION     metres

ERROR ELLIPSE

EMIN  7XXN   LENGTH OF SEMIMINOR AXIS OF NAVIGATION     metres

ERROR ELLIPSE

FIXF  7--N   PRIME NAVIGATION AID FIX FLAG              code

HEAD  7XXN   PLATFORM HEADING                           degrees

(relative to True North)

HGHT  7XXN   HEIGHT/ALTITUDE ABOVE SEA SURFACE          metres

HTSF  7XXN   HEIGHT ABOVE SEA FLOOR                     metres

LATD  7--N   LATITUDE DEGREES (North +ve)               degrees

LATM  7--N   LATITUDE MINUTES WITHIN DEGREE             mins arc

(North +ve)

LOND  7--N   LONGITUDE DEGREES (East +ve)               degrees

LONM  7--N   LONGITUDE MINUTES WITHIN DEGREE            mins arc

(East +ve)

MAGN  7XXN   MAGNETIC VARIATION FROM TRUE NORTH         degrees

MMFX  7XXN   METHOD CODE FOR POSITION FIXING            code

PTCH  7XXN   PITCH ANGLE                                degrees

RADD  7XXN   HORIZONTAL DISTANCE OF OBJECT FROM         metres

REFERENCE POINT

RANG  7XXN   DIRECT DISTANCE OF OBJECT FROM             metres

REFERENCE POINT

ROLL  7XXN   ROLL ANGLE                                 degrees

SIDE  7XXN   ANGLE OF SIDESLIP (AIRCRAFT)               degrees

SPDG  7XXN   TRUE PLATFORM SPEED ACROSS THE GROUND      m/s

SPDI  7XXN   INDICATED PLATFORM SPEED (AIRCRAFT)        m/s

SPDR  7XXN   RELATIVE PLATFORM SPEED THROUGH THE        m/s

AIR/WATER

SPDV  7XXN   VERTICAL PLATFORM SPEED                    m/s

VERT  7XXN   VERTICAL DISTANCE OF OBJECT ABOVE          metres

REFERENCE POINT

XTRK  7XXN   ACROSS TRACK DISPLACEMENT                  metres

(starboard +ve)

ZNTH  7XXN   ZENITH ANGLE OF OBJECT FROM REFERENCE      degrees

POINT

Physical oceanography codes

---------------------------

PPPP  KMMS       Parameter Name                          Units

----  ----   ---------------------------------------   --------

CAST  7--N   HYDROGRAPHIC CAST TYPE                     code

CHLR  7--D   CHLORINITY (PARTS/THOUSAND)                g/kg

CHLS  7--D   CHLOROSITY                                 kg/m**3

CNDC  7XXD   ELECTRICAL CONDUCTIVITY                    mhos/m

DENS  7--D   SEA DENSITY                                kg/m**3

DOXY  7--D   DISSOLVED OXYGEN                           mmol/m**3

DRDP  7XXD   DEPTH OF DROGUE                            metres

EWCM  7--D   EAST (MAGNETIC) COMPONENT OF CURRENT       m/s

EWCT  7--D   EAST (TRUE) COMPONENT OF CURRENT           m/s

HCDM  7--D   DIRECTION TO WHICH HORIZONTAL CURRENT      degrees

IS FLOWING (relative to Magnetic North)

HCDT  7--D   DIRECTION TO WHICH HORIZONTAL CURRENT      degrees

IS FLOWING (relative to True North)

HCSP  7--D   HORIZONTAL CURRENT SPEED                   m/s

ICEF  7--N   FLAG FOR ICE IN VICINITY OF HYDROGRAPHIC   code

OBSERVATIONS

LVLS  7--N   SELECTION OF DEPTH LEVELS                  code

NSCM  7--D   NORTH (MAGNETIC) COMPONENT OF CURRENT      m/s

NSCT  7--D   NORTH (TRUE) COMPONENT OF CURRENT          m/s

POTM  7--D   POTENTIAL TEMPERATURE                      degrees C

PRES  7--D   SEA PRESSURE (sea surface = 0)             decibars

PRHB  7--N   PROBE HIT BOTTOM INDICATOR                 code

PSAL  7--D   PRACTICAL SALINITY                         ----

RELP  7--D   RELATIVE TOTAL PRESSURE                    decibars

SALD  7--N   SALINITY UNITS FLAG                        code

SCDT  7--D   DIRECTION TO WHICH SEA SURFACE CURRENT     degrees

IS FLOWING (relative to True North)

SCSP  7--D   SEA SURFACE CURRENT SPEED                  m/s

SECC  7XXD   SECCHI DISC DEPTH                          metres

SLEV  7XXD   OBSERVED SEA LEVEL                         metres

SSAL  7--D   SALINITY (PRE-1978 DEFINITION)             g/kg

(PARTS/THOUSAND)

SSPS  7--D   SEA SURFACE PRACTICAL SALINITY             ----

SSSL  7--D   SEA SURFACE SALINITY (PRE-1978             g/kg

DEFINITION) (PARTS/THOUSAND)

SSTP  7--D   SEA SURFACE TEMPERATURE                    degrees C

SVEL  7--D   SOUND VELOCITY                             m/s

TDFL  7--D   TRACE DIRECTION FLAG                       code

TEMP  7--D   SEA TEMPERATURE                            degrees C

TGRD  7--D   SEA TEMPERATURE GRADIENT                   degC/m

TOTP  7--D   TOTAL PRESSURE (atmosphere+sea pressure)   decibars

USAL  7--D   UNDEFINED SALINITY (Prac. Salin or         ----

parts/thousand) - see also SALD7AAN

WCLR  7XXD   WATER COLOUR (FOREL-ULE SCALE)             code

Wave codes

----------

PPPP  KMMS       Parameter Name                          Units

----  ----   ---------------------------------------   --------

BDIR  7XXN   BANDWIDTH OF DIRECTIONAL ANALYSIS          degrees

SEAS  7XXD   SEA STATE (WMO CODE 3700)                  code

SWDR  7--D   DIRECTION FROM WHICH SWELL IS COMING       degrees

(relative to True North)

SWHT  7--D   SWELL HEIGHT                               metres

SWPR  7--D   SWELL PERIOD                               seconds

VAVH  7--D   AVERAGE HEIGHT HIGHEST ONE THIRD WAVES     metres

VBRF  7--D   WAVES SPECTRAL WIDTH (BROADNESS)           ----

VCAR  7--D   CHARACTERISTIC WAVE HEIGHT (4*RMS)         metres

VCMX  7--D   MAXIMUM CREST TO TROUGH WAVE HEIGHT        metres

VCXX  7--D   AUTO-SPECTRUM OF NORTH-SOUTH TILT          deg**2/Hz

VCXY  7--D   CO-SPECTRUM OF NORTH-SOUTH AND             deg**2/Hz

EAST-WEST TILTS

VCYY  7--D   AUTO-SPECTRUM OF EAST-WEST TILT            deg**2/Hz

VCZX  7--D   CO-SPECTRUM OF HEAVE AND NORTH-SOUTH       m.deg/Hz

TILT

VCZY  7--D   CO-SPECTRUM OF HEAVE AND EAST-WEST TILT    m.deg/Hz

VDEP  7XXN   SPECIFIED DIRECTION OF WAVE ENERGY         degrees

PROPAGATION

VDIR  7--D   VISUAL ESTIMATE OF DIRECTION FROM WHICH    degrees

WAVES ARE COMING(relative to True North)

VDSD  7--D   DIRECTIONAL WAVE SPECTRUM DENSITY          m**2/Hz

VEST  7--D   VISUAL AVERAGE WAVE HEIGHT                 metres

VMED  7--D   WAVE SPECTRUM MEAN ENERGY DIRECTION        degrees

VMNL  7--D   MINIMUM WAVE LEVEL                         metres

VMTA  7--D   ZEROETH MOMENT OF WAVE SPECTRUM            m**2

VMTB  7--D   FIRST MOMENT OF WAVE SPECTRUM              m**2.Hz

VMTC  7--D   SECOND MOMENT OF WAVE SPECTRUM             (m.Hz)**2

VMTD  7--D   THIRD MOMENT OF WAVE SPECTRUM              m**2.Hz**3

VMTE  7--D   FOURTH MOMENT OF WAVE SPECTRUM             m**2.Hz**4

VMTM  7--D   FIRST NEGATIVE MOMENT OF WAVE SPECTRUM     m**2/Hz

VMTN  7--D   SECOND NEGATIVE MOMENT OF WAVE SPECTRUM    (m/Hz)**2

VMWD  7--D   MEAN WAVE DIRECTION FROM CROSS SPECTRA     degrees

VMXL  7--D   MAXIMUM WAVE LEVEL                         metres

VNUM  7--D   WAVE NUMBER FROM CROSS SPECTRA             degrees/m

VPED  7--D   WAVE SPECTRUM PEAK ENERGY DIRECTION        degrees

VPER  7--D   VISUAL ESTIMATE OF WAVE PERIOD             seconds

VQXY  7--D   QUAD-SPECTRUM OF NORTH-SOUTH AND           deg**2/Hz

EAST-WEST TILTS

VQZX  7--D   QUAD-SPECTRUM OF HEAVE AND NORTH-SOUTH     m.deg/Hz

TILT

VQZY  7--D   QUAD-SPECTRUM OF HEAVE AND EAST-WEST       m.deg/Hz

TILT

VRMS  7--D   RMS WAVE DISPLACEMENT                      metres

VSDN  7--D   WAVE VARIANCE SPECTRAL DENSITY : S(f)      m**2/Hz

VSMA  7--D   SPECTRAL MOMENTS (-1,0) WAVE PERIOD        seconds

VSMB  7--D   SPECTRAL MOMENTS (0,1) WAVE PERIOD         seconds

VSMC  7--D   SPECTRAL MOMENTS (0,2) WAVE PERIOD         seconds

VSMD  7--D   SPECTRAL MOMENTS (2,4) WAVE PERIOD         seconds

VSPR  7--D   WAVE DIRECTIONAL SPREAD FROM CROSS         degrees

SPECTRA

VSWD  7--D   WAVES SPECTRAL WIDTH FROM MOMENTS          ----

VTCA  7--D   AVERAGE WAVE CREST PERIOD                  seconds

VTDH  7--D   TUCKER DRAPER SIGNIFICANT WAVE HEIGHT      metres

VTKC  7--D   SECOND HIGHEST WAVE CREST                  metres

VTKD  7--D   SECOND LOWEST WAVE TROUGH                  metres

VTPK  7--D   WAVE SPECTRUM PEAK PERIOD                  seconds

VTZA  7--D   AVERAGE ZERO CROSSING WAVE PERIOD          seconds

VTZM  7--D   PERIOD OF MAXIMUM ZERO CROSSING WAVE       seconds

VWSA  7--D   INSTANTANEOUS HEAVE ACCELERATION           m/s**2

VWSE  7--D   INSTANTANEOUS WATER SURFACE ELEVATION      metres

VWTE  7--D   WATER SURFACE EAST-WEST TILT ANGLE         degrees

VWTN  7--D   WATER SURFACE NORTH-SOUTH TILT ANGLE       degrees

VZMX  7--D   MAXIMUM ZERO CROSSING WAVE HEIGHT          metres

WMDP  7--D   MEAN WATER DEPTH                           metres

WRDP  7--D   MEAN RECORD WATER DEPTH                    metres

WSDP  7--D   STILL WATER DEPTH                          metres

Meteorology codes

-----------------

PPPP  KMMS       Parameter Name                          Units

----  ----   ---------------------------------------    ---------

ABSH  7XXA   ABSOLUTE HUMIDITY                          g/m**3

ASTD  7XXB   AIR-SEA TEMPERATURE DIFFERENCE             degrees C

ATMP  7XXA   ATMOSPHERIC PRESSURE AT ALTITUDE           hPa

ATMS  7XXA   ATMOSPHERIC PRESSURE AT SEA LEVEL          hPa

ATPT  7XXA   ATMOSPHERIC PRESSURE TENDENCY              hPa/hr

BRIT  7XXA   BRIGHTNESS (RADIATIVE) TEMPERATURE         degrees C

CCVR  7XXA   TOTAL CLOUD COVER IN TENTHS OF SKY         ----

CLCL  7XXA   TYPE OF LOW ALTITUDE CLOUD                 code

(WMO CODE 0513)

CLCM  7XXA   AMOUNT OF LOW/MEDIUM ALTITUDE CLOUD        code

(OKTAS - WMO CODE 2700)

CLDA  7XXA   TOTAL CLOUD AMOUNT (OKTAS-WMO CODE 2700)   code

CLDB  7XXA   CLOUD BASE ALTITUDE                        metres

CLDH  7XXA   CLOUD BASE HEIGHT (WMO CODE 1600)          code

CLDT  7XXA   CLOUD TYPE (WMO CODE 0500)                 code

CMCM  7XXA   TYPE OF MEDIUM ALTITUDE CLOUD              code

(WMO CODE 0515)

CVTQ  7--A   CO-VARIANCE OF AIR TEMPERATURE AND         degC.g/kg

SPECIFIC HUMIDITY

CVWQ  7--A   CO-VARIANCE OF WIND SPEED AND SPECIFIC     (m/s).g/kg

HUMIDITY

CVWS  7--A   CO-VARIANCE OF WIND SPEED COMPONENTS       (m/s)**2

CVWT  7--A   CO-VARIANCE OF WIND SPEED AND AIR          degC.m/s

TEMPERATURE

DEWD  7XXA   DEW POINT DEPRESSION                       degrees C

DEWT  7XXA   DEW POINT TEMPERATURE                      degrees C

DRYT  7XXA   DRY BULB TEMPERATURE                       degrees C

DTDZ  7--A   VERTICAL AIR TEMPERATURE GRADIENT          degC/m

GDIR  7XXA   DIRECTION FROM WHICH GUST WIND IS          degrees

BLOWING (relative to True North)

GSPD  7XXA   GUST WIND SPEED                            m/s

HMXR  7XXA   HUMIDITY MIXING RATIO                      g/kg

LINC  7XXA   LONG-WAVE INCOMING RADIATION               W/m**2

LOUT  7XXA   LONG-WAVE OUTGOING RADIATION               W/m**2

LWCT  7XXA   LIQUID WATER CONTENT                       g/m**3

NETR  7XXA   NET RADIATION                              W/m**2

NIRR  7XXA   NEAR-INFRARED RADIATION                    W/m**2

POTT  7XXA   POTENTIAL AIR TEMPERATURE                  degrees C

PRRT  7XXA   PRECIPITATION RATE                         mm/hr

PRTN  7XXA   PRECIPITATION AMOUNT                       mm

QSOL  7XXG   GROUND HEAT FLUX                           W/m**2

RELH  7XXA   RELATIVE HUMIDITY                          percent

SDAT  7--A   STANDARD DEVIATION OF AIR TEMPERATURE      degrees C

SDHU  7XXA   STANDARD DEVIATION OF SPECIFIC HUMIDITY    g/kg

SDIF  7XXA   SHORT-WAVE DIFFUSE RADIATION               W/m**2

SDIR  7XXA   SHORT-WAVE DIRECT RADIATION                W/m**2

SDWS  7--A   STANDARD DEVIATION OF WIND SPEED           m/s

SINC  7XXA   SHORT-WAVE INCOMING RADIATION              W/m**2

SOLT  7XXG   GROUND (SOIL) TEMPERATURE                  degrees C

SOUT  7XXA   SHORT-WAVE OUTGOING RADIATION              W/m**2

SPEH  7XXA   SPECIFIC HUMIDITY                          g/kg

STAG  7XXA   STAGNATION TEMPERATURE                     degrees C

TDIF  7--A   AIR TEMPERATURE DIFFERENCE BETWEEN TWO     degrees C

LEVELS (UPPER-LOWER)

TWCT  7XXA   TOTAL WATER CONTENT                        g/m**3

ULTR  7XXA   ULTRA-VIOLET RADIATION                     W/m**2

VAPP  7XXA   ACTUAL WATER VAPOUR PRESSURE               hPa

VIRT  7XXA   VIRTUAL AIR TEMPERATURE                    degrees C

VISB  7XXA   HORIZONTAL VISIBILITY                      metres

VRAT  7--A   VARIANCE OF AIR TEMPERATURE                degC**2

VRHU  7XXA   VARIANCE OF SPECIFIC HUMIDITY              (g/kg)**2

VRWS  7--A   VARIANCE OF WIND SPEED                     (m/s)**2

VWSH  7XXA   VERTICAL WIND SHEAR                        (m/s)/km

WDIR  7XXA   DIRECTION FROM WHICH WIND IS BLOWING       degrees

(relative to True North)

WETT  7XXA   WET BULB TEMPERATURE                       degrees C

WFBS  7XXA   WIND FORCE ON BEAUFORT SCALE               code

WSPD  7XXA   HORIZONTAL WIND SPEED                      m/s

WSPE  7XXA   EASTWARD (TRUE) COMPONENT OF WIND SPEED    m/s

WSPN  7XXA   NORTHWARD (TRUE) COMPONENT OF WIND SPEED   m/s

WTHA  7XXA   PAST WEATHER (WMO CODE 4561)               code

WVER  7XXA   VERTICAL WIND SPEED                        m/s

WWCD  7XXA   PRESENT WEATHER (WMO CODE 4677)            code

Geophysics codes

----------------

PPPP  KMMS       Parameter Name                          Units

----  ----   ---------------------------------------   --------

BATH  7--N   BATHYMETRIC DEPTH                          metres

DATM  7--N   BATHYMETRIC DATUM                          code

GBGA  7XXN   BOUGER GRAVITY ANOMALY                     milligals

GEOT  7--N   GRAVITY EOTVOS CORRECTION                  milligals

GFAA  7--N   FREE AIR GRAVITY ANOMALY                   milligals

GRAV  7XXN   OBSERVED GRAVITY                           milligals

MAGC  7XXN   MAGNETIC FIELD CORRECTION                  nanoteslas

MAGR  7--N   RESIDUAL MAGNETIC FIELD                    nanoteslas

MAGT  7XXN   TOTAL MAGNETIC FIELD                       nanoteslas

SVCZ  7XXN   BATHYMETRY SOUND VELOCITY CORRECTION       ----

AREA

TWTT  7XXN   BATHYMETRIC TWO WAY TRAVEL TIME            seconds

Chemistry codes

---------------

PPPP  KMMS       Parameter Name                          Units

----  ----   ---------------------------------------   --------

ALKY  7XXD   TOTAL ALKALINITY                           mmol/m**3

AMON  7--D   AMMONIUM (NH4-N) CONTENT                   mmol/m**3

CALK  7XXD   CARBONATE ALKALINITY                       mmol/m**3

CNQF  7--N   CHEMICAL CONTENT QUALIFIER FLAG            code

CORG  7--D   ORGANIC CARBON CONTENT                     mmol/m**3

CPHL  7XXD   CHLOROPHYLL-A CONTENT                      mg/m**3

CTOT  7--D   TOTAL CARBON (C) CONTENT                   mmol/m**3

HSUL  7--D   HYDROGEN SULPHIDE (H2S-S) CONTENT          mmol/m**3

NORG  7--D   ORGANIC NITROGEN CONTENT                   mmol/m**3

NTOT  7--D   TOTAL NITROGEN (N) CONTENT                 mmol/m**3

NTRA  7--D   NITRATE (NO3-N) CONTENT                    mmol/m**3

NTRI  7--D   NITRITE (NO2-N) CONTENT                    mmol/m**3

NTRZ  7--D   NITRATE + NITRITE CONTENT                  mmol/m**3

PHOS  7--D   PHOSPHATE (PO4-P) CONTENT                  mmol/m**3

PHPH  7XXD   HYDROGEN ION CONCENTRATION (pH)            ----

SLCA  7--D   SILICATE (SIO4-SI) CONTENT                 mmol/m**3

TPHS  7--D   TOTAL PHOSPHOROUS (P) CONTENT              mmol/m**3

Special IGOSS Indicator codes

-----------------------------

PPPP  KMMS       Parameter Name                          Units

----  ----   ---------------------------------------   --------

APER  7XXN   AVERAGING PERIOD FOR MEASUREMENT           CODE

(WMO CODE TABLE 2604)

GGCD  7XXN   PERIOD OF CURRENT MEASUREMENT (DRIFT)      code

(WMO CODE 2265)

GGCM  7XXN   INDICATOR FOR METHOD OF CURRENT            code

MEASUREMENT (WMO CODE 2266)

GGDI  7XXN   INDICATOR FOR DIGITIZATION                 code

(WMO CODE 2262)

GGEC  7XXN   DURATION AND TIME OF EULERIAN CURRENT      code

MEASUREMENT (WMO CODE 2264)

GGIN  7--N   IGOSS BATHY/TESAC INDICATORS               code

GGLC  7XXN   QUALITY OF LOCATION (WMO CODE 3311)        code

GGMS  7--N   IGOSS MESSAGE IDENTIFIER                   code

GGSL  7XXN   METHOD OF SALINITY/DEPTH MEASUREMENT       code

(WMO CODE 2263)

GGST  7XXN   QUALITY OF BUOY SATELLITE TRANSMISSION     code

(WMO CODE 3313)

GGWI  7XXN   WIND SPEED INDICATOR (WMO CODE 1853)       code

3.0 MEDS non-standard codes.

Note that codes generated by NODC are 3 alphanumerics followed by a

# symbol.

SVA$ = specific volume anomaly in units of ???

SGT$ = sigma-t in units ?? (unitsof density)

SDA$ = sea density anomaly in units ???

PPPP  KMMS       Parameter Name                          Units

----  ----   ---------------------------------------    --------

ACC$         MEDS accesion number                       ----

ADID         AODC database ID                           ----

ADNB         Number of blocks averaged in the           ----

spectrum. (Note that this is the same as

NBL1)

ADNS         Number of samples per fourier block        ----

ADST         Automatic gain control                     ----

ADSV         Automatic gain control variance            ----

AED1         AES wave direction (from METOC hindcast).  degrees

This is probably the wind direction.

APC1         Averaging period (in TRACKOB: mC) for      minutes

surface current

APR$         The group in TRACKOB supplying the         ----

averging periods used for temperature,

salinity, currents and the units of the

current. (See APER)

APS1         Averaging period (in TRACKOB: mS) for      minutes

salinity

APT1         Averaging period (in TRACKOB: mT) for      minutes

sea surface temperature

AREA         Area code used by NODC                     code

PA = Pacific

IN = Indian

AT = Atlantic

ARG$         ARGOS number given to a platform           ----

AST1         Internal temperature from the              deg.C

non-synoptic part of WRIPS buoy data.

AST2         Internal temperature from the synoptic     deg.C

part of WRIPS buoy data.

BCI$         The indicator k5 used in the BATHY code    code

to tell the method of current

measurement. This is equivalent to CCGM

BKT$         Bucket sample                              ----

BQA$         QA in BUOY code form                       ----

BTC1         BT temperature correction                  deg.C

BTN1         BT slide number                            ----

BTR1         BT surface reference temperature           deg.C

BTS1         BT surface temperature                     deg.C

BYM$         Previous bulletin year-month as YYMM       ----

B12$         Vitamin B12                                milli-microg/lt

CCLL         Platform call sign                         ----

CDX1         Dissolved organic C = CORG DX (mg/lt)      ?????

CLW1         The divider network setting on the FM to   ----

digital conversion system

CLW2         The ratio of playback to record speed for  ----

the digitizer

CLW3         The clock frequency                        Hz

CLW4         The percent modulation of the waverider    Percent

FM signal

CLW5         The mean center frequency of the           Hz

waverider FM signal

CLW6         The zero intercept on the frequency axis   Hz

for frequency vs height

CLW7         The slope of the frequency/height          m/Hz

calibration

CLW8         The height of the pressure cell above the  m

bottom

CPH$         Chlorophyll-a content, determined by the   mg/m**3

Welchmeyer method

CPX1         Particulate C = CORG PX (mg/lt)            ?????

CSC1         Chief scientist's name                     ----

CSID         CSIRO database ID                          ----

CSN$         IGOSS call sign                            ----

CTF$         The indicator that encodes which quality   code

control tests for chemical data that

were failed.

CTP$         The indicator that encodes which quality   code

control tests on chemical data have been

executed.

DAT#         NODC originator's date                     ----

DCLN         The magnetic declination                   degrees

DDB$         The date as YYYYMMDD of deployment         ----

DEB$         The date as YYYYMMDD of the last message   ----

received.

DEC$         The code for why messages ceased.          code

DEOW         Code to indicate if the magnetic           code

declination is east or west:

1 = East, 2 = West

DET$         The time as HHMM of the last message       ----

received.

DHM$         Day, hour, and minute as DDHHMM and a      ----

real number.

DKT$         Recorder type for IGOSS BTs. This is       ----

equivalent to RCT$.

DLA$         Latitude (north +ve) at deployment         degrees

(see DLT$)

DLO$         Longitude (+/- 180 west +ve) at            degrees

deployment

DLT$         Latitude (north +ve) at deployment         degrees

DMH$         The day of the month, as DD, of the        day

extreme high value.

DML$         The day of the month, as DD, of the        day

extreme low value.

DMN$         The daily mean value.                      m

DN3$         The date as YYYYMMDD of analysis of

Nitrate

DPO$         The date as YYYYMMDD of analysis of

Phosphate

DPT$         The deployment time (GMT) as HHMM          ----

DRD$         The date as YYYYMMDD that the drogue       ----

was lost

DRT$         The drogue type code                       code

DSO$         The date as YYYYMMDD of analysis of

Silicate

DSP1         The Juszko directional spread factor for   ----

the most energetic peak in a spectrum.

DSP2         The Juszko directional spread factor for   ----

the second most energetic peak in a

spectrum

DTC$         Calibration date as YYYYMMDD               ----

DTE$         The day, month and year as indicated in    ----

IGOSS messages. So DDMMY where DD is the

day, MM is the month and Y is the last

digit of the year.

DTH$         The day, hour and minute of the extreme    ----

high

DTL$         The day, hour and minute of the extreme    ----

low

DT2$         The date as YYYYMMDD of the end of a tow.  ----

When this is used, the date field

records the start date.

EAG1         Name of the exchange agency                ----

EIT$         Engine intake water sample                 ----

ELA$         The error in latitude                      degrees

ELO$         The error in longitude                     degrees

ERS$         Percent residual least squares error by    ----

Juszko in 10 parameter model fit to

directional spectrum.

EST$         The error in SST                           deg.C

EXP$         The originator's experiment number         ----

FLO1         Fluoride                                   ????

FLU$         Fluorescence                               %

FLU1         Fluorescence                               mg/m**3

FLU2         Fluorescence                               volts

FRE$         Code for fall rate equation used. See      code

IGOSS code table.

FRQ1         Frequency of the most energetic peak       hertz

in spectrum

FRQ2         Frequency of the second most energetic     hertz

peak in spectrum

F11$         CFC11 measurements                         pmol/kg

F12$         CFC12 measurements                         pmol/kg

GE1$         Contents of the first engineering group    code

in a DRIBU message

GE2$         Contents of the second engineering group   code

in a DRIBU message

GE3$         Contents of the third engineering group    code

in a DRIBU message

GGC$         The 5 character 66... group used in a      code

TESAC message to indicate information

about the subsurface data collection.

GGI$         The 5 character 88.. group used in a       code

BATHY and a TESAC message to indicate

information about the subsurface profile.

GGK6         Method of removing ship velocity code      code

(k6 of a TESAC message)

GG1$         The contents of the group 111QdQx in       ----

FM18-X BUOY code form

GG2$         The contents of the group 222QdQx in       ----

FM18-X BUOY code form

GG3$         The contents of the group 333QdQx in       ----

FM18-X BUOY code form

GIN$         Contents of the group 1QpQ2QtwQ4 in the    code

DRIBU message

GNC#         NODC cruise code                           code

GPA$         Geopotential anomaly                       dyn.m

GPT$         Pressure tendency code (see WMO code       code

table 0200)

HGR$         Hourly height water level derived from     m

MEDS Gradicon to cm precision

HIN$         The integrated water level for an hour.    m

HIS$         Hourly height water level derived from     m

unknown historical sources to cm

precision

HL1$         The height of the first daily low          m

HL2$         The height of the second daily low         m

HMN$         The minimum height read in an hour.        m

HMT$         Hull mounted sensor                        ----

HMX$         The maximum height read in an hour.        m

HM2$         The time as HHMM of the the end of a tow.  ----

When this is used, the time field

records the start time.

HOG$         Hourly height water level derived from     m

Ogdensberg to centimeter precision

HOH$         Hourly height water level derived from     m

Ontario Hydro to centimeter precision

HPR$         Hourly height predicted value.             m

HSC$         Hourly height water level derived from     m

MEDS Scalar to cm precision

HTH$         Hourly heights supplied from the West      m

Coast.

HTL$         Height of the XBT launcher above the       m

water

HW1$         The height of the first daily high         m

HW2$         The height of the second daily high        m

HZY$         Hourly height water level derived from     m

MEDS Zycor to cm precision

H00$         Hourly heights supplied by MEDS from       m

TATS gauges.

H15$         Height value read 15 minutes after the     m

hour.

H30$         Height value read 30 minutes after the     m

hour.

H45$         Height value read 45 minutes after the     m

hour.

H60$         Hourly height water level derived from     m

TATS stations and reduced by rounding

to a precision of cm

ICD$         Equivalent to IST$                         ----

ICT$         Ice thickness                              m

INS1         Institute                                  ----

INTP         Instrument type (used with wavec data)     code

1 = Waverider

2 = WAVEC

3 = ENDECO

4 = Satellite waverider

IST$         Instrument name                            ----

MOD CM = modified current meter to collect

T and S only.

JPT$         JGOFS Protocol Code Table                  ----

LAT$         The latitude in IGOSS format of DDMM       ----

where DD is the degrees and MM the

minutes of latitude.

LAT#         NODC originators latitude                  degrees

LBL1         The block length                           sec

LNG$         Longitude from GPS (+ West)                degrees

LN2$         The second longitude (from DRIBU           degrees

message or for ocean profile data the

longitude records the start longitude and

this records the end longitude of a tow)

LON$         The longitude in IGOSS format of DDMM      ----

where DD is the degrees and MM the

minutes of longitude.

LON#         NODC originators longitude                 degrees

LTG$         Latitude from GPS (+ North)                degrees

LT2$         The second latitude (from DRIBU            degrees

message or for ocean profile data the

latitude records the start latitude and

this records the end latitude of a tow)

LW1$         The height of the first daily low          m

LW2$         The height of the second daily low         m

MMN$         Monthly mean waterlevel                    m

MNS$         Minimum value of salinity                  PSU

MNT$         Minimum value of temperature               degC

MRT#         NODC instrument for referrence temperature ----

MBT reference type

MTH$         Links a observed value PCODE to a method   ----

or insturmentation PCODE

MXS$         Maximum value of salinity                  PSU

MXT$         Maximum value of temperature               degC

NBD1         The number of bad samples in a surface     ----

elevation time series.

NBL1         The number of blocks averaged to derive    ----

a wave spectrum.

NET$         Net mesh size                              microns

NH3$         NH3-N in micrograms/liter                  ?????

NID#         NODC internal database identifier          code

NPT#         NODC P3 format flag indicating that a      code

station has either only derived values or

at least one profile of measured values

NUL$         A null parameter. It is used when it is    ----

necessary to reserve the position of

code.

NUM$         Number of values used in computation       ----

NWAV         The number of waves in the record          ----

OCL#         NODC OCL reference number                  ----

OCR$         The originator's cruise number             ----

ODD1         The direction from which waves are         degrees

coming for the most energetic peak in

a spectrum

ODD2         The direction from which waves are         degrees

coming for the second most energetic

peak in a spectrum

OID$         Originator's station identifier            ----

OLS#         NODC OCL unique station number             ----

OLP$         Particulate oil                            mg/m2

OLW$         Total dissolved/dispersed oil              mg/m3

ONM1         Originator's name                          ----

OSI$         Originator's sample identifier             ----

OXS$         Percent Oxygen saturation                  percent

O18$         O18/O16 ratio                              percent

PAR$         Photosynthetic active radiation            uE/m**2-s

PAR1         Photosynthetic active radiation            unknown

PDT$         The date as DDMMY of the last position     ----

(from the DRIBU message)

PEA$         Potential energy anomaly                   Kg-m/sec**2

PEQ$         Probe type and fall rate equation code     ----

table (IGOSS)

PFR$         WMO codes 1770 (IxIxIx) + 4770 (XrXr)      code

as used in the JJYY BATHY code

PGI#         NODC P3 software identifier                code

PHA$         Phaeophytin                                mg/m***3

PHI$         Inorganic phosphate                        mmol/m**3

PH1$         Replicate sample of inorganic phosphate    mmol/m**3

PLA#         NODC platform/ship code                    code

PLT$         Percent light transmittance                percent

PNM1         Platform name                              ----

PON$         Particulate Organic Nitrogen               mg/**3

PRO1         Project name                               ----

PRO#         NODC project code                          code

PRP$         Primary productivity                       mgC/m**3/hr

PRQ$         Primary productivity                       mgC/m**3/day

PRV$         Velocity probe                             m/s

PRT$         Probe type code (see IGOSS code table)     ----

NODC uses PRT#

PTM$         The time as HHMM/ of the last position     ----

(from the DRIBU message)

PTP1         Platform type                              code

QCF$         The indicator that encodes which MEDS      code

quality control tests have failed for

temperature and salinity data.

QCP$         The indicator that encodes which MEDS      code

quality control tests have been

executed for temperature and salinity

data.

QFF$         The indicator that encodes which ORSTOM    code

quality control tests have failed for

temperature and salinity data.

QFP$         The indicator that encodes which ORSTOM    code

quality control tests have been

executed for temperature and salinity

data.

QNF#         NODC indicator of QC tests failed at the   code

station level

QNP#         NODC indicator of QC tests made at the     code

station level

RCRD         Indicates a record has been created.       code

RCT$         XBT recorder type (see IGOSS code table)   code

This is equivalent to DKT$.

REC$         A record in the MEDS time series format.   ----

RECD         The record number of the tape containing   ----

the raw data.

REP$         Records that one or more profiles have     code

replicate samples at one or more depths.

The profiles affected are encoded in the

same way QCF$, QCP$ are set

RTC$         Reference temperature code                 code

RTD$         Reference temperature water depth          m

RTM$         Reference temperature                      degC

SCR$         Instrument scan rate (from towed CTDs)     secs

SDA$         Sea density anomaly                        Kg/m**3

SDS$         Standard deviation of salinity             PSU

SDT$         Standard deviation of temperature          degC

SED1         WES sea direction.                         degrees T

SEH1         WES sea height.                            m

SEM$         Significant period M0/M1 of wind sea       sec

SEP1         WES sea period.                            sec

SER1         Instrument serial number                   ----

SGT$         Sigma-t                                    kg/m**3

SHA1         Ochi and Hubble spectral shape parameter   ----

for the most energetic peak in the

spectrum

SHA2         Ochi and Hubble spectral shape parameter   ----

for the second most energetic peak

in the spectrum

SHP#         Ship name                                  ----

SIDE         The side number of the tape containing     ----

the raw data. (Note that this

duplicates a standard GF3 code used for

another purpose)

SIN1         Salinity instrument (from radio log of a   ----

BATHY or TESAC message)

SLA$         The last latitude recorded                 degrees

SLO$         The last longitude recorded                degrees

SMI$         Interval between burst samples             mins

SOU$         Sounding depth (See BATH)                  m

SPC$         Surface Current Magnitude Spread, RMS      m/s

deviation between measured current

components and decomposition of the

estimated current vector in the

antenna-looking directions. For a

homogeneous current, a small value

indicates a good current estimate.

(SPRC from AGRA)

SRT$         Sea surface reference temperature          deg.C

SSD#         NODC Depth of sea surface current sensor   m

SST1         Average sea temperature from the           deg.C

non-synoptic part of WRIPS buoy data.

SST2         Average sea temperature from the           deg.C

synoptic part of WRIPS buoy data.

STC$         Spectral type code by Juszko               ----

STI#         NODC Surface temperature instrument type   code

STM#         NODC Sea surface temperature methodology   code

STP$         Sampler type                               code

SUL$         Sulphate (SO4)                             mol/m**3

SVA$         Specific volume anomaly                    m**3/Kg

SVR$         SEAS version number                        code

SWD#         Swell direction (WMO code 0877)            code

SWM$         Significant period M0/M1 of swell          sec

TAPE         The tape number of the tape containing     ----

the raw data.

TCD$         Total cardon dioxide                       umol/kg

TED$         Total energy in the discrete portion of    m**2

the spectrum (Great Lakes 32 yr hindcast)

TEP$         Energy in the parametric portion of the    m**2

spectrum (Great Lakes 32 yr hindcast)

TIN1         Temperature instrument (from radio log     ----

of a BATHY or TESAC message)

TH1$         The time, as HHMM, of the first daily      ----

high

TH2$         The time, as HHMM, of the second daily     ----

high

TIM#         NODC originator's time (HHMM)              ----

TKI$         TRACKOB Indicator                          Mt Ms Mc ic

TL1$         The time, as HHMM, of the first daily      ----

low

TL2$         The time, as HHMM, of the second daily     ----

low

TMN$         The time, as HHMM, of occurrance of the    ----

minimum value. (Note the maximum value

is 2359.)

TMX$         The time, as HHMM, of occurrance of the    ----

maximum value. (Note the maximum value

is 2359.)

TSG$         Thermosalinograph temperature and          ----

salinity sensor

TSS$         Total suspended solids                     mg/m**3

TUR$         Turbidity                                  percent

UND#         NODC unknown data type                     code

URE$         Urea (H2NCONH2)                            mmol/m**3

VAP1         AES wave period (from the METOC            sec

hindcast).

VAV1         Average heave from the non-synoptic part   m

of WRIPS buoy data.

VAV2         Average heave from the synoptic part of    m

WRIPS buoy data.

VCA$         Apparent crest period of a wave            sec

VCH1         AES wave height (from the METOC            m

hindcast). It is the maximum wave height

found in the 5 degree square.

VCP1         Significant wave height of the most        m

energetic peak in a spectrum.

VCP2         Significant wave height of the second      m

most energetic peak in a spectrum.

VCR1         The significant wave height calculated     m

over the complete frequency range of a

spectrum.

VCS$         Estimate of the significant wave height    m

from slope measurements.

VEI1         Maximum E-W slope.                         ----

VEM1         Minimum E-W slope.                         ----

VEN1         E-W slope of the minimum wave.             ----

VERS         Software version number                    ----

VEX1         E-W slope of the maximum wave.             ----

VFF1         Non-MEDS assigned quality code (see        code

SEAKEM WAVEC documentation).

VIS$         Visibility code (WMO code table 4300)      code

VMD1         Primary Wave Mean Direction, Mean wave     degrees T

direction of highest peak in spectrum (Dm1

from AGRA)

VMD2         Secondary Wave Mean Direction, Mean wave   degrees T

direction of highest peak in spectrum (Dm2

from AGRA)

VMX1         Maximum Zero Crossing Wave Height from     m

the non-synoptic part of WRIPS buoy

data.

VMX2         Maximum Zero Crossing Wave Height from     m

the synoptic part of WRIPS buoy data.

VNI1         Maximum N-S slope.                         ----

VNM1         Minimum N-S slope.                         ----

VNN1         N-S slope of the minimum wave.             ----

VNR1         Spectral narrowness defined as             ----

SQRT((M1*M0-M1**2)/M1) (see SEAKEM

WAVEC documentation).

VNS1         Minimum slope.                             ----

VNX1         N-S slope of the maximum wave.             ----

VPA$         Average apparent wave period               sec

VPD1         Primary Wave Peak Direction, Peak wave     degrees T

direction of highest peak in spectrum (Dp1

from AGRA)

VPD2         Secondary Wave Peak Direction, Peak wave   degrees T

direction of highest peak in spectrum (Dp2

from AGRA)

VPK1         Peakedness defined as                      ----

2 (SUM (F(i)P2(i)dF(i)/MO2 (see SEAKEM

documentation for the WAVEC)

VPM$         Wave period of maximum wave height,        sec

Calculated from Tz(VTZA) (Thmax from AGRA)

VPN1         Period of the spectral minimum             sec

VSB#         Horizontal visibility (WMO code 4300)      code

VSP1         Primary Wave Direction Spread, Directional degrees

Spread of wave energy of highest peak in

spectrum (SPR1 from AGRA)

VSP2         Secondary Wave Direction Spread,           degrees

Directional Spread of wave energy of

highest peak in spectrum (SPR2 from AGRA)

VSN1         Spectral minimum                           m**2/hz

VST1         Maximum wave steepness.                    ----

VSX1         Spectral maximum                           m**2/hz

VSX2         Secondary Wave Spectral Density, Density   m**2/hz

of second highest peak in spectrum (SDp2

from AGRA)

VTD1         Dominant period.                           sec

VTP$         The original peak period as received at    sec

MEDS

VTP1         Secondary wave Peak Period, Period of the  sec

second highest peak in spectrum (Tp2 from

AGRA)

VTS$         The spectral peak from slope               ?????

measurements.

VWH$         The significant wave height supplied by    m

the data collector.

VXH$         Monthly extreme high waterlevel            m

VXL$         Monthly extreme low waterlevel             m

VXS1         Maximum slope.                             ----

VZA1         Average zero crossing period from the      sec

non-synoptic part of WRIPS buoy data.

VZA2        Average zero crossing period from the       sec

synoptic part of WRIPS buoy data.

VZS$        Average wave period from slope measurments  sec

WAIT        Used in the conversion of time to depth     ----

by ARGOS (by CSIRO)

t=WAIT+(FREQ*i) used in

depth(i)=a*t-0.001*b*t**2

WCF$        WMO code form designator                    ----

eg. 18-X BUOY

WDC$        Wind direction code table                   ----

WDR#        Direction from which wind is blowing        code

(WMO code 0877)

WFR1        Wind frictional velocity.                   m/sec

WHT$        Height of wind generated waves              m

WIR$        Wire out                                    m

WMO$        WMO platform identifier number              ----

WOI$        The WAVEOB indicator group 00IaImIp from    code

section 0 of the code.

WPR$        Period of wind generated waves              sec

WSP#        Horizontal wind speed                       knots

WSC$        Wind force code table                       ----

WSS$        Scalar wind speed (the arithmetic average   m/s

of wind speeds over the sampling period)

WVD#        Direction (true) from which waves are       code

coming (WMO code 0877)

WVH#        Wave height (WMO code 1555)                 code

WVP#        Wave period (WMO code 3155)                 code

XEQ$        XBT fall rate equation used (see IGOSS      code

table)

YEA#         NODC originator's year                     year

4.0 MEDS codes sorted alphabetically within GF3 tables

General purpose codes

---------------------

PPPP  KMMS       Parameter Name                         Units

----  ----   ---------------------------------------    --------

ACC$         MEDS accesion number                       ----

ADID         AODC database ID                           ----

ADST         Automatic gain control                     ----

ADSV         Automatic gain control variance            ----

AREA         NODC area code                             code

ARG$         ARGOS number given to a platform           ----

CCLL         Platform call sign                         ----

CSC1         Chief scientist's name                     ----

CSID         CSIRO database ID                          ----

CSN$         IGOSS call sign                            ----

CTF$         The indicator that encodes which quality   code

control tests for chemical data that

were failed.

CTP$         The indicator that encodes which quality   code

control tests on chemical data have been

executed.

DEC$         The code for why messages ceased.          code

DRT$         The drogue type code                       code

EAG1         Name of the exchange agency                ----

EXP$         The originator's experiment number         ----

GNC#         NODC cruise code                           code

INS1         Institute                                  ----

INTP         Instrument type (used with wavec data)     code

1 = Waverider

2 = WAVEC

3 = ENDECO

4 = Satellite waverider

IST$         Instrument name                            ----

JPT$         JGOFS Protocol Code Table                  ----

NID#         NODC internal database identifier          code

NPT#         NODC P3 format flag indicating that a      code

station has either only derived values or

at least one profile of measured values

NUL$         A null parameter. It is used when it is    ----

necessary to reserve the position of

code.

OCL#         NODC OCL reference number                  ----

OCR$         The originator's cruise number             ----

OID$         Originator's station identifier            ----

OLS#         NODC OCL unique station number             ----

ONM1         Originator's name                          ----

OSI$         Originator's sample identifier             ----

PGI#         NODC P3 software identifier                code

PNM1         Platform name                              ----

PLA#         NODC platform/ship code                    code

PRO1         Project name                               ----

PRO#         NODC project code                          code

PTP1         Platform type                              code

QCF$         The indicator that encodes which quality   code

control tests have failed for

temperature and salinity data.

QCP$         The indicator that encodes which quality   code

control tests have been executed for

temperature and salinity data.

QFF$         The indicator that encodes which ORSTOM    code

quality control tests have failed for

temperature and salinity data.

QFP$         The indicator that encodes which ORSTOM    code

quality control tests have been

executed for temperature and salinity

data.

QNF#         NODC indicator of QC tests failed at the   code

station level

QNP#         NODC indicator of QC tests made at the     code

station level

RCRD         Indicates a record has been created.       code

REC$         A record in the MEDS time series format.   ----

REP$         Records that one or more profiles have     code

replicate samples at one or more depths.

The profiles affected are encoded in the

same way QCF$, QCP$ are set

SER1         Instrument serial number                   ----

SHP#         Ship name                                  ----

STI#         Surface temperature instrument type        NODC code

SVR$         SEAS version number                        code

UND#         NODC unknown data type                     code

VERS         Software version number                    ----

VFF1         Non-MEDS assigned quality code (see        code

SEAKEM WAVEC documentation).

WMO$         WMO platform identifier number             ----

Methods codes

---------------------

PPPP  KMMS       Parameter Name                         Units

----  ----   ---------------------------------------    --------

MRT#         NODC instrument for referrence temperature ----

MTH$         Links a observed value PCODE to a method   ----

or insturmentation PCODE

STM#         Sea surface temperature methodolgy         code

WAIT         Used in the conversion of time to depth    ----

by ARGOS (by CSIRO)

Instrument characteristics codes

---------------------

PPPP  KMMS       Parameter Name                         Units

----  ----   ---------------------------------------    --------

NET$         Net mesh size                              microns

SCR$         Instrument scan rate (from towed CTDs)     secs

Biological Oceanography codes

-----------------------------

PPPP  KMMS       Parameter Name                         Units

----  ----   ---------------------------------------    --------

CPH$         Chlorophyll-a content, determined by the   mg/m**3

Welchmeyer method

PAR$         Photosynthetic active radiation            uE/m**2-s

PAR1         Photosynthetic active radiation            unknown

PHA$         Phaeophytin                                mg/m***3

PRP$         Primary productivity                       mgC/m**3/hr

PRQ$         Primary productivity                       mgC/m**3/day

URE$         Urea (H2NCONH2)                            mmol/m**3

TSS$         Total suspended solids                     mg/m**3

Data Operation codes

---------------------

PPPP  KMMS       Parameter Name                         Units

----  ----   ---------------------------------------    --------

MNS$         Minimum value of salinity                  PSU

MNT$         Minimum value of temperature               degC

MXS$         Maximum value of salinity                  PSU

MXT$         Maximum value of temperature               degC

NUM$         Number of values used in computation       ----

SDT$         Standard deviation of temperature          degC

SDS$         Standard deviation of salinity             PSU

TKI$         TRACKOB Indicator                          Mt Ms Mc ic

Date and time within day codes

------------------------------

PPPP  KMMS       Parameter Name                         Units

----  ----   ---------------------------------------    --------

DAT#         NODC originator's date                     ----

DDB$         The date as YYYYMMDD of deployment         ----

DEB$         The date as YYYYMMDD of the last message   ----

received.

DET$         The time as HHMM of the last message       ----

received.

DHM$         Day, hour, and minute as DDHHMM and a      ----

real number.

DN3$         The date as YYYYMMDD of analysis of

Nitrate

DPO$         The date as YYYYMMDD of analysis of

Phosphate

DPT$         The deployment time (GMT) as HHMM          ----

DRD$         The date as YYYYMMDD that the drogue       ----

was lost

DSO$         The date as YYYYMMDD of analysis of

Silicate

DTC$         Calibration date as YYYYMMDD               ----

DTE$         The day, month and year as indicated in    ----

IGOSS messages. So DDMMY where DD is the

day, MM is the month and Y is the last

digit of the year.

DTH$         The day, hour and minute of the extreme    ----

high

DTL$         The day, hour and minute of the extreme    ----

low

DT2$         The date as YYYYMMDD of the end of a tow.  ----

When this is used, the date field

records the start date.

TIM#         NODC originator's time (HHMM)              ----

YEA#         NODC originator's year                     year

Time and frequency codes

------------------------

PPPP  KMMS       Parameter Name                         Units

----  ----   ---------------------------------------    --------

HM2$         The time as HHMM of the the end of a tow.  ----

When this is used, the time field

records the start time.

PDT$         The date as YYMMJ of the last position     ----

(from the DRIBU message)

PTM$         The time as HHMM/ of the last position     ----

(from the DRIBU message)

SMI$         Interval between burst samples             mins

Position and navigation codes

-----------------------------

PPPP  KMMS       Parameter Name                         Units

----  ----   ---------------------------------------    --------

DCLN         The magnetic declination                   degrees

DEOW         Code to indicate if the magnetic           code

declination is east or west:

1 = East, 2 = West

DLA$         Latitude (north +ve) at deployment         degrees

(see DLT$)

DLO$         Longitude (+/- 180 west +ve) at            degrees

deployment

DLT$         Latitude (north +ve) at deployment         degrees

ELA$         The error in latitude                      degrees

ELO$         The error in longitude                     degrees

LAT$         The latitude in IGOSS format of DDMM       ----

where DD is the degrees and MM the

minutes of latitude.

LAT#         NODC originators latitude                  degrees

LNG$         Longitude from GPS (+ West)                degrees

LN2$         The second longitude (from DRIBU           degrees

message or for ocean profile data the

longitude records the start longitude and

this records the end longitude of a tow)

LON$         The longitude in IGOSS format of DDMM      ----

where DD is the degrees and MM the

minutes of longitude.

LON#         NODC originators longitude                 degrees

LTG$         Latitude from GPS (+ North)                degrees

LT2$         The second latitude (from DRIBU            degrees

message or for ocean profile data the

latitude records the start latitude and

this records the end latitude of a tow)

SLA$         The last latitude recorded                 degrees

SLO$         The last longitude recorded                degrees

Physical oceanography codes

---------------------------

PPPP  KMMS       Parameter Name                         Units

----  ----   ---------------------------------------    --------

BKT$         Bucket sample                              ----

BTC1         BT temperature correction                  deg.C

BTN1         BT slide number                            ----

BTR1         BT surface reference temperature           deg.C

BTS1         BT surface temperature                     deg.C

EIT$         Engine intake water sample                 ----

EST$         The error in SST                           deg.C

GPA$         Geopotential anomaly                       dyn.m

HMT$         Hull mounted sensor                        ----

HTL$         Height of the XBT launcher above the       m

water

ICT$         Ice thickness                              m

PEA$         Potential energy anomaly                   Kg-m/sec**2

PLT$         Percent light transmittance                percent

PRT$         Probe type code (see IGOSS code table)     code

NODC uses PRT#

PRV$         Velocity probe                             m/s

RCT$         XBT recorder type (see IGOSS code table)   code

This is equivalent to DKT$.

RTC$         Reference temperature code                 code

RTD$         Reference temperature water depth          m

RTM$         Reference temperature                      degC

SDA$         Sea density anomaly                        Kg/m**3

SGT$         Sigma-t                                    kg/m**3

SRT$         Sea surface reference temperature          deg.C

SSD#         NODC Depth of sea surface current sensor   m

STP$         Sampler type                               code

SVA$         Specific volume anomaly                    m**3/Kg

TSG$         Thermosalinograph temperature and          ----

salinity sensor

TUR$         Turbidity                                  percent

WIR$         Wire out                                   m

XEQ$         XBT fall rate equation used (see IGOSS     code

table)

Wave codes

----------

PPPP  KMMS       Parameter Name                         Units

----  ----   ---------------------------------------    --------

ADNB         Number of blocks averaged in the           ----

spectrum. (Note that this is the same as

NBL1)

ADNS         Number of samples per fourier block        ----

AED1         AES wave direction (from METOC hindcast).  degrees

This is probably the wind direction.

AST1         Internal temperature from the              deg.C

non-synoptic part of WRIPS buoy data.

AST2         Internal temperature from the synoptic     deg.C

part of WRIPS buoy data.

CLW1         The divider network setting on the FM to   ----

digital conversion system

CLW2         The ratio of playback to record speed for  ----

the digitizer

CLW3         The clock frequency                        Hz

CLW4         The percent modulation of the waverider    Percent

FM signal

CLW5         The mean center frequency of the           Hz

waverider FM signal

CLW6         The zero intercept on the frequency axis   Hz

for frequency vs height

CLW7         The slope of the frequency/height          m/Hz

calibration

CLW8         The height of the pressure cell above the  m

bottom

DSP1         The Juszko directional spread factor for   ----

the most energetic peak in a spectrum.

DSP2         The Juszko directional spread factor for   ----

the second most energetic peak in a

spectrum

ERS$         Percent residual least squares error by    ----

Juszko in 10 parameter model fit to

directional spectrum.

FRQ1         Frequency of the most energetic peak       hertz

in spectrum

FRQ2         Frequency of the second most energetic     hertz

peak in spectrum

LBL1         The block length                           sec

NBD1         The number of bad samples in a surface     ----

elevation time series.

NBL1         The number of blocks averaged to derive    ----

a wave spectrum.

NWAV         The number of waves in the record          ----

ODD1         The direction from which waves are         degrees

coming for the most energetic peak in

a spectrum

ODD2         The direction from which waves are         degrees

coming for the second most energetic

peak in a spectrum

RECD         The record number of the tape containing   ----

the raw data.

SED1         WES sea direction.                         degrees T

SEH1         WES sea height.                            m

SEM$         Significant period M)/M1 of wind sea       sec

SEP1         WES sea period.                            sec

SHA1         Ochi and Hubble spectral shape parameter   ----

for the most energetic peak in the

spectrum

SHA2         Ochi and Hubble spectral shape parameter   ----

for the second most energetic peak

in the spectrum

SIDE         The side number of the tape containing     ----

the raw data. (Note that this

duplicates a standard GF3 code used for

another purpose)

SPC$         Surface Current Magnitude Spread, RMS      m/s

deviation between measured current

components and decomposition of the

estimated current vector in the

antenna-looking directions. For a

homogeneous current, a small value

indicates a good current estimate.

(SPRC from AGRA)

SST1         Average sea temperature from the           deg.C

non-synoptic part of WRIPS buoy data.

SST2         Average sea temperature from the           deg.C

synoptic part of WRIPS buoy data.

STC$         Spectral type code by Juszko               ----

SWD#         Swell direction (WMO code 0877)            code

SWM$         Significant period M0/M1 of swell          sec

TAPE         The tape number of the tape containing     ----

the raw data.

TED$         Total energy in the discrete portion of    m**2

the spectrum (Great Lakes 32 yr hindcast)

TEP$         Energy in the parametric portion of the    m**2

spectrum (Great Lakes 32 yr hindcast)

VAP1         AES wave period (from the METOC            sec

hindcast).

VAV1         Average heave from the non-synoptic part   m

of WRIPS buoy data.

VAV2         Average heave from the synoptic part of    m

WRIPS buoy data.

VCA$         Apparent crest period of a wave            sec

VCH1         AES wave height (from the METOC            m

hindcast). It is the maximum wave height

found in the 5 degree square.

VCP1         Significant wave height of the most        m

energetic peak in a spectrum.

VCP2         Significant wave height of the second      m

most energetic peak in a spectrum.

VCR1         The significant wave height calculated     m

over the complete frequency range of a

spectrum.

VCS$         Estimate of the significant wave height    m

from slope measurements.

VEI1         Maximum E-W slope.                         ----

VEM1         Minimum E-W slope.                         ----

VEN1         E-W slope of the minimum wave.             ----

VEX1         E-W slope of the maximum wave.             ----

VMD1         Primary Wave Mean Direction, Mean wave     degrees T

direction of highest peak in spectrum (Dm1

from AGRA)

VMD2         Secondary Wave Mean Direction, Mean wave   degrees T

direction of highest peak in spectrum (Dm2

from AGRA)

VMX1         Maximum Zero Crossing Wave Height from     m

the non-synoptic part of WRIPS buoy

data.

VMX2         Maximum Zero Crossing Wave Height from     m

the synoptic part of WRIPS buoy data.

VNI1         Maximum N-S slope.                         ----

VNM1         Minimum N-S slope.                         ----

VNN1         N-S slope of the minimum wave.             ----

VNR1         Spectral narrowness defined as             ----

SQRT((M1*M0-M1**2)/M1) (see SEAKEM

WAVEC documentation).

VNS1         Minimum slope.                             ----

VNX1         N-S slope of the maximum wave.             ----

VPA$         Average apparent wave period               sec

VPD1         Primary Wave Peak Direction, Peak wave     degrees T

direction of highest peak in spectrum (Dp1

from AGRA)

VPD2         Secondary Wave Peak Direction, Peak wave   degrees T

direction of highest peak in spectrum (Dp2

from AGRA)

VPK1         Peakedness defined as                      ----

2 (SUM (F(i)P2(i)dF(i)/MO2 (see SEAKEM

documentation for the WAVEC)

VPM$         Wave period of maximum wave height,        sec

Calculated from Tz(VTZA) (Thmax from AGRA)

VPN1         Period of the spectral minimum             sec

VSP1         Primary Wave Direction Spread, Directional degrees

Spread of wave energy of highest peak in

spectrum (SPR1 from AGRA)

VSP2         Secondary Wave Direction Spread,           degrees

Directional Spread of wave energy of

highest peak in spectrum (SPR2 from AGRA)

VSN1         Spectral minimum                           m**2/hz

VST1         Maximum wave steepness.                    ----

VSX1         Spectral maximum                           m**2/hz

VSX2         Secondary Wave Spectral Density, Density   m**2/hz

of second highest peak in spectrum (SDp2

from AGRA)

VTD1         Dominant period.                           sec

VTP$         The original peak period as received at    sec

MEDS

VTP1         Secondary wave Peak Period, Period of the  sec

second highest peak in spectrum (Tp2 from

AGRA)

VTS$         The spectral peak from slope               ?????

measurements.

VWH$         The significant wave height supplied by    m

the data collector.

VXS1         Maximum slope.                             ----

VZA1         Average zero crossing period from the      sec

non-synoptic part of WRIPS buoy data.

VZA2         Average zero crossing period from the      sec

synoptic part of WRIPS buoy data.

VZS$         Average wave period from slope             sec

measurements

WHT$         Height of wind generated waves             m

WPR$         Period of wind generated waves             sec

WVD#         Direction (true) from which waves are      code

coming (WMO code 0877)

WVH#         Wave height (WMO code 1555)                code

WVP#         Wave period (WMO code 3155)                code

Meteorology codes

-----------------

PPPP  KMMS       Parameter Name                         Units

----  ----   ---------------------------------------    --------

GPT$         Pressure tendency code (see WMO code       code

table 0200)

VIS$         Visibility code (WMO code table 4300)      code

VSB#         Horizontal visibility (WMO code 4300)      code

WDC$        Wind direction code table                   ----

WDR#        Direction from which wind is blowing        code

(WMO code 0877)

WFR1         Wind frictional velocity.                  m/sec

WSC$        Wind force code table                       ----

WSP#        Horizontal wind speed                       knots

WSS$        Scalar wind speed (the arithmetic average   m/s

of wind speeds over the sampling period)

Geophysics codes

----------------

PPPP  KMMS       Parameter Name                         Units

----  ----   ---------------------------------------    --------

SOU$         Sounding depth (See BATH)                  m

Chemistry codes

---------------

PPPP  KMMS       Parameter Name                         Units

----  ----   ---------------------------------------    --------

B12$         Vitamin B12                                milli-microg/lt

CDX1         Dissolved organic C = CORG DX (mg/lt)      ?????

CPX1         Particulate C = CORG PX (mg/lt)            ?????

FLU$         Flourescence                               %

FLU1         Fluorescence                               mg/m**3

FLU2         Fluorescence                               volts

F11$         CFC11 measurements                         pmol/kg

F12$         CFC12 measurements                         pmol/kg

FLO1         Flouride                                   ????

NH3$         NH3-N in micrograms/liter                  ?????

OLP$         Particulate oil                            mg/m2

OLW$         Total dissolved/dispersed oil              mg/m3

OXS$         Percent Oxygen saturation                  percent

O18$         O18/O16 ratio                              percent

PHI$         Inorganic phosphate                        mmol/m**3

PH1$         Replicate sample of inorganic phosphate    mmol/m**3

PON$         Particulate Organic Nitrogen               mg/**3

SUL$         Sulphate (SO4)                             mol/m**3

TCD$         Total cardon dioxide                       umol/kg

Special IGOSS Indicator codes

-----------------------------

PPPP  KMMS       Parameter Name                         Units

----  ----   ---------------------------------------    --------

APC1         Averaging period (in TRACKOB: mC) for      minutes

surface current

APR$         The group in TRACKOB supplying the         ----

averging periods used for temperature,

salinity, currents and the units of the

current.

APS1         Averaging period (in TRACKOB: mS) for      minutes

salinity

APT1         Averaging period (in TRACKOB: mT) for      minutes

sea surface temperature

BCI$         The indicator k5 used in the BATHY code    code

to tell the method of current

measurement. This is equivalent to CCGM

BQA$         QA in BUOY code form                       ----

BYM$         Previous bulletin year-month as YYMM       ----

DKT$         Recorder type for IGOSS BTs. This is       ----

equivalent to RCT$

FRE$         Code for fall rate equation used. See      code

IGOSS code table.

GE1$         Contents of the first engineering group    code

in a DRIBU message

GE2$         Contents of the second engineering group   code

in a DRIBU message

GE3$         Contents of the third engineering group    code

in a DRIBU message

GIN$         Contents of the group 1QpQ2QtwQ4 in the    code

DRIBU message

GGC$         The 5 character 66... group used in a      code

TESAC message to indicate information

about the subsurface data collection.

GGI$         The 5 character 88.. group used in a       code

BATHY and a TESAC message to indicate

information about the subsurface profile.

GGK6         Method of removing ship velocity code      code

(k6 of a TESAC message)

GG1$          The contents of the group 111QdQx in      ----

FM18-X BUOY code form

GG2$          The contents of the group 222QdQx in      ----

FM18-X BUOY code form

GG3$          The contents of the group 333QdQx in      ----

FM18-X BUOY code form

PEQ$         Probe type and fall rate equation code     ----

table (IGOSS)

PFR$         WMO codes 1771 (IxIxIx) + 4770 (XrXr)      code

as used in the JJYY BATHY code

SIN1         Salinity instrument (from radio log of a   ----

BATHY or TESAC message)

TIN1         Temperature instrument (from radio log     ----

of a BATHY or TESAC message)

WCF$        WMO code form designator                    ----

eg. 18-X BUOY

WOI$        The WAVEOB indicator group 00IaImIp from    code

section 0 of the code.

Water Level codes

-----------------

PPPP  KMMS       Parameter Name                         Units

----  ----   ---------------------------------------    --------

DMH$         The day of the month, as DD, of the        day

extreme high value.

DML$         The day of the month, as DD, of the        day

extreme low value.

DMN$         The daily mean value.                      m

HGR$         Hourly height water level derived from     m

MEDS Gradicon to cm precision

HIN$         The integrated water level for an hour.    m

HIS$         Hourly height water level derived from     m

unknown historical sources to cm

precision

HL1$         The height of the first daily low          m

HL2$         The height of the second daily low         m

HMN$         The minimum height read in an hour.        m

HMX$         The maximum height read in an hour.        m

HOG$         Hourly height water level derived from     m

Ogdensberg to centimeter precision

HOH$         Hourly height water level derived from     m

Ontario Hydro to centimeter precision

HPR$         Hourly height predicted value.             m

HSC$         Hourly height water level derived from     m

MEDS Scalar to cm precision

HTH$         Hourly heights supplied from the West      m

Coast.

HW1$         The height of the first daily high         m

HW2$         The height of the second daily high        m

HZY$         Hourly height water level derived from     m

MEDS Zycor to cm precision

H00$         Hourly heights supplied by MEDS from       m

TATS gauges.

H15$         Height value read 15 minutes after the     m

hour.

H30$         Height value read 30 minutes after the     m

hour.

H45$         Height value read 45 minutes after the     m

hour.

H60$         Hourly height water level derived from     m

TATS stations and reduced by rounding

to a precision of cm

LW1$         The height of the first daily low          m

LW2$         The height of the second daily low         m

MMN$         Monthly mean waterlevel                    m

TMN$         The time, as HHMM, of occurrance of the    ----

minimum value. (Note the maximum value

is 2359.)

TMX$         The time, as HHMM, of occurrance of the    ----

maximum value. (Note the maximum value

is 2359.)

TH1$         The time, as HHMM, of the first daily      ----

high

TH2$         The time, as HHMM, of the second daily     ----

high

TL1$         The time, as HHMM, of the first daily      ----

low

TL2$         The time, as HHMM, of the second daily     ----

low

VXH$         Monthly extreme high waterlevel            m

VXL$         Monthly extreme low waterlevel             m