ENDEAVOR CRUISE 223

THIS IS A STATION BY STATION DOCUMENTATION FILE CREATED AFTER THE CRUISE

FROM THE STATION LOG SHEETS TO HAVE SOME RECORD ON DISK OF STATION EVENTS.

TEST STATION 999:

Extremely noisy data.  Looked good til about 120 db then dropped out.

Will reterminate after cast.

Only first bottle confirmed.

Microvax crashed 2 times with Integer*2 errors.

Bottles 15 and 16 broken during recovery.

Bottle 19 leaked.

Bottle 22 no water.

STATION 1:

CTD down to about 100 db before realized RS232 not connected to Deck Unit.

Brought CTD back up and redeployed.

Raw data unreadable.  Header file messed up due to integer*4 modifications.

Bottle #2 broken during recovery.

Bottle #3 leaked.

Bottle #7 no water.

STATION 2:

Winch slowed to 10 m/min @ 2330db due to large wire angle.

Bottle #3 leaked.

Bottle #11 no water.

STATION 3:

No water in bottles #3, #13, #20.

STATION 4:

Bottle #3 leaked.

No record tags for bottles at 1850db, 1600db, and 700db.

STATION 5:

Bottle #23 leaked.

STATION 6:

No record tag at 500db.

Bottles #3 and #9 leaked.

STATION 7:

No record tag at 900db.

Bottles #7 and #24 leaked.

STATION 8:

No record tags at 1450db or 750db.

Bottles #3 and #15 leaked.

STATION 9:

Extra scan tagged in rosette file during upcast.

Bottles #3 and #12 leaked.

STATION 10:

Log sheet says:  "Depth at middle of cast (10m from bottom): 1293"

whatever that means.

STATION 11:

Record tag #7 tagged twice (the first is no good).

Bottles #7 and #9 leaked.

STATION 12:

Bottle #3 leaked.

STATION 13:

Winch stopped suddenly on downcast at 3570db.

Bottle #19 leaked.

STATION 14:

Bottles #3,15,16,18 leaked.

STATION 15:

Bottles #3, #12 leaked.

STATION 16:

Cast #1 aborted in order to manuever away from another ship.

Bottles #6, #11 leaked.

STATION 17:

Aquisition stopped logging on upcast at 2850db.  No Msg.

Bottles #3,#16,#17,#20,#22 leaked.

STATION 18:

OK

STATION 19:

Bottles #16,#17 leaked.

STATION 20:

No record tag at 2830 db.

All bottles collected by rosette firing pin at '23'.

Will use bottles to determine.  Suspect 20-23 should be one depth deeper.

Those below 20 may also be shifted?

STATION 21:

OK

STATION 22:

Cast 1 total wire out = 1640.  All stop cocks open.  No H2O samples.

Redo this station.

Cast 2:  OK

STATION 23:

Disregard 1st and 9th CTD record tags.

Bottle #3 leaked.

Bottle #4 no water.

STATION 24:

OK.

STATION 25:

OK

STATION 26:

OK

STATION 27:

OK

STATION 28:

Stopped briefly at 620 db to correct plotter and find PDR crossing point.

Bottle #3 leaked.

Bottles 1,2 leaked too.

STATIOIN 29:

OK

STATION 30:

No record tag at 700 db.

STATION 31:

OK

STATION 32:

OK

STATION 33:

Bottle #23 did not fire.

Suspect firing pin was set to fire bottle 24 first.

STATION 34:

Two record tags at 1200 db.  Disregard first of the two.

STATION 35:

OK

STATION 36:

Record tag for bottom bottle erroneous.  Take information from log sheet.

STATION 37:

OK

STATION 38:

OK

STATION 39:

Missing first 800db of downcast on audio tape.

STATION 40:

OK

STATION 41:

OK

STATION 42:

First scan in record tag file is extra.  Disregard it.

STATION 43:

OK

STATION 44:

OK

STATION 45:

Some of downcast missing on audio recording.

No record tag for bottle at 2550db.

STATION 46:

OK

STATION 47:

This station has a confusing write up on record tag and bottle ordering,

but as far as I can tell, there is simply an extra tag between 1800db and

1600 db in the ORIGINAL tag file.