MICRO-VAX CTD DATA PROCESSING SYSTEM:  KNORR 138   WOCE   P6

AT WOODS HOLE OCEANOGRAPHIC INSTITUTION

CAM. APR 92.

Rev May 92.

General notes on the micro-VAX and usage

The processing described herin will constantly refer to various VAX

proceedures used.  Knowledge of VAX proceedures such as Editing, Copying,

and Command language use are necessary to do the processing.

Command files exist for most of the programs used in the processing.

These files allow parameters required by the programs to be supplied from

a file, rather than entered interactively.  This speeds up the process and

lessens the chance of operator error in inputting large sets of parameters.

Command files are identified by the .COM suffix to the filename.  Comments

(following an !) are written on most of the lines that need to be changed

in the command files.

For each station the command files must be edited, to change any

station specific parameters (latitude, longitude,water depth, time, etc.).

When the station specific changes are completed, the program run by the

command file can be executed.  Example command files supplied for

the following programs: CTDED, PRSORT, STAPLOT, and CTDLIST84.

The attached table shows the normal sequence of events for the

CTD data processing.  The subsequent documentation describes how the

data acquired by the processes used on the WHOI KNORR CRUISE 138 during

May-August 1992.  The Acquisition process is described elsewhere.

Subdirectories have been set up.  The root directory is [CTD.KN138P003].

Subdirectories have been created for easier separation of data processing

tasks.  These include:  cals, cond, ctded78, hydro, mver, oxy, plot5 and

ros.  The cals subdir contains all the CTD instrument calibration files

and their pre-cruise fits.  The ctded78 subdir contains the command files

to edit the aqui89 .RAW data files into clean downtraces with corrected

header information.  The .EDT files get stored here until backup to 9T

and TK50.  The hydro subdir contains all the .TMP, .SOX, & .NUT files.

Both the hydro and nutrient groups have access to this subdir, as they

extract the .tmp files and put back their corresponding .sox or .nut files.

The ros subdir is then used to merge the .tmp and .sox files to produce

files for overplotting (.DYN) and for calibration fitting (.WSD).  The

.WSD file is then transferred to the cond and oxy subdir to fit for either

conductivity or oxygen.  Once the .CTD binary data (top level dir) has

been processed with oxygens (converting M versions into X versions),

the old 'M' versions can be stowed away into the mver subdir in case

the oxygens need to be reprocessed.  The plot5 subdir is used when

making PLOT5 plots.

PROCESSING SUMMARY:

Refer to "CTD DATA PROCESSING" notebook for more detail, and

see attached command files for examples.

I.     ACQUIRE TO AND PROCESS DATA FROM 9T TAPE

II.    FILL OUT CTD CRUISE LOG ON BIG CLIPBOARD

(the data for the cruise log sheet is obtained from the individual

station log sheets.)

THIS WILL MAKE THE CTDED78 STEPS ROUTINE

III.   CTDED78 EDIT

a.) SET DEF [CTD.KN138P003.CTDED78]

b.) EDIT CTDED###.COM to edit STA #, DATE, TIME, P MAX, DEPTH,

AND RECORD LIMITS ACCORDING TO CTD CRUISE LOG

****Note:  The edit program has been found to truncate the data

by 7-13 dbars if the true pressure limits are recorded.

The program does NOT simply edit by record limits.  To avoid

truncation, add 15-20 dbars on to the max pressure limit

in the command file.

Note: If you change instruments, you will need to edit the instrument

number and the cals associated with it

c.) Type @CTDED### (executes CTDED)

d.) Print output file (CTDEDXXX.LST) for process listing notebook

e.) You will ALSO need REC LIMs for running LOGCTDED#.COM

Note: UNLESS THERE IS SOME REAL PROBLEM WITH THE DATA YOU SHOULDN'T HAVE

TO CHANGE THE EDIT CRITERIA in CTDED###.COM.  IF THERE IS A REAL

PROBLEM WITH NOISE LOOK IN THE  CTD DATA PROCESSING MANUAL FOR

INSTRUCTIONS.  INCREASING MAXBAD TO A HIGHER NUMBER USUALLY

WORKS.  MAXBAD IS SET TO 3 FOR GENERALLY CLEAN DATA.

IV.  PLOT78

a.) You can use this program as a tool to check or look at

raw data files (CTDED78 format), *.RAW or *.EDT.

It is a good idea to verify the output log file from AQUI89

in dua4:[ctd.aqui.log] to make sure record limits are being

calculated from max pressure!

$ASSign 0035A000.RAW for035      !msa0: is other option

$R [CTD.CTDATA.EXE]PLT78M       (= logical plt78)

6,-35/    Output, Input  (note: input is - for disk,

+ for tape)

138,03/    Cruise #, Project #

IDs for X & Y_  R#,PR/     Rec #, Press   (note: must be in caps)

----

Change line number_2       (Change PR limits)

?_0,6500,500,500,2

Change line number_5       (ISCAN)

?_146,0,1 (1 scan every rec,plot off,list on)

Change line number_4       (REC limits)

?_500,1250

Change line number_0       To Continue

>_1

/               This will list out data  (= do loop)

Line #12: errchk=0  (lists all data incl. flagged data)

errchk=-1 (flagged data omitted, smoother)

CTRL C to exit or change user input

V.  BACKUP STEP

(to be done every 5 stations, or when out of space, whichever

happens sooner.)

a.) Use BACKUP to copy *.RAW files to TK50. You can get up to

10 stations on one tape.  Label tapes AQRAW1, AQRAW2...

b.) Do the same for *.EDT files.  Label the TK50's CTDED1, CTDED2..

You can get twice as many .edt files as .raw files on one

tape since the .edt file contains only the editted downcast.

c.) Next, copy the *.EDT files to 9T tape in a format that PRSORT

can then read.  There's a command file in [.ctded78] called

LOGCTDED#.COM that will run CTLOG.  This must be editted

with the correct station numbers and corresponding record limits.

NOTE:  Each new tape MUST be initialized with CTID first!  If you

copy only a few stations at a time, you can keep using the same

tape (up to 10 stations).  In this case do NOT initialize the

tape again.  THE TAPE WILL SKIP TO THE END of THE LAST FILE ON

THAT TAPE BEFORE COPYING THE NEXT.

$mou/for msa0:

$ass msa0: for009

$CTID

1   (# tape drives)

03   (project #)

KN138 CTDED P601   (objective)

P601 (tape ID)

P601 (source ID)

CTRL-Y to exit

$SET MAG/REW/NOUNLOAD MSA0:

$DEASS FOR009

d.)  CTDISK will copy a 9 track file to disk, if by chance a disk

file is corrupt or low disk space closes the file during

acquisition.

$mou/for/block=2064

$ass msa0: for010

$ass 0001A001.raw for011

$RUN CTDISK

VI. PRESSURE SORTING

a.) SET DEF <CTD.KN138P003> (to move back to the root subdirectory)

b.) EDIT PSSTA##.PST/OUTPUT=PSSTA##.COM  ACCORDING TO STATION #

The .PST suffix indicates that the PRSORT program has operated

on the command file of the same first name.  So edit the *.PST

file to create a command file from which the program can be

executed.

The scaling factors are all set up with PRE-Cruise lab cals

When rosette samples from several stations are available,

the data should be fit to these values and the calibration

factors changed accordingly.

ALSO, there are different command files for running PRSORT

on files from tape and disk.  Make sure you have the right

one.

All WOCE cruises use an initial cast # = 1.  If Gerrard

stations occur first, casts may be as high as 5.  Make

sure the cast number in the command file agrees with the

actual cast identified on the log sheet.

c.) type @PSSTA## to execute the command file that runs PRSORT.

d.) The output file (*.DLD) for this will go to the printer

automatically.

Now you have *M*.CTD versions in <CTD.KN138P003>

(this file name consists of sta#,"M",cast#.CTD

ie., 0054M001.CTD)

VII. INITIAL PLOTTING

a.) Edit <CTD.KN138P003>STAPLOT.COM

you'll need to change station number , cast number and

depth

b.) type  @STAPLOT

c.) make sure the ZETA plotter is turned "ON" and ready,

then type    ZPLOT, and when prompted, type   G (For GO)

This will make a plot of SA,TE,OX vs. PR for each station.

(file in Plot Notebook)

VIII. THETA/SALINITY PLOTS

a.) Edit <CTD.KN138P003>CTD5TS.COM

edit for sta#, cast#, correct directory, .DYN file, and axes scales

b.) type @CTD5TS

c.) make sure the ZETA plotter is turned "ON" and ready,

then type    ZPLOT, and when prompted, type   G.

Overplot water sample data if it's available.  To produce a *.DYN

file run HYDOUT (see XII).

IX.  MERGING THE SALINITY AND OXYGEN VALUES WITH THE TAG AND HEADER FILES

1.   The command file r2d2:[ctd.aqui.btldata]MAKETEMP.com will do

the following four steps a-d.

a.) COPY the .wrw (unscaled tag file) and .hed (header file) from

the dua4:[ctd.aqui.data] acquisition microvax to a

subdirectory on the post-processing microvax r2d2:[ctd.aqui.btldata].

Note: dua4 is synonymous with ctd3

b.) RUN BTLFMTVX

Input station:   0003a001        (no extension, this id grabs

*.wrw, *.hed with same name.

A template calmodel.c00 file

is also needed for input.)

#.BT2 is output automatically, along with a #.C00 file which

is the calibration file for that station, ie. it has the

calibrations from the .hed file overwritten into the

template calmodel.c00 file.

c.)  RUN WOCTMPV2

Input station:  0003a001.bt2    (id grabs *.c00 with the same name.)

/               (type in cast number if other than 1)

#.TMP is output automatically.

d.)  Copy #.TMP files over to the HYDRO subdir ie

[CTD.KN138P003.hydro]

The hydrographers will ftp the .tmp files to their

PC's and then ftp back their corresponding .SOX files.

The nutrient folks will do the same and ftp back their

.NUT files.

2.   Some command files are set up in [ctd.kn138p003.hydro] to

merge watersample data with SEAMERG2:

TMPMERG2.com, SOXMERG2.com, NUTMERG2.com.

e.)  RUN SEAMERG2

Input:  append *.tmp files

add *.sox and *.nut files

Output:  ship/cruise.SEA     =WOCE format watersample file

This program was written by Jane Dunworth and has its own

documentation.

f.)  $COPY [ctd.kn138p003.hydro]*.sox and [ctd.aqui.BTLdata]*A*.bt2 files

to  [ctd.kn138p003.ros] to prepare for the next step.

3.  Command file is [ctd.kn138p003.ros]BTLMERG2.COM

g.)  RUN BTLMRGV2

Input station:  0003a001          (no extension, this id grabs

*.bt2 and *.sox of the same name.)

Output:  0003S001.BT2             (raw reformatted CTD data with

-                     bottle salinity and oxygen.)

Need to append all the *S*.bt2 files together into a file

with the extension .LST.  Note:  this file contains a special

header.  You should have a .LST file for each instrument.

A separate .LST file for each CTD is needed for input to CONVERT

for fitting each instrument's conductivity sensor.

EX: KN138C10.LST

X.   BOTTLE DATA FIT COMPUTATIONS

The basic conductivity calibration sequence is comprised of three

programs:  PBIAS, CONVERT and CONCALD.  PBIAS and CONVERT are run before

the actual fitting.

1.  RUN PBIAS

Input:  Single *.c00 file for one station

Can also use a batch dir listing

Output:  ship/cruise/ctd.PBI

***BEST OPTION: You can simply edit and append on to the .PBI file

instead of running this program.  The pbias for each station can

be taken from the cruise log sheet and manually inserted into the file.

2.  RUN CONVERT

Input:  ship/cruise/ctd               (no extension, this id grabs

*.LST, *.PBI, *.C00 with same

name.  Make sure you create

a *.c00 file with the correct

cals.)

Output:  ship/cruise/ctd.WSD          (inverts bottle salt to cond

using P & T cals in above .c00

file.)

NOTE: You should have a .WSD file for each instrument. IE

KN138C10.WSD, KN138C7.WSD, KN138C9.WSD.  Then make an aggregate

.WSD file with all instruments (stations) combined.  IE KN138.WSD

This aggregate file is necessary for producing a .DYN file for

overplotting, while a separate .WSD file for each CTD is needed

for input to CONCALD.  The aggregate file may require some cut

and pasting.

XI.  FIT FOR CONDUCTIVITY SLOPE AND BIAS TO THE REST OF THE CORRECTED DATA

3.  RUN CONCALD

Input:  ship/cruise/ctd      (no extension, grabs the .WSD file)

Output:  *.HIS      cond bi and sl, with histogram of residuals

*.REJ      rejected data

*.RES      residuals of data in final fit

Notes: The usual procedure is to apply the laboratory conductivity

bias and just fit for the slope.

Can also edit on pressure to make data fit better in the deep

water.

You can make up groups by copying and editting this file

into corresponding station bounds.

$CONCALD

KN138C10

/

/

/     or enter 2000,6500 to fit only in deep water

2/

-.0018     enter manually the lab bias

A plot of delta conductivity (rosette minus ctd salt) VS.

station number can be plotted with .RES output using PLOT5.

See DELTAC.COM in root directory [ctd.kn138p003].

This helps you decide what groupings should be used to load

the data.....perhaps the data should be fit to groups of

stations excluding some high standard deviation stations

within those groups.

XII. CREATING A *.DYN FOR OVERPLOTTING

The command file HYDOUTV.COM is used to create a hydrographic

data file in .DYN format, which can be used as for input to display

programs, such as CTD80D.

INPUT:  *.WSD  contains scaled CTD/Rosette data, aggregate all CTD's

OUTPUT: *.DYN format hydrographic file

$HYDOUTV

KN138.WSD

/

N

KN

138

KN138.DYN        (Output file you wish to create)

/

Note:  Must first delete old .DYN file!!

Check to see quality of watersample data and fits by overplotting

theta S or O  w/ bottle data.  Bottle data should overlay station

CTD traces in the deep water.

To LIST or look at the *.DYN file:

Ouput is P,Depth,Theta,Rosette Salt,Rosette Oxygen,T

$LISTGRID

/

/

/

6/                  (List to screen or use 30/ to output to file)

KN138.DYN           (Input)

0/                  (SKIP N files)

NOTE:

It will be necessary to do additional work for cruises on which more

than one CTD instrument was used to acquire data.  The *.DYN file in

the top level directory should contain ALL stations.

XIII. CREATE AN M.LIS FILE FOR INPUT INTO CRSUMMARY TO CREATE SUBINDEX FILE

a.) SET DEF [CTD.KN138P003]

b.) type DIR *M*.CTD/OUTPUT=Mver.LIS  (this creates a file

containing all the PRSORT output file names)

c.) type EDIT Mver.LIS

make sure the disk name has only 4 letters, edit it out

if it does not

d.) Execute CRSUMARY1

***Note:  MUST ENTER RESPONSES IN CAPTIAL LETTERS!!!!!!

$CRSUMARY1

Mver.LIS       (Input file)

N              (All sta?)

N

N              (List to screen?)

This creates a subindex file (SUBINDEX.CTD) with *M* versions

e.) copy SUBINDEX.CTD MSUBINDEX.CTD

during the cruise you may want to identify what stations

are in each subindex.ctd file when you rename it

ex.  msubindex1t30.ctd

xsubindex1t15.ctd

XIV.  TRANSFORM OXYGENS USING A SUBINDEX FILE

Create a subindex.ctd file here with only the stations you want to be

used for O2 calibrations...excluding those stations with funky data.

(high standard deviations)

Oxygen fitting routine is comprised of three steps:

1. RUN OXEXTVX1

Input:  *.DYN

station limits  - will then cross-match with SUBINDEX.CTD file

and will extract OC and OT from *.CTD files

Output: *.OXY

Name the *.oxy file according to sta and ctd.  EX c10sta4t19.oxy

so that you can tell which stations are included in the fit.

2. RUN OXFITMR       Produces the best fit based on the following criteria

Input:  *.OXY        (NO EXTENSION, give c10sta4t19 id for ex)

Output: *.HIS        oxygen parameters and histogram

*.REJ        rejected data

*.RES        residuals of fitted data

FOR025.DAT   file containing 6 parameters which can be

included in calcl*.com. RENAME to c10sta4t14.dat

immediately following run or else you will

not be able to tell which for025.dat file goes

with which stations.

$OXFITMR

C10STA4T19

/

/

6/     fit for 6 parameters initially

99/

99/

Programs defaults with an edit criteria of 2.8 standard deviations,

oxygen min = 0, oc slope = .0014.  The user can change these by

referring to the documentation.

Ideally fit to 6 parameters:  bias,slope,pcor,tcor,wt,lag.

However, under typical circumstances may set wt and lag and fit

for other 4.  Default for Lag is 8 and default for Wt is .75.

$OXFITMR

C10STA4T19

/

/

4/     fit for 4 parameters

1,1

2,2

3,3

4,4

99/

99/

The .RES file can be plotted (delta oxygen plot) as in the

conductivity procedure with PLOT5.   See DELTAOX.com.

3. OXCALCL             This transforms the OC and OT terms in your

*.CTD files to OX

Default output version is *X*.CTD

Command file is CALCLkn138.com

@CALCL

input best fit O2 cals for each grouping FOR025.DAT from OXFITMR

6/ set station bounds...only now you want your subindex file to be one

with all stations in it and not just the stations you used for the fit

input station bounds

11/ print out FOR020.DAT file

5/ exit

CREATE A NEW SUBINDEX.CTD FILE

$DIR *X*.CTD/output=Xver.LIS

and run CRSUMARY1 as before to create a new SUBINDEX.CTD file.

First you must delete the other subindex.ctd; file,

then copy the one that you create to XSUBINDEX.CTD

OPTIONAL NOTES:

If you decide to reload with new O2 cal groups...then rename X versions

to U versions. Then output a new set of X versions. That way X versions

are always the most recent.

This can be done with <ctdsoft.coms>rename.com

NOTE* CRSUMARY1 cannot be used to make a subindex file from renamed files.

XV.  MAKE OXYGEN/PRESSURE PLOT

a.)  Edit CTD5PO  in the same manner as above

b.)  type @CTD5PO to create the FOR095 file

c.)  type ZPLOT, then G when prompted.

XVI. MAKE THETA/OXYGEN PLOTS

a.)  Edit [CTD.KN138P003] CTD5TO.COM (and change the same parameters as

in CTD5TS.com)

b.)  type  @CTD5TO  to create the FOR095 file to plot

c.)  type  ZPLOT, then G when prompted, to send plotfile to the ZETA

XVII. SECOND ROUND OF EDITING THE DATA

Use <CTD.CTDATA.EXE>CTDED  for removing outrageous spikes.

I set up a command file with all the corrections in it so on the next

load of the data I can just rerun the command file and all will be

fixed.  (see [CTD.KN138P003]kn138CTDED.COM)

some important branches to use are:

2,1/ branch is used to look at a particular station.

most other branches will not work unless you are pointing to a

particular station.

16/  outputs a "E" version, multiple files with the same name cannot be

created.  This branch followed by a slash will output cast 0

8,1/ branch is a listing branch for checking data

19/  branch 19/ interpolates whatever variable over a certain pr range.

VAR 1=TEMP   VAR2=SALT  VAR3=OXY

12/  tells you density inversions (T,S, or P jump)

24/  branch puts out -9 for OXY only

25/  branch controls where -9's go

see the CTD data processing notebook for more details.

XVIII. EDIT STATION HEADERS

INDEXED.EXE

This program accesses SUBINDEX.CTD and takes station file header info

from it.  Three branches are used to identify and correct headers:

1/  branch to edit subindex file

Command file is KN138INDEXED.com

2/  branch to transfer data, puts changes in *.CTD data files

Command file is INDEXTRNSFR.com

3/  branch for diagnostics

Usually run on line

XIX. STATION SUMMARIES FROM A SUBINDEX FILE

a.) Execute CTDLIST84

This program accesses both the SUBINDEX.CTD file and the

actual station files (*X*.CTD) but gets information from

the actual station files

$CTDLIST84

30/

1,2/

r2d2:

P                         This gives a station summary

KN                        of positions, depths, etc

138,03/

12/

13/

5/

REN FOR030.DAT KN138CRSUM.LST

Additional listings:

10/ branch to output station listings

7,3/ branch to output station min max values

NOTE:  CTDLIST84 looks at SUBINDEX.CTD;1 only, it will not look at

a higher version

XX.  KN138ATSEA.DOC

PLEASE update this with personnel, hardware problems, co and ox fits,

ctded and indexed information.