*****************  NODCEXCH.DOC  *************************

* All records are fixed length 35 bytes long *

The first 6 records contain header information:

REC 1= SHIP, CRUISE #, STATION #, CAST #

Ship is a 2 character mneumonic:

AT = Atlantis II   KN = Knorr   OC = Oceanus  TT = Thompson

Cruise is a sequential number.

Cast # is taken from INDEX record.

REC 2= DATE & TIME

Year(2 digit) - month - day  Time is G.M.T., 24 hr clock

REC 3= LAT & LONG

integer degrees, real minutes. Negative for south, west.

REC 4= MAX PRESSURE, WATER DEPTH

Pressure is in decibars, water depth in meters

REC 5= pressure interval,instrument #, scans/sec of time series data

REC 6= # observations and format specification for the data on tape

REC 7= headers for data columns (variable labels)

REC 8 thru End-of-File mark are data observations.

Order of variables within a record = pres,temp,salt,oxygen,quality.

Pressure is in decibars,temperature in degrees Celsius,salt is in

1978 International Practical Salinity Scale units, and oxygen is

reported in milliliters per liter.

Quality word defined: If positive, the quality word contains the

number of observations from the time-series data that went into

the pressure bin. Negative quality words denote data which has

been interpolated. The value of the negative number reflects which

variable or variables have been modified, based on the variable

location in the CTD-VAX data file : -1 for T, -2 for S, -4 for O2,

-3 for T & S, -5 for T & O, -6 for  S & O, -7 for T,S & O.

A positive quality word can be used to obtain a rough estimate of

lowering rate:

lowering rate= sample rate * pressure interval/ quality #

LRATE = lowering rate

time = start time (hr:min) + total P differential/ LRATE