**********************************************************************

*******                                                        *******

*******                r/v THOMAS WASHINGTON                   *******

*******                         P16                            *******

*******                     TUNES LEG 3                        *******

*******                                                        *******

**********************************************************************

Chief Scientist:  Lynne Talley

Co-Chief Scientist:  Greg Johnson

Project Name:  P16C

Dates (leg3):  31 August 1991 - 01 October 1991

Ports-of-call (leg3):  Tahiti - Hawaii

CTD instrument and station numbers:

CTD 10  Stations 1-250

EVEN stations 252-284

Stations 287-326

CTD 9   ODD stations 251-285

Aborted stations:

-------------------------------------------------------------------------------

Leg 3       CTD Software Tech:   Carol MacMurray          WHOI

Leg 3       CTD Hardware Tech:   Gary Bond, Peter Landry  WHOI

Leg 3             Hydrography:   Rosette samples -

ox,sa   George Knapp             WHOI

Bob Stanley              WHOI

nuts   Joe Jennings             OSU

Hernan Garcia            OSU

freon   Dave Wisegarver          PMEL

Tracy McCallister        PMEL

he,tr   Mike Mathewson           WHOI

Kyung-Ryul Kim           SIO

C02   Peter Guenther           SIO

Ed Peltzer               WHOI

Leg 3                     LVS:

C14   Paul Quay                UW

Leonard Lopez            SIO

Leg 3        ADCP                Frank Bahr               U.Hawaii

Leg 3        Bio-optics          Carol Knudson            LDGO

Leg 3        Resident Tech:      Gene Pillard             SIO

Computer Tech:      George Bouchard          SIO

Marine Tech:        Jim Wells                SIO

________________________________________________________________________________

Shorebased processor:

MicroVAX Data subdirectory: R2D2:<CTD.TW016P003>

--------------------------------------------------------------------------------

************************************************************************

*                                                                      *

*                   CRUISE SUMMARY STATION LISTING                     *

*                                                                      *

************************************************************************

Run on  1-OCT-91   at 09:09:24

R/V THOMAS WASHINGTON

STATION  SUMMARY

<CTD.TW016P003>

SH CRU STAT DV CST CTD DA MO YR  ST GMT  END GMT  LATITUDE   LONGITUDE   P MAX

-- --- ---- -- --- --- -- -- --  ------  -------  --------   ---------   ------

TW  16 221   E 001 10   1  9 91    815     938   -17 30.20  -150 29.60   3639.0

TW  16 222   E 001 10   1  9 91   1412    1534   -17  0.10  -150 29.90   3805.0

TW  16 223   E 001 10   1  9 91   2028    2149   -16 30.00  -150 29.70   4117.0

TW  16 224   E 001 10   2  9 91    219     333   -15 59.65  -150 29.80   3421.0

TW  16 225   E 002 10   2  9 91   1330    1454   -15 31.50  -150 39.40   4381.0

TW  16 226   E 001 10   3  9 91    421     547   -14 59.70  -150 50.20   4591.0

TW  16 227   X 001 10   3  9 91   1050    1219   -14 29.60  -151  0.00   4487.0

TW  16 228   X 001 10   3  9 91   1711    1845   -13 59.90  -150 59.90   4371.0

TW  16 229   X 001 10   3  9 91   2329      56   -13 29.80  -150 59.90   4691.0

TW  16 230   X 001 10   4  9 91    548     718   -12 59.68  -151  0.06   4661.0

TW  16 231   X 001 10   4  9 91   1158    1326   -12 29.74  -151  0.00   4857.0

TW  16 232   X 001 10   4  9 91   1831    2004   -12  0.00  -151  0.00   4945.0

TW  16 233   X 001 10   5  9 91    200     329   -11 29.99  -150 59.78   5075.0

TW  16 234   X 001 10   5  9 91    837    1019   -10 59.80  -150 59.90   5127.0

TW  16 235   E 004 10   6  9 91   1115    1246   -10 29.90  -150 59.90   4941.0

TW  16 236   E 001 10   6  9 91   1757    1926    -9 59.90  -150 59.70   4723.0

TW  16 237   X 001 10   7  9 91     54     224    -9 29.70  -150 59.80   4989.0

TW  16 238   X 001 10   7  9 91    724     836    -8 59.90  -150 59.90   3883.0

TW  16 239   X 001 10   7  9 91   1307    1427    -8 30.00  -150 59.90   4215.0

TW  16 240   X 001 10   7  9 91   1919    2058    -7 59.90  -150 59.90   5167.0

TW  16 241   E 001 10   8  9 91    230     410    -7 30.10  -150 59.90   5345.0

TW  16 242   X 001 10   8  9 91    935    1114    -6 59.90  -150 59.90   5241.0

TW  16 243   X 001 10   8  9 91   1616    1751    -6 29.90  -150 59.90   5067.0

TW  16 244   E 003 10   9  9 91    545     720    -6  1.20  -151  0.20   5231.0

TW  16 245   X 001 10   9  9 91   1640    1819    -5 29.90  -150 59.80   5111.0

TW  16 246   X 002 10   9  9 91   2347     115    -5  0.40  -151  0.40   4951.0

TW  16 247   X 001 10  10  9 91    635     810    -4 30.00  -150 59.90   4703.0

TW  16 248   X 001 10  10  9 91   1304    1439    -4  0.00  -151  0.00   4693.0

TW  16 249   E 002 10  10  9 91   2025    2155    -3 29.40  -151  0.50   4779.0

TW  16 250   E 001 10  11  9 91    234     407    -2 59.90  -151  0.00   4843.0

TW  16 251   E 001  9  11  9 91    819     949    -2 50.00  -150 59.70   4796.0

TW  16 252   E 001 10  11  9 91   1259    1430    -2 40.00  -151  0.00   4865.0

TW  16 252   X 001 10  11  9 91   1259    1430    -2 40.00  -151  0.00   4865.0

TW  16 253   E 001  9  11  9 91   1738    1904    -2 29.90  -150 59.90   4877.0

TW  16 254   X 002 10  11  9 91   2235       7    -2 19.90  -151  0.10   5039.0

TW  16 255   E 001  9  12  9 91    325     513    -2  9.80  -150 59.80   4893.0

TW  16 256   E 001 10  12  9 91    817     945    -1 59.80  -150 59.40   4821.0

TW  16 256   X 001 10  12  9 91    817     945    -1 59.80  -150 59.40   4821.0

TW  16 257   E 001  9  12  9 91   1242    1412    -1 49.90  -150 59.90   4899.0

TW  16 258   X 001 10  12  9 91   1650    1820    -1 39.80  -150 59.90   4957.0

TW  16 259   E 004  9  13  9 91   1141    1310    -1 31.00  -150 59.60   4931.0

TW  16 260   X 001 10  13  9 91   1846    2019    -1 19.80  -150 59.60   4907.0

TW  16 261   E 001  9  14  9 91      8     137    -1  9.80  -150 59.90   4869.0

TW  16 262   X 001 10  14  9 91    443     616    -1  0.00  -151  0.10   4781.0

TW  16 263   X 001  9  14  9 91    924    1048     0-49.70  -150 59.80   4363.0

TW  16 264   X 001 10  14  9 91   1315    1439     0-39.80  -150 59.90   4531.0

TW  16 265   X 001  9  14  9 91   1731    1850     0-29.80  -150 59.70   4383.0

TW  16 266   X 002 10  14  9 91   2203    2327     0-19.90  -151  0.00   4431.0

TW  16 267   X 001  9  15  9 91    234     349     0-10.00  -151  0.30   4141.0

TW  16 268   E 001 10  15  9 91    640     803     0  0.30  -151  0.00   4389.0

TW  16 269   X 001  9  15  9 91   1113    1226     0  9.80  -150 59.80   3817.0

TW  16 270   X 001 10  15  9 91   1458    1622     0 20.20  -150 59.90   4421.0

TW  16 271   X 001  9  15  9 91   1934    2039     0 30.20  -151  0.10   3327.0

TW  16 272   X 001 10  15  9 91   2330      46     0 40.20  -151  0.00   4243.0

TW  16 273   X 001  9  16  9 91    342     503     0 50.00  -151  0.00   4273.0

TW  16 274   X 001 10  16  9 91    745     856     1  0.10  -150 59.80   3849.0

TW  16 275   X 001  9  16  9 91   1159    1319     1  9.90  -151  0.00   4325.0

TW  16 276   X 001 10  16  9 91   1558    1700     1 20.10  -150 59.80   3231.0

TW  16 277   X 001  9  16  9 91   1945    2102     1 30.10  -150 59.90   4157.0

TW  16 278   X 001 10  17  9 91     17     134     1 40.10  -151  0.00   4263.0

TW  16 279   E 001  9  17  9 91    448     607     1 50.00  -151  0.00   4183.0

TW  16 279   X 001  9  17  9 91    448     607     1 50.00  -151  0.00   4183.0

TW  16 280   E 001 10  17  9 91    852    1022     2  0.30  -150 59.90   4453.0

TW  16 281   X 001  9  17  9 91   1329    1456     2  9.80  -151  0.00   4543.0

TW  16 282   X 001 10  17  9 91   1753    1922     2 20.20  -151  0.00   4793.0

TW  16 283   X 001  9  17  9 91   2328      54     2 30.00  -151  0.00   4779.0

TW  16 284   X 001 10  18  9 91    403     529     2 40.00  -151  0.00   4675.0

TW  16 285   E 001  9  18  9 91    853    1025     2 50.40  -150 59.90   4805.0

TW  16 286   E 001 10  18  9 91   1325    1500     2 59.90  -150 59.90   5165.0

TW  16 287   X 002 10  18  9 91   2050    2220     3 30.00  -151  0.10   4927.0

TW  16 288   X 002 10  19  9 91    900    1039     4  0.60  -150 59.90   5141.0

TW  16 289   X 001 10  19  9 91   1937    2118     4 30.00  -150 59.80   5233.0

TW  16 290   X 001 10  20  9 91    249     423     5  0.20  -151  0.10   5141.0

TW  16 291   X 001 10  20  9 91    910    1052     5 29.40  -151  5.90   5309.0

TW  16 292   X 001 10  20  9 91   1540    1717     5 59.10  -151 11.10   5203.0

TW  16 293   X 002 10  20  9 91   2243      20     6 28.00  -151 16.10   5347.0

TW  16 294   E 001 10  21  9 91    525     704     6 57.90  -151 21.70   5471.0

TW  16 295   X 001 10  21  9 91   1211    1353     7 27.20  -151 28.00   5401.0

TW  16 296   X 001 10  21  9 91   1845    2023     7 57.30  -151 34.10   5231.0

TW  16 297   X 001 10  22  9 91    139     316     8 26.02  -151 39.18   5215.0

TW  16 298   X 001 10  22  9 91    809     944     8 56.30  -151 45.10   5133.0

TW  16 299   X 001 10  22  9 91   1422    1600     9 25.20  -151 49.80   5281.0

TW  16 300   X 003 10  23  9 91    308     445     9 55.80  -151 55.80   5349.0

TW  16 301   X 001 10  23  9 91   1340    1510    10 24.10  -152  2.00   4865.0

TW  16 302   X 001 10  23  9 91   2000    2136    10 54.30  -152  7.10   5431.0

TW  16 303   X 001 10  24  9 91    515     655    11 23.00  -152 12.80   5445.0

TW  16 304   X 001 10  24  9 91   1200    1341    11 52.40  -152 19.10   5269.0

TW  16 305   X 001 10  24  9 91   1834    2020    12 22.10  -152 23.70   5491.0

TW  16 306   X 001 10  24  9 91    225     411    12 52.10  -152 30.10   5655.0

TW  16 307   X 001 10  25  9 91    915    1103    13 21.20  -152 35.80   5707.0

TW  16 308   X 003 10  25  9 91   2340     135    13 50.80  -152 40.70   6157.0

TW  16 309   X 001 10  26  9 91   1230    1420    14 20.20  -152 47.00   6091.0

TW  16 310   X 001 10  26  9 91   1932    2121    14 50.10  -152 53.00   5917.0

TW  16 311   X 001 10  27  9 91    305     449    15 19.20  -152 59.10   5777.0

TW  16 312   X 001 10  27  9 91    949    1132    15 49.20  -153  3.90   5523.0

TW  16 313   X 001 10  27  9 91   1625    1759    16 18.20  -153  9.90   5051.0

TW  16 314   X 002 10  27  9 91   2335     112    16 48.20  -153 16.20   5273.0

TW  16 315   X 001 10  28  9 91    608     744    17 17.10  -153 21.90   5159.0

TW  16 316   X 001 10  28  9 91   1200    1334    17 39.10  -153 25.80   5007.0

TW  16 317   X 003 10  28  9 91   2346     115    18  0.00  -153 30.00   5023.0

TW  16 318   X 001 10  29  9 91   1032    1206    18 12.30  -153 59.10   5241.0

TW  16 319   X 001 10  29  9 91   1656    1829    18 24.20  -154 28.10   5255.0

TW  16 320   X 001 10  29  9 91   2318      58    18 37.10  -154 57.10   5303.0

TW  16 321   X 001 10  30  9 91     51     712    18 49.30  -155 25.10   4373.0

TW  16 322   E 001 10  30  9 91   1012    1115    18 51.80  -155 33.70   3223.0

TW  16 323   X 001 10  30  9 91   1329    1423    18 51.10  -155 35.20   2557.0

TW  16 324   X 001 10  30  9 91   1612    1650    18 52.20  -155 36.40   1839.0

TW  16 325   X 001 10  30  9 91   1829    1853    18 52.70  -155 38.30    933.0

TW  16 326   X 001 10  30  9 91   1956    2008    18 52.80  -155 39.40    281.0

CRUISE     MIN/MAX DATE    MIN LAT/LON MX LAT/LON   INST   NV MIN MAX TOT P WH

--------- ----------------- ----------- ---------- -------- -- --- --- --- - --

TW016P003 91- 9- 1 91- 9-30  -18. -156.  19. -150. 10 9 0 0  5 221 326 109 Y

************************************************************************

*                                                                      *

*                     A C Q U I S I T I O N                            *

*                                                                      *

************************************************************************

-----------------------

PRE-CRUISE CALIBRATIONS

-----------------------

RUSKCAL (PC based programs)

CTD 10    10AG91TC.CAL   main instrument  TE,CO

10AG91PR.CAL                    PR

CTD  9    9AG91TC.CAL   backup instrument TE,CO

9AG91PR.CAL                     PR

-----------------------

AT SEA DATA ACQUISITION

-----------------------

MICROVAX II

with WHOI AQUI89 acquistion package (Version 1.0)

Logging data to:   Audio

9T

Microvax disk file (###A###.RAW) in CTD78 format

(*.WRW,*.WSC,*.HED,*.ERR) in ASCII format

CTD 10

AT SEA COMMON USED FOR DATA ACQUISITION   CTD#10

ID  attribute 1     attribute 2     slope        bias           sens lag

PR  -0.239333E-08   0.000000E+00   0.100318E+00  -0.489652E+00   0.000000E+00

TE   0.489679E-11   0.000000E+00   0.499687E-03  -0.341983E-03   0.250000E+00

CO  -0.650000E-05   0.150000E-07   0.100456E-02  -0.884232E-02   0.000000E+00

SW   0.280000E+01   0.300000E+04   0.100000E+01   0.000000E+00   0.000000E+00

OC  -0.360000E-01   0.115000E-03   0.148000E-02   0.000000E+00   0.000000E+00

OT   0.750000E+00   0.000000E+00   0.128000E+00   0.000000E+00   0.000000E+00

TP   0.000000E+00   0.000000E+00  -0.896300E-02   0.358710E+02   0.000000E+00

RT  -0.351986E-10   0.309482E-15   0.501011E-03  -0.200438E+01   0.000000E+00

TR   0.000000E+00   0.000000E+00   0.100000E+01   0.000000E+00   0.000000E+00

QU   0.840000E-06   0.668000E-01   0.100000E+01   0.217000E+02   0.000000E+00

CTD 9

AT SEA COMMON USED FOR DATA ACQUISITION   CTD#9

ID  attribute 1     attribute 2     slope        bias           sens lag

PR   0.724254E-09   0.000000E+00   0.100506E+00  -0.823025E+00   0.000000E+00

TE   0.330672E-11   0.000000E+00   0.500056E-03  -0.957694E-02   0.250000E+00

CO  -0.650000E-05   0.150000E-07   0.977069E-03  -0.944740E-02   0.000000E+00

SW   0.280000E+01   0.300000E+04   0.100000E+01   0.000000E+00   0.000000E+00

OC  -0.360000E-01   0.115000E-03   0.148000E-02   0.000000E+00   0.000000E+00

OT   0.750000E+00   0.000000E+00   0.128000E+00   0.000000E+00   0.000000E+00

TP   0.000000E+00   0.000000E+00  -0.909770E-02   0.370952E+02   0.000000E+00

RT  -0.620002E-10   0.554677E-15   0.501522E-03  -0.198726E+01   0.000000E+00

QU   0.000000E+00   0.000000E+00   0.100000E+01   0.000000E+00   0.000000E+00

COMMENTS regarding data acquisition:

LEG 3:  Initial stations very noisy - pressure, oxygen and salinity

spikes.  Ctd was cabled to only one center conductor, with

the other two conductors being wired to the two rosette

pylons for the 36 bottle rosette package.  Redundant temp

was not coming up the wire.  Oxygen severely erratic.

Prior to station 225, the OTM sensor (redundant temp)

was temporarily disconnected to try to increase current

to the ctd and oxygen pump.  CTD failure at 1600m on the

upcast, all AC went dead.  Cast aborted at 1500m.  IC chip

was replaced, station 225 cast 5 (2000m) still noisy.

Increased voltage to the fish by switching to a different

power supply.  Station 226 somewhat cleaner but somewhow got

out of phase towards end of cast.  Plotter died twice.

Removed SBE oxygen pump to reduce total power consumption

of CTD package.  Use of pump exceeded compliance voltage

capability (150 VDC).  Phase also adjusted station 227,

data very clean, oxygen and pressure very smooth.  No

hysteresis (which was evident on prior stations).

Seabeam had problems (gyro?) between sta 227 and 228.

O2 dropouts stat 231-232 upcast.  CTD package hit side

of ship upon retrieval of sta 233.  Three bottles broken.

More O2 dropouts on upcast sta 234.  Upon retrieval of

sta 234, bottles 17-24 were not fired.  Failure was due

to short to seaground in center conductor of E/M wire

(pylon # 1-24 position).  Short 113 ohms from slipring

and 180 ohms from fish termination.  Rewired CTD pkg

with one conductor dedicated to pylon # 1 plus CTD

and second conductor to pylon # 2.

Sta 235 and 236 O2 dropouts first 350 m on downcast.

Sta 235 O2 dropouts and redt temp problems on upcast.

No pinger stations 235,236,238.

Bottom contact stations 242 and 245.  No CO shift

between 242 and 243 although .004 shift between 245 and

246.

Lots of scatter among bottle salts.  Changed autosals

prior to running station 246.  Salts much tighter.

Aqui TE lag changed to .35 prior to 246.

First ADCP station 251 with CTD #9.  Battery weak from

sitting in sun on deck.  Not enough current to fish.

As a result, conductivity jumped near end of downcast

when more juice was applied.  Station was processed

by prsorting the upcast.

CTD 9 showed pressure hysteresis for the first three

casts (sta 251,253,255).  Sensor settled down by

station 257,259. Hysteresis came back station 261 and

seemed to remain a characteristic for the duration of

casts with ctd 9.

Retermination prior to sta 259.  Just before deploying

259, fatal failure with ctd 9.  OTM board fried.

As a result, Redt temp serious departure from plat

temp for this station.  For rest of ctd 9 stations,

used ctd 10's OTM.  Switch out every station.

Station 268 at the equator (actually just a little

north of it).  Sta 268 showed conductivity jump

in deep water.

Stations 260-290 showed lots small oxygen spiking

pressure range 1500-3000.  Prior to station 286,

tied off endcaps, lowered compliance voltage and

debubbled receptacle to try and alleviate spikes.

Spiking variable until stations 290-291 when

spikes mysteriously disappeared.

Station 288, hit bottom again.  PDR showed contact.

No apparent shift between 288 and 289.  But station

292, co started drifting fresh.  Co cell died station

294.  Replaced sensor prior to station 295.

OTM flaky on upcast station 302.  New termination

prior to station 303, sea cable from ctd to rosette

got hung up on cart.

George switched to salt patrol sta 304.  Cleaned

the salinometer which was caked with salt.

Aqui died on upcast stations 305,307.  No response

when tagging, pausing..etc.

Recalibrated ctd 10's new conductivity cell after

station 308.  Station 309 has new cals in aqui

template file.

Pressure hysteresis ctd # 10 by station 306.

BOTTOM CONTACT:  242, 245  PDR did not show bottom contact,

but salt spike at bottom on aqui plot for both stations.

No shift in conductivity between stat 242-243, but one

did occur between 245-246.

288.  PDR showed bottom contact.  No shift.

But co cell started drifting station 292.  Co cell died 294.

PROBLEM STATIONS: Stations 221-226 ctd oxygen traces will not fit bottles

Prsorted uptrace station 251, ctd 9.  Ctd  oxygen will

not fit bottles.

*************************************************************************

*                                                                       *

*               S H I P B O A R D   P R O C E S S I N G                 *

*                                                                       *

*************************************************************************

DESCRIPTION OF COMPUTER SYSTEM USED:

CTLOG   **currently not being used due to hardware malfunction

CTDED78

run on MicroVAX Acquisition CTD78 format raw data DISK files.

output to MicroVAX disk files [CTD.TW016P003.CTDED78]####D###.EDT

error identification downtrace

Water sample Programs:  BTLFMTVX, WOCETMPV   (to produce WOCE template)

BTLMRGVX, SEAMERGE   (to merge sa,ox,nuts data)

CONVERT,  HYDOUTV    (to create .dyn file)

Water sample filenames:   Salinity and Oxygen Rosette Samples

for overplotting ctd and bottle data

TW16.WSD - all stations

TW16.DYN - all stations for overplotting

Merged CTD, SA, OX, Nutrient water sample filename:

TW16.SEA - Woce template, all stations appended together

[COMMENTS]

*************************************************************************

*                                                                       *

*                   C A L I B R A T I O N S                             *

*                                                                       *

*************************************************************************

CTD SENSOR CALIBRATIONS used for POST CRUISE PROCESSING

********************************************************************

******                                                       *******

******    CALIBRATION of PRESSURE and TEMPERATURE SENSORS    *******

******                                                       *******

********************************************************************

.CAL format calibration data files on PC in C:\RUSKCAL

works with RUSKA and NEW Titanium Pressure sensor

CTD #10   10AG91TC.CAL  PRE-cruise TE, CO

.FIT  for listing of scaling factors and residuals

10AG91PR.CAL  PRE-cruise PR

.FIT

10OCT91T.CAL  POST-cruise TE

.FIT

10OCT91C.CAL  POST-cruise CO

.FIT

10NV91PR.CAL  POST-cruise PR

.FIT

10NV91PI.CAL  POST-cruise PR at ice (0 deg. C) temp

.FIT

10TWTE.CAL  POST-cruise COMBINED TE JUN/AUG/OCT 1991

.FIT

CTD #9     9AG91TC.CAL  PRE-cruise TE, CO

.FIT  for listing of scaling factors and residuals

9AG91PR.CAL  PRE-cruise PR

.FIT

9OCT91T.CAL  POST-cruise TE

.FIT

9OCT91C.CAL  POST-cruise CO

.FIT

9NV91PR.CAL  POST-cruise PR

.FIT

9NV91PI.CAL  POST-cruise PR at ice (0 deg. C) temp

.FIT

PROGRAM:    RUSKCAL.EXE

--------

PRESSURE:  QUADRATIC DOWN/UP  (0-9000-0 db range)

--------

BI= -(pre-lowering deck unit PR reading)

BIAS        SLOPE         ATTR 1

PRE-CRUISE  CTD10  -.489652E0   .100318E0    -.239333E-8

CTD9   -.823025E0   .100506E0     .724254E-9

POST-CRUISE CTD10  -.185372E1   .100331E0    -.265076E-8

CTD9   -.173307E1   .100489E0     .779511E-9

***The same calibrations were used for scaling Rosette data

and Pressure averaging the CTD downtrace.

POST-cruise calibrations were used for final data load.

PROGRAM:    RUSKCAL.EXE

-----------

TEMPERATURE:  quadratic fit

-----------

BIAS         SLOPE         ATTR 1

PRE-CRUISE  CTD10  -.180007E-3   .499735E-3    .396770E-11

CTD9   -.957694E-2   .500056E-3    .330672E-11

POST-CRUISE CTD10  -.316865E-3   .499891E-3    .191662E-11

CTD9   -.853737E-2   .500074E-3    .247454E-11

POST-COMB0  CTD10  -.232143E-3   .499794E-3    .341585E-11

(Jun,Aug,Oct 1991)

CTD 9,10= temperature time lag of .25 seconds

Changed TE lag to .35 prior to aquiring station 246.

POST-COMBO cals were used for CTD #10 final data load.

POST-CRUISE cals were used for CTD #9 final data load.

PROGRAM:    POLY.EXE

-----------

REDUNDANT TEMPERATURE:  cubic fit

-----------

BIAS         SLOPE         ATTR 1       ATTR 2

PRE-CRUISE  CTD10  -.200438E+1   .501011E-3   -.351986E-10   .309482E-15

CTD9   -.198726E+1   .501522E-3   -.620002E-10   .554677E-15

No Post-cruise RT cals.

PRE-cruise cals were used for final data load.

PROGRAM:    RUSKCAL.EXE

-------------

CONDUCTIVITY:

-------------

BIAS         SLOPE

PRE-CRUISE  CTD10  -.113775E-1   .100462E-2  cell A

CTD9   -.944740E-2   .977069E-3

POST-CRUISE CTD10   .122951E-2   .100737E-2  cell B

CTD9   -.151393E-1   .976219E-3

*********************************************************

*******                                           *******

*******  CALIBRATION OF THE CONDUCTIVITY SENSOR   *******

*******                                           *******

*********************************************************

<CTD.CTDEV.CAL>CONVERT.EXE   (AUG 91)

CONCALD.EXE   (AUG 91)

-------------------

INITIAL DATA LOAD:

-------------------

DATE:  Used CONCALD to compute conductivity scaling factors based

on best fit of CTD data to corresponding Rosette data

CTD 10:  INITIAL load   STATIONS 222 - 225

LAB CAL BIAS

TE lag= .25

BIAS         SLOPE

-.113775E-1   .100411E-2

**** SECOND load   STATIONS 227 - 234

****               applied from sta 227 through sta 294

Bottle - CTD discrepancy sta 240-270

LAB CAL BIAS

TE lag= .25

BIAS         SLOPE

-.113775E-1   .100425E-2

(lab bias)

**** THIRD load   STATIONS 295 - 297, 300 - 305

****               applied from sta 295 through sta 305

****              NEW CO CELL CTD 10

TE lag= .25

BIAS         SLOPE

-.420778E-2   .100729E-2

**** FOURTH load  STATIONS 306 - 314

****               applied from sta 306 through sta 314

****              STATION DEPENDENT DRIFT FOR SLOPE

hysteresis

TE lag= .25

BIAS         SLOPE

306     -.420778E-2   .10072012E-2

307     -.420778E-2   .10071952E-2

308     -.420778E-2   .10071892E-2

309     -.420778E-2   .10071833E-2

310     -.420778E-2   .10071773E-2

311     -.420778E-2   .10071713E-2

312     -.420778E-2   .10071654E-2

313     -.420778E-2   .10071594E-2

314     -.420778E-2   .10071534E-2

**** FIFTH load   STATIONS 315 - 326

****               applied from sta 315 through sta 326

****              hysteresis

TE lag= .25

BIAS         SLOPE

-.420778E-2   .10071533E-2

TEST load    STATIONS 246 - 254  CTD10 only

NOT APPLIED AT SEA

TE lag= .25

BIAS         SLOPE

-.113775E-1   .10043221E-2   p>2500

(lab bias)

Slope adj station 241.

Comparison of deep water theta/salt plots of CTD (M version) traces

and rosette data (TW16.DYN) yielded the following:

CTD 10 stations 227-234 differ from end of leg 2 by about .0015.

This may be due to the difference between the two standards.  CTD

shifted (from bottom contact) at station 246.  At this point, ctd

lays fresh of the bottle data.  Overall, ctd does not drift,

but by station 270, bottle data has drifted over to meet the ctd.

CTd still loaded with cals from 227-234.  Difficult to explain.

CTd and rosette samples overlay 10 North MW893 data perfectly.

New conductivity cell for number 10 also matches perfectly after

fitting with CONCALD.

Suspect pressure hysteresis CTD 10 beginning at station 306??

TRouble fitting water samples.

Delta conductivity vs. station plot looks like a sine wave for #10.

SUGGESTIONS FOR other DATA LOADS:  Need to figure out ctd and bottle

discrepancy sta 240-270 first!!

-----------------------------------

CTD 9: **** FIRST load   STATIONS 275 - 285

applied from sta 275 through 285

STATION DEPENDENT DRIFT FOR SLOPE

BETA = 0

LAB CAL BIAS

TE lag= .25

BIAS         SLOPE

STA 275      -.944740E-2   .97520131E-3

STA 277      -.944740E-2   .97516257E-3

STA 279      -.944740E-2   .97512383E-3

STA 281      -.944740E-2   .97508509E-3

STA 283      -.944740E-2   .97504635E-3

STA 285      -.944740E-2   .97500761E-3

**** SECOND load   STATIONS 255 - 261

applied from sta 255 through 261

USE slope from 261 for sta 261 - 273

STATION DEPENDENT DRIFT FOR SLOPE

BETA = 0

LAB CAL BIAS

BIAS         SLOPE

STA 255      -.944740E-2   .97492714E-3

STA 257      -.944740E-2   .97491228E-3

STA 259      -.944740E-2   .97489742E-3

STA 261      -.944740E-2   .97488256E-3

STA 265-273      -.944740E-2   .97488256E-3

(ODD STA ONLY)

STA 253      -.944740E-2   .97507495E-3

(SLOPE ADJUST STA 255 +.006)

STA 251      -.944740E-2   .97532698E-3

(SLOPE ADJUST STA 255 +.016)

Comparison of deep water theta/salt plots of CTD (M version) traces

and rosette data (TW16.DYN) yielded the following:

First load for stations 275-285 match ctd 10 and MW893 fairly

well.  Needs minor adjustment.  Second load sta 255-261 does not

match ctd 10 as it is fit to its watersample data, whereas 10 is not.

This is the bottle-ctd discrepancy we need to figure out.

CTD 9 is salty of ctd 10 over this station range 251-273.

Pressure hysteresis makes CTD 9 miss bottle data in all but the

deepest water.

SUGGESTIONS FOR other DATA LOADS:  Need to figure out ctd and bottle

discrepancy CTD # 10 sta 240-270 first!!

<CTD.CTDATA.PRSORT91>PRSORT91.EXE   (AUG 91)

Program PRSORT91 was used to convert time series CTD78 format data

to VAX disk files of pressure averaged observations at 2 db intervals.

The '.CTD' data files include variables OC and OT from which oxygen

calibrations will be refined later.

PRSORT91 was created to encorporate the new titanium pressure

sensor and new pressure temperature algorithm.  See the subroutine

PRESSC.FOR.  This sensor should not exhibit any hysteresis.  Also,

the SAL78 was corrected, replacing C1535 with 42.914  (formerly 42.909).

This will increase salinity by about .005.

ALSO, the OC slope was hardwired to .0014 to concur with the

OXEXTVX1 routine.  The *.CTD file encorporates this new number, however

the *.DLD listing reflects the old slope of .00148.

The following FINAL command files were used to run PRSORT loading

data from CTDED CTD78 FORMAT tapes:

************************************************

******                                    ******

******    CALIBRATION of OXYGEN SENSOR    ******

******                                    ******

************************************************

Basic Method for generating oxygen fits:

----------------------------------------

<CTD.CTDEV.WS>HYDOUTV    (AUG 91)

used to transform TW16.WSD (from CONVERT) to TW16.DYN (NODC format)

LISTGRID produces ASCII listing (STALEG#.LST) of Rosette

data from '.DYN' file

<CTD.CTDEV.CAL>OXEXTVX1  (AUG 91)

used to generate *.OXY file from selected stations

of water sample data from the appropriate SUBINDEX.CTD

file and TW16.DYN file by extracting data scans from

CTD downtrace profiles which correspond to Rosette

sampling pressures.

<CTD.CTDEV.CAL>OXFITMR   (AUG 91)

run on each *.OXY file to arrive at scaling

parameters for the oxygen sensor; fit the least number

of parameters necessary to reduce s.d. to +/- 2.8

<CTD.CTDATA.EXE>OXCALCL    (26-jun-86)

used to apply oxygen scaling factors derived by OXFITMR

to PRSORTed  .CTD files.  Outputs 'X' version.

$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

$$$$$$                                                               $$$$$$

$$$$$$          PROGRESSION OF OXYGEN SENSOR CALIBRATIONS            $$$$$$

$$$$$$                                                               $$$$$$

$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

--------------------------

I....  DATE :   'X' versions created from *M* files

--------------------------

THESE 'X' versions were created with the following fits generated

to the station bounds listed :

OXEXTVX1 used to generate a series of .OXY files for oxygen

calibration groups

OXFITMR run on each OXY file separately to arrive at scaling

parameters for oxygen sensor as follows:

fit 4, 5 OR 6 parameters, starting with the first until sd +/- 2.8 reached

set OXMIN to .001

OUTPUT is an FOR025.DAT file which can then be input to CALCL

TW221.DAT    CTD #10  Oxygen sensor A   applied to sta 221

TW222.DAT                               applied to sta 222

TW223.DAT                               applied to sta 223

TW224.DAT                               applied to sta 224

TW225.DAT                               applied to sta 225

TW226.DAT                               applied to sta 226

TW227T234.DAT                               applied to sta 227-233

TW234F.DAT                               applied to sta 234

TW235T237.DAT                               applied to sta 235-237

TW238T240.DAT                               applied to sta 238-240

TW241.DAT                               applied to sta 241

TW241T242.DAT                               applied to sta 242

TW243T245.DAT                               applied to sta 243-281

TW282T288.DAT                               applied to sta 282-288

TW289T293.DAT                               applied to sta 289-293

TW294T295.DAT                               applied to sta 294-295

TW294T299.DAT                               applied to sta 296-299

TW300T304.DAT                               applied to sta 300-304

TW305T310.DAT                               applied to sta 305-310

TW311T316.DAT                               applied to sta 311-321

TW322T326.DAT                               applied to sta 322-326

TW251F.DAT    CTD #9  Oxygen sensor A    applied to sta 251

TW253.DAT                               applied to sta 253

TW257.DAT                               applied to sta 257

TW259.DAT                               applied to sta 259

TW261T271.DAT                               applied to sta 255,261-271

TW273T285.DAT                               applied to sta 273-285

OXYGEN PARAMETERS GENERATED FROM OXFITMR FOR ABOVE STATION GROUPINGS:

FIT

___

CTD 10:

221       =   0.000  0.1000E+01  0.1500E-03 -0.3600E-01  0.7500E+00  0.8000E+01

222       =   0.000  0.1000E+01  0.1500E-03 -0.3600E-01  0.7500E+00  0.8000E+01

223       =   0.085  0.6300E+00  0.1833E-03 -0.1491E-01  0.7500E+00  0.8000E+01

224       =   0.198  0.4325E+00  0.1312E-03 -0.1023E-01  0.7500E+00  0.8000E+01

225       =   0.085  0.6300E+00  0.1833E-03 -0.1491E-01  0.7500E+00  0.8000E+01

226       =   0.080  0.6382E+00  0.1706E-03 -0.1459E-01  0.7500E+00  0.8000E+01

227-234   =   0.039  0.8518E+00  0.1483E-03 -0.2485E-01  0.8930E+00  0.2657E+01

234F      =  -0.079  0.1232E+01  0.1572E-03 -0.3670E-01  0.7500E+00  0.8000E+01

235-237   =   0.032  0.8889E+00  0.1524E-03 -0.2666E-01  0.7500E+00  0.5370E+01

238-240   =   0.038  0.8831E+00  0.1516E-03 -0.2494E-01  0.9653E+00 -0.9865E+00

241       =   0.028  0.9375E+00  0.1472E-03 -0.2684E-01  0.8602E+00  0.8000E+01

241-242   =   0.064  0.8087E+00  0.1468E-03 -0.2326E-01  0.9935E+00  0.4065E+01

243-245   =  -0.008  0.1027E+01  0.1524E-03 -0.2914E-01  0.8675E+00  0.1379E+01

282-288   =  -0.001  0.9945E+00  0.1516E-03 -0.2826E-01  0.9059E+00  0.5383E+01

289-293   =   0.001  0.1002E+01  0.1484E-03 -0.2844E-01  0.8269E+00  0.7603E+01

294-295   =   0.002  0.9787E+00  0.1499E-03 -0.2683E-01  0.8371E+00  0.6250E+01

294-299   =   0.005  0.9738E+00  0.1504E-03 -0.2689E-01  0.8338E+00  0.6014E+01

300-304   =   0.000  0.1023E+01  0.1463E-03 -0.2919E-01  0.7500E+00  0.7228E+01

305-310   =   0.002  0.9925E+00  0.1473E-03 -0.2891E-01  0.7500E+00  0.8000E+01

311-316   =   0.000  0.1022E+01  0.1439E-03 -0.2980E-01  0.7500E+00  0.8000E+01

322-326   =   0.000  0.1032E+01  0.1407E-03 -0.3134E-01  0.8407E+00  0.3844E+01

CTD 9:

251       =  -0.004  0.1055E+01  0.1105E-03 -0.3554E-01  0.5647E+00  0.8000E+01

253       =  -0.028  0.9616E+00  0.1677E-03 -0.3043E-01  0.9670E+00  0.8000E+01

257       =  -0.023  0.9127E+00  0.1708E-03 -0.3175E-01  0.7500E+00  0.8000E+01

259       =  -0.024  0.8979E+00  0.1658E-03 -0.3049E-01  0.7500E+00  0.8000E+01

261-271   =  -0.015  0.8771E+00  0.1717E-03 -0.2957E-01  0.7500E+00  0.8000E+01

273-285   =  -0.004  0.8285E+00  0.1658E-03 -0.2805E-01  0.8403E+00  0.5060E+01

Comparison of full profile pressure/O2 plots of CTD traces

and rosette data (  .DYN) yielded the following:

Stations 221-226 could not be fit to their watersample data

even by fitting each station individually.  Stations 227-  show

supreme performance.

Station 251 (ctd #9) was prsorted with the upcast.  CTd trace

would not fit bottle data.

Erratic oxygen spiking stations 264-291 ctd 10 stations only.

Seemed pressure related- 1500-300db range only.

**************************************************

*********                                *********

*********        QUALITY CONTROL         *********

*********                                *********

**************************************************

<CTD.CTDATA.EXE>CTDED.EXE  (27-oct-86)           P16CTDED.COM

To correct erroneous data scans (i.e. interpolate over pressure

bounds to get rid of salinity and oxygen spikes in the data)

Outputs 'E' version.

STATION       VARIABLE       ACTION        P1, P2      COMMENTS

------------     --------    -----------     ---------    --------------------

CTD 10

221            OX           19/          933,937     INTERPOLATE

957,961

973,977

1007,1015

1021,1031

1765,1771

1779,1785

1805,1809

1815,1819

1829,1837

1857,1865

1873,1877

1887,1909

1955,1979

222            OX           19/          921,925     INTERPOLATE

933,937

943,949

1559,1581

1623,1635

1637,1643

1663,1673

1683,1691

1717,1723

223            OX           19/          715,767     INTERPOLATE

1033,1037

1933,1999

224            OX           19/          635,639     INTERPOLATE

901,907

917,937

1725,1729

1843,1851

1857,1861

1865,1871

1935,1955

225            OX           19/          875,905     INTERPOLATE

915,919

2159,2167

2203,2255

226            OX           19/          929,941     INTERPOLATE

943,947

2087,2151

3811,3825

4349,4365

235            OX           19/          305,321     INTERPOLATE

236            OX           20/            1,4.884   FUDGE SURFACE OX

19/            1,31      INTERPOLATE

73,231

325,355

241            SA           19/          289,293     INTERPOLATE

244                 NO EDITS, OUTPUT CAST 3

249            SA           19/         1911,1917     INTERPOLATE

250            SA           19/          567,571      INTERPOLATE

20/         4843,34.698   FUDGE BOTTOM SA

19/         4785,4843

252            SA           20/         1,35.452      FUDGE SURF SA

3,35.452

256            SA           20/         1,35.604      FUDGE SURF SA

264            OX           19/         1599,1605     INTERPOLATE

1635,1643

1647,1651

1659,1667

1669,1677

1681,1687

1693,1697

1703,1711

1719,1727

1735,1739

1965,1871

2025,2029

268            SA           19/          311,327     INTERPOLATE

22/   .0028,3063,4389    APPLY SA BI SHIFT

19/         3061,3065    INTERPOLATE

2313,2327

2401,2407

2433,2443

2557,2563

2605,2611

2727,2735

270            OX           19/         1641,1647    INTERPOLATE

1693,1699

274            OX           19/         1339,1345    INTERPOLATE

2665,2675

2693,2701

2725,2731

2745,2753

280            SA           19/          593,599     INTERPOLATE

286            SA           19/          643,647     INTERPOLATE

653,657

294            SA           20/         2101,34.644  FUDGE SA VALUE

2351,34.655

2503,34.660

2685,34.664

2909,34.668

3151,34.673

3403,34.678

3665,34.682

4001,34.688

4301,34.691

4635,34.693

5347,34.693

5471,34.693

19/         1993,2101    INTERPOLATE

2101,2351

2351,2503

2503,2685

2685,2909

2909,3151

3151,3403

3403,3665

3665,4001

4001,4301

4301,4635

4635,5347

5347,5471

322            SA           19/          555,561     INTERPOLATE

571,605

931,945

OX           19/          571,605

769,773

875,883

-----------------------

CTD 9:   STATION 251 SPECIAL CASE

First run CONVERT251.COM on station 251

TO CONVERT UPCAST TO DOWNCAST!!!

M         version

251,1/

16/

99/    OUTPUT TEMP TO CAST 99

Then run CALCL to fit oxygens

Then run CTDED to edit spikes

STATION       VARIABLE       ACTION        P1, P2      COMMENTS

------------     --------    -----------     ---------    --------------------

CTD 9

251            TE           19/         3054,3732     INTERPOLATE

SA                        758,792

1010,1028

2058,2092

3048,3734

OX                        768,786

2062,2096

3054,3058

3074,3080

3714,3740

253            OX           19/         2855,2859     INTERPOLATE

2865,2883

4627,4631

4647,4667

255            TE           19/            5,11       INTERPOLATE

57,91

97,139

141,207

255,267

SA           20/            3,35.440   FUDGE SURF SA

181,35.000   FUDGE SA

183,34.997

19/            3,181

183,223

OX           19/           55,73

97,133

137,163

3111,3131

4307,4313

257            OX           19/         2249,2317     INTERPOLATE

2743,2767

3837,3857

20/         4923,4.300    FUDGE BOTTOM OX

19/         4805,4923     INTERPOLATE

259            OX           19/         3515,3531     INTERPOLATE

3759,3763

261            OX           19/         4137,4185     INTERPOLATE

279            SA           20/         1,34.718      FUDGE SURF SA

3,34.718

285            SA           19/          125,163      INTERPOLATE

2273,2277

_______________________________________________________________________________

<CTD.CTDATA.EXE>CRSUMARY1.EXE   (12-jan-83)

To create SUBINDEX.CTD file of selected versions of CTD data files

FINAL ARCHIVED VERSIONS OF SUBINDEX.CTD

DATES   1991              SUBINDEX.CTD   final XE version data files

1991              MSUBINDEX.CTD  prsort 'M' versions

1991              FSUBINDEX.CTD  final XE versions

(copy of subindex.ctd)

...............................................................................

<CTD.CTDATA.EXE>INDEXED.EXE   (21-aug-84)

To correct erroneous header information in SUBINDEX.CTD files

and transfer that corrected info to the station data files.

branch 3:  auto-check mode to scan for errors, run interactively

branch 1:  to correct header info in SUBINDEX.CTD file

branch 2:  transfer header info

from SUBINDEX.CTD file to data files

Header information corrections:

STATION       CATEGORY         CORRECTION         COMMENTS

------------     --------    -------------------     -------------------------

245/                                  STATION 245.....................

SP

-5 29.90 -150 59.80/

EP

-5 29.80 -151 00.40/

251/                                  STATION 251  ......................

SP

-2 50.00 -150 59.70/

EP

-2 50.50 -151 00.30/

261/                                  STATION 261  ......................

ST

0008

263/                                  STATION 263  ......................

SP

-0 -49.70 -150 59.80/

EP

-0 -49.80 -150 59.50/

264/                                  STATION 264  ......................

SP

-0 -39.80 -150 59.90/

EP

-0 -40.30 -150 59.30/

265/                                  STATION 265  ......................

SP

-0 -29.80 -150 59.70/

EP

-0 -29.80 -150 59.40/

266/                                  STATION 266  ......................

SP

-0 -19.90 -151 00.00/

EP

-0 -20.50 -151 00.00/

267/                                  STATION 267  ......................

SP

-0 -10.00 -151 00.30/

EP

-0 -10.40 -151 00.10/

All stations were corrected with ADCP (Acoustic Doppler Current Profiler) data

[ctduser.jmt1.nerctape]readpostion.exe

reads in NAV file from 9T

outputs POSITION#.dat file

where.exe

reads in postion#.dat file

outputs whereout#.dat file

(which can be included in MW893INDEXED.com

DATE         COMMAND FILE      DOCUMENTATION       COMMENTS

-----------      ------------      -------------       -------------------------

P16INDEXED.COM         TWINDEXED.LST   all stations, first pass,

branch 1 only

INDEXTRNSFER.COM    INDEXTRNSFER.LST     all stations, branch 2

INDEXEDB3.DAT        all stations, branch 3

**************************************************

*********                                *********

*********           B A C K U P          *********

*********                                *********

**************************************************

Command file:  @CTDA:<CTDSOFT.COMS>BACKUP    BLOCK=10000  BPI=1600

Run interactively.

DATE         TAPE LABEL         WHERE STORED      SAVE_SET NAME(S)

------       ------------       --------------     ----------------

DA-MON-YEAR       SHCRU            CLARK/MCLEAN      1. SHCRUPROJ.FIN

2. SHCRUPROJ.BCK

**************************************************

*********                                *********

*********             N O D C            *********

*********                                *********

**************************************************

<CTD.CTDATA.EXE>NODCEXCH.EXE  (29-may-85)

WHERE       QUALITY CHECK

DATE         ALL STAS?    DENSITY      STORED      G HEIMERDINGER

----         ---------    -------      ------      --------------

********************************************************

*********                                      *********

*********        P U B L I C A T I O N S       *********

*********                                      *********

********************************************************