FILE TYPE 017 - PRESSURE GAUGE - 7/1/76 VERSION

THIS FORMAT IS DESIGNED TO RECORD TIME SERIES PRESSURE VALUES AT

DEPTH FOR A BOTTOM-MOUNTED OR ANCHORED INSTRUMENT. THE MEASUREMENTS

SUPPORT STUDIES FOR DETERMINING SEA SLOPE, TIDAL AND STORM FLOWS AND

CIRCULATION PATTERNS, WHICH PROVIDE INFORMATION ON TRANSPORT

CHARACTERISTICS FOR A PARTICULAR AREA.

THE FORMAT CONSISTS  OF  FOUR  DATA  RECORDS  FOR  REPORTING  TOTAL

PRESSURE  (IN  DECIBARS)  VS  TIME  AS  WELL  AS INSTRUMENT POSITION AND

INSTRUMENT  DEPTH,  DATES  OF  OPERATION,  WATER  DEPTH,  GAUGE  NUMBER,

INSTITUTION AND OTHER SUPPLEMENTARY INFORMATION INCLUDING A  RECORD  FOR

TEXT.

ALL RECORDS IN THIS FORMAT ARE 50 COLUMNS IN LENGTH. DATA CAN BE

REPORTED FOR ANY TIME INTERVAL (ACTUAL OR FILTERED) AND IS EXPRESSED IN

HOURS AND MINUTES. THIS FILE IS SORTED BY STATION NUMBER (GAUGE NUMBER)

RECORD TYPE AND SEQUENCE NUMBER TO OBTAIN THE PROPER SEQUENCE OF

RECORDS.

THE FIRST NINE COLUMNS FOR ALL RECORDS ARE TO BE USED FOR FILE TYPE

(COLUMNS 1-3) AND FILE IDENTIFIER (COLUMNS 4-9). THE FILE IDENTIFIER, TO

BE ASSIGNED BY THE ORIGNATOR, IS A UNIQUE ORIGINATOR ID FOR EACH DATA

SUBMISSION. AFTER SUBMISSION, THE NODC REASSIGNS TO THIS FIELD A UNIQUE

NODC IDENTIFIER FOR INTERNAL USE.

****4/1/85 RECORD '2' - ADDED STATION IDENTIFIER****

%

017/PG 1

PARAMETER                      DESCRIPTION                        SC

TEXT RECORD                    ALWAYS '1'                              10

GAUGE NUMBER              FIVE-CHARACTER FIELD ASSIGNED BY THE    11

ORIGINATOR - ALSO INCLUDED ON RECORD

TYPES 2, 3 AND 4

TEXT                      TWENTY-CHARACTER FIELD FOR COMMENTS OR  16

PERTINENT INFORMATION

SEQUENCE NUMBER           XXXXX - USED FOR SORTING TEXT RECORDS   36

BLANKS                                                            41

GAUGE MASTER RECORD I          ALWAYS '2'                              10

GAUGE NUMBER              SEE RECORD '1'                          11

LATITUDE                  DDMMXX   PLUS HEMISPHERE 'N' OR 'S' -   16

MINUTES TO HUNDREDTHS

LONGITUDE                 DDDMMXX   PLUS HEMISPHERE 'E' OR 'W' -  23

MINUTES TO HUNDREDTHS

DEPTH OF PRESSURE         XXXXX (METERS TO TENTHS)                31

GAUGE

NUMBER OF DETAIL          XXXXX - USED TO INDICATE NUMBER OF      36

RECORDS                 DETAIL RECORDS (4) TO FOLLOW

STATION IDENTIFIER        10-CHARACTER ORIGINATOR STATION         41

IDENTIFIER

GAUGE MASTER RECORD II         ALWAYS '3'                              10

GAUGE NUMBER              SEE RECORD '1'                          11

DEPTH TO BOTTOM           XXXXX (WHOLE METERS)                    16

METER USAGE SEQUENCE      XXX - USED FOR INDICATING NUMBER OF     21

NUMBER                  TIMES METER HAS BEEN USED

INSTITUTION               TWO-CHARACTER NODC INSTITUTION CODE -   24

USE CODE 0218

LOCATION NAME             SIX-CHARACTER NAME DETERMINED BY THE    26

ORIGINATOR

BLANKS                                                            32

DETAIL RECORD                  ALWAYS '4'                              10

GAUGE NUMBER              SEE RECORD '1'                          11

DATE (GMT)                YYMMDD                                  16

TIME (GMT)                XXXXXX (HOURS, MINUTES TO HUNDREDTHS)   22

TOTAL PRESSURE            XXXXXXXX (DECIBARS TO THOUSANDTHS)      28

SEQUENCE NUMBER           XXXXX - USED FOR SORTING DATA RECORDS   36

TEMPERATURE               XXXXX NEGATIVE TEMPERATURES ARE         41

PRECEDED BY A MINUS SIGN ADJACENT TO

TEMPERATURE VALUE - DEG C TO THOUSANDTHS

BLANKS                                                            46

%%022

FILE TYPE 022 - SALINITY/TEMPERATURE/DENSITY MEASUREMENTS (STD/CTD)

08/20/82 VERSION

THIS FORMAT IS DESIGNED TO RECORD MICROSTRUCTURE MEASUREMENTS OF

SALINITY OR CONDUCTIVITY, TEMPERATURE AND SIGMA-T VALUES VS DEPTH TO

SUPPORT STUDIES OF TRANSPORT AND ALTERATION OF CONTAMINANTS BY THE

ENVIRONMENT.

THIS FORMAT CONSISTS OF EIGHT RECORDS WHICH INCLUDE, IN ADDITION TO

SIX ENTRIES OF TEMPERATURE, SALINITY, SIGMA-T PER RECORD, EXTENSIVE

SEA SURFACE AND CLIMATOLOGY FIELDS, POSITION, DATE, TIME AND DEPTH

INFORMATION AND A TEXT RECORD.

DATA CAN BE RECORDED AT DIFFERENT DEPTH INTERVALS WHICH ARE

IDENTIFIED IN THE SCAN FIELD. A RECENT ADDITION TO THE FORMAT IS A RECORD

WITH DISSOLVED OXYGEN AND TRANSMISSIVITY FIELDS FORMATTED SIMILAR TO THE

TEMPERATURE/SALINITY DATA RECORD.

ALL RECORDS IN THIS FORMAT ARE 120 COLUMNS IN LENGTH. THIS FILE IS

SORTED BY STATION NUMBER (CAST NUMBER), RECORD TYPE AND SEQUENCE NUMBER

TO OBTAIN THE PROPER SEQUENCE OF RECORDS.

THE FIRST NINE COLUMNS FOR ALL RECORDS ARE TO BE USED FOR FILE TYPE

(COLUMNS 1-3) AND FILE IDENTIFIER (COLUMNS 4-9). THE FILE IDENTIFIER, TO

BE ASSIGNED BY THE ORIGNATOR, IS A UNIQUE ORIGINATOR ID FOR EACH DATA

SUBMISSION. AFTER SUBMISSION, THE NODC REASSIGNS TO THIS FIELD A UNIQUE

NODC IDENTIFIER FOR INTERNAL USE.

*****FILETYPE 022 - 3/30/79 - ADDED NEW DETAIL RECORD 3 -RECORD *****

*****TYPE '5'                                                   *****

***** 12/21/81 - ADD NEW RECORD TYPES '6' AND '7'        *****

***** 04/21/82 - ADD NEW RECORD TYPE '6'          *****

****4/1/85 RECORD '2' - ADDED SALINITY FLAG****

****6/1/89 RECORD '2' - ADDED CAST DIRECTION****

%

022/PG 1

PARAMETER                    DESCRIPTION                       SC

TEXT RECORD                    ALWAYS '1'                              10

CAST NUMBER               FIVE-CHARACTER FIELD ASSIGNED BY THE    11

ORIGINATOR - ALSO INCLUDED ON RECORD

TYPES 2,3 AND 4

TEXT                      100-CHARACTER FIELD - USED FOR COMMENTS 16

OR PERTINENT INFORMATION

SEQUENCE NUMBER           XXXXX - USED FOR SORTING TEXT RECORDS   116

MASTER RECORD                  ALWAYS '2'                              10

CAST NUMBER               SEE RECORD '1'                          11

LATITUDE                  DDMMXX   PLUS HEMISPHERE 'N' OR 'S' -   16

MINUTES TO HUNDREDTHS

LONGITUDE                 DDDMMXX   PLUS HEMISPHERE 'E' OR 'W' -  23

MINUTES TO HUNDREDTHS

CRUISE IDENTIFICATION     TEN-CHARACTER FIELD ASSIGNED BY THE     31

ORIGINATOR

NUMBER OF SCANS           XXXXX - USED TO INDICATE NUMBER OF      41

SCANS PER STATION (FIVE/RECORD)

DATE (GMT)                YYMMDD                                  46

TIME (GMT)                XXXX (HOURS AND MINUTES)                52

SAMPLE INTERVAL INDICATOR ONE-DIGIT CODE - USE CODE 0216          56

SAMPLE INTERVAL           XXX  - WHEN INDICATOR CODE=1 (EQUAL     57

SPACED DEPTHS) - (METERS TO TENTHS)

BAROMETRIC PRESSURE       XXXXX (MILLIBARS TO TENTHS)             60

WET BULB TEMPERATURE      XXXX NEGATIVE TEMPERATURES ARE PRECEDED 65

BY A MINUS SIGN ADJACENT TO TEMPERATURE

VALUE - DEG C TO TENTHS

DRY BULB TEMPERATURE      XXXX NEGATIVE TEMPERATURES ARE PRECEDED 69

BY A MINUS SIGN ADJACENT TO TEMPERATURE

VALUE - DEG C TO TENTHS

WIND DIRECTION            XX - TWO-DIGIT CODE - WMO 885/887 -     73

DIRECTION FROM - USE CODE 0110

WIND SPEED                XX (WHOLE KNOTS)                        75

WEATHER                   ONE-DIGIT CODE - WMO 4501 - USE CODE    77

0108

SEA STATE                 ONE-DIGIT CODE - WMO 3700 - USE CODE    78

0109

VISIBILITY                ONE-DIGIT CODE - WMO 4300 - USE CODE    79

0157

CLOUD TYPE                ONE-DIGIT CODE - WMO 0500 - USE CODE    80

0053

CLOUD AMOUNT              ONE-DIGIT CODE - WMO 2700 - USE CODE    81

0105

INSTRUMENT INFORMATION    TWENTY-CHARACTER FIELD FOR TYPE OF      82

INSTRUMENT, SERIAL NUMBER, ETC

LOCATION NAME             SIX-CHARACTER NAME DETERMINED BY THE    102

ORIGINATOR

DEPTH TO BOTTOM           XXXXX (WHOLE METERS)                    108

MAXIMUM DEPTH OF CAST     XXXX (WHOLE METERS)                     113

SALINITY FLAG             ONE CHARACTER CODE - USE CODE 0502      117

CAST DIRECTION            ONE CHARACTER CODE - USE CODE 0508      118

BLANKS                                                            119

%

022/PG 2

DETAIL RECORD 1                ALWAYS '3'                              10

CAST NUMBER               SEE RECORD '1'                          11

DEPTH                     XXXXX (METERS TO TENTHS)                16

TEMPERATURE               XXXXX NEGATIVE TEMPERATURES ARE         21

PRECEDED BY A MINUS SIGN ADJACENT TO

TEMPERATURE VALUE - DEG C TO THOUSANDTHS

SALINITY                  XXXXX - PARTS PER THOUSAND TO           26

THOUSANDTHS

SIGMA-T                   XXXX - TO HUNDREDTHS                    31

SCAN CONDITION            ONE-CHARACTER CODE INDICATING METHOD OF 35

SCANNING DATA - USE CODE 0080

DEPTH                     XXXXX (METERS TO TENTHS)                36

TEMPERATURE               XXXXX NEGATIVE TEMPERATURES ARE         41

PRECEDED BY A MINUS SIGN ADJACENT TO

TEMPERATURE VALUE - DEG C TO THOUSANDTHS

SALINITY                  XXXXX - PARTS PER THOUSAND TO           46

THOUSANDTHS

SIGMA-T                   XXXX - TO HUNDREDTHS                    51

SCAN CONDITION            ONE-CHARACTER CODE INDICATING METHOD OF 55

SCANNING DATA - USE CODE 0080

DEPTH                     XXXXX (METERS TO TENTHS)                56

TEMPERATURE               XXXXX NEGATIVE TEMPERATURES ARE         61

PRECEDED BY A MINUS SIGN ADJACENT TO

TEMPERATURE VALUE - DEG C TO THOUSANDTHS

SALINITY                  XXXXX - PARTS PER THOUSAND TO           66

THOUSANDTHS

SIGMA-T                   XXXX - TO HUNDREDTHS                    71

SCAN CONDITION            ONE-CHARACTER CODE INDICATING METHOD OF 75

SCANNING DATA - USE CODE 0080

DEPTH                     XXXXX (METERS TO TENTHS)                76

TEMPERATURE               XXXXX NEGATIVE TEMPERATURES ARE         81

PRECEDED BY A MINUS SIGN ADJACENT TO

TEMPERATURE VALUE - DEG C TO THOUSANDTHS

SALINITY                  XXXXX - PARTS PER THOUSAND TO           86

THOUSANDTHS

SIGMA-T                   XXXX - TO HUNDREDTHS                    91

SCAN CONDITION            ONE-CHARACTER CODE INDICATING METHOD OF 95

SCANNING DATA - USE CODE 0080

DEPTH                     XXXXX (METERS TO TENTHS)                96

TEMPERATURE               XXXXX NEGATIVE TEMPERATURES ARE         101

PRECEDED BY A MINUS SIGN ADJACENT TO

TEMPERATURE VALUE - DEG C TO THOUSANDTHS

SALINITY                  XXXXX - PARTS PER THOUSAND TO           106

THOUSANDTHS

SIGMA-T                   XXXX - TO HUNDREDTHS                    111

SCAN CONDITION            ONE-CHARACTER CODE INDICATING METHOD OF 115

SCANNING DATA - USE CODE 0080

SEQUENCE NUMBER           XXXXX - USED FOR SORTING DATA RECORDS   116

%

022/PG 3

DETAIL RECORD 2                ALWAYS '4'                              10

CAST NUMBER               SEE RECORD '1'                          11

DEPTH                     XXXXX (METERS TO TENTHS)                16

DISSOLVED OXYGEN          XXXXX - ML/L TO THOUSANDTHS             21

TRANSMISSIVITY            XXXXX (PERCENT TO THOUSANDTHS)          26

BLANKS                                                            31

SCAN CONDITION            ONE-CHARACTER CODE INDICATING METHOD OF 35

SCANNING DATA - USE CODE 0080

DEPTH                     XXXXX (METERS TO TENTHS)                36

DISSOLVED OXYGEN          XXXXX - ML/L TO THOUSANDTHS             41

TRANSMISSIVITY            XXXXX (PERCENT TO THOUSANDTHS)          46

BLANKS                                                            51

SCAN CONDITION            ONE-CHARACTER CODE INDICATING METHOD OF 55

SCANNING DATA - USE CODE 0080

DEPTH                     XXXXX (METERS TO TENTHS)                56

DISSOLVED OXYGEN          XXXXX - ML/L TO THOUSANDTHS             61

TRANSMISSIVITY            XXXXX (PERCENT TO THOUSANDTHS)          66

BLANKS                                                            71

SCAN CONDITION            ONE-CHARACTER CODE INDICATING METHOD OF 75

SCANNING DATA - USE CODE 0080

DEPTH                     XXXXX (METERS TO TENTHS)                76

DISSOLVED OXYGEN          XXXXX - ML/L TO THOUSANDTHS             81

TRANSMISSIVITY            XXXXX (PERCENT TO THOUSANDTHS)          86

BLANKS                                                            91

SCAN CONDITION            ONE-CHARACTER CODE INDICATING METHOD OF 95

SCANNING DATA - USE CODE 0080

DEPTH                     XXXXX (METERS TO TENTHS)                96

DISSOLVED OXYGEN          XXXXX - ML/L TO THOUSANDTHS             101

TRANSMISSIVITY            XXXXX (PERCENT TO THOUSANDTHS)          106

BLANKS                                                            111

SCAN CONDITION            ONE-CHARACTER CODE INDICATING METHOD OF 115

SCANNING DATA - USE CODE 0080

SEQUENCE NUMBER           XXXXX - USED FOR SORTING DATA RECORDS   116

DETAIL RECORD 3                ALWAYS '5'                              10

CAST NUMBER               SEE RECORD '1'                          11

DEPTH                     XXXXX (METERS TO TENTHS)                16

TEMPERATURE               XXXXX NEGATIVE TEMPERATURES ARE         21

PRECEDED BY A MINUS SIGN ADJACENT TO

TEMPERATURE VALUE - DEG C TO THOUSANDTHS

CONDUCTIVITY              XXXXX (MMHO/CM TO THOUSANDTHS)          26

BLANKS                                                            31

SCAN CONDITION            ONE-CHARACTER CODE INDICATING METHOD OF 35

SCANNING DATA - USE CODE 0080

DEPTH                     XXXXX (METERS TO TENTHS)                36

TEMPERATURE               XXXXX NEGATIVE TEMPERATURES ARE         41

PRECEDED BY A MINUS SIGN ADJACENT TO

TEMPERATURE VALUE - DEG C TO THOUSANDTHS

CONDUCTIVITY              XXXXX (MMHO/CM TO THOUSANDTHS)          46

BLANKS                                                            51

SCAN CONDITION            ONE-CHARACTER CODE INDICATING METHOD OF 55

SCANNING DATA - USE CODE 0080

DEPTH                     XXXXX (METERS TO TENTHS)                56

%

022/PG 4

TEMPERATURE               XXXXX NEGATIVE TEMPERATURES ARE         61

PRECEDED BY A MINUS SIGN ADJACENT TO

TEMPERATURE VALUE - DEG C TO THOUSANDTHS

CONDUCTIVITY              XXXXX (MMHO/CM TO THOUSANDTHS)          66

BLANKS                                                            71

SCAN CONDITION            ONE-CHARACTER CODE INDICATING METHOD OF 75

SCANNING DATA - USE CODE 0080

DEPTH                     XXXXX (METERS TO TENTHS)                76

TEMPERATURE               XXXXX NEGATIVE TEMPERATURES ARE         81

PRECEDED BY A MINUS SIGN ADJACENT TO

TEMPERATURE VALUE - DEG C TO THOUSANDTHS

CONDUCTIVITY              XXXXX (MMHO/CM TO THOUSANDTHS)          86

BLANKS                                                            91

SCAN CONDITION            ONE-CHARACTER CODE INDICATING METHOD OF 95

SCANNING DATA - USE CODE 0080

DEPTH                     XXXXX (METERS TO TENTHS)                96

TEMPERATURE               XXXXX NEGATIVE TEMPERATURES ARE         101

PRECEDED BY A MINUS SIGN ADJACENT TO

TEMPERATURE VALUE - DEG C TO THOUSANDTHS

CONDUCTIVITY              XXXXX (MMHO/CM TO THOUSANDTHS)          106

BLANKS                                                            111

SCAN CONDITION            ONE-CHARACTER CODE INDICATING METHOD OF 115

SCANNING DATA - USE CODE 0080

SEQUENCE NUMBER           XXXXX - USED FOR SORTING DATA RECORDS   116

%

022/PG 5

DETAIL RECORD 4                ALWAYS '6'                              10

CAST NUMBER               SEE RECORD '1'                          11

PRESSURE                  XXXXX (DECIBARS TO TENTHS)              16

TEMPERATURE               XXXXX NEGATIVE TEMPERATURES ARE         21

PRECEDED BY A MINUS SIGN ADJACENT TO

TEMPERATURE VALUE - DEG C TO THOUSANDTHS

SALINITY                  XXXXX - PARTS PER THOUSAND TO           26

THOUSANDTHS

SIGMA-T                   XXXX - TO HUNDREDTHS                    31

SCAN CONDITION            ONE-CHARACTER CODE INDICATING METHOD OF 35

SCANNING DATA - USE CODE 0080

PRESSURE                  XXXXX (DECIBARS TO TENTHS)              36

TEMPERATURE               XXXXX NEGATIVE TEMPERATURES ARE         41

PRECEDED BY A MINUS SIGN ADJACENT TO

TEMPERATURE VALUE - DEG C TO THOUSANDTHS

SALINITY                  XXXXX - PARTS PER THOUSAND TO           46

THOUSANDTHS

SIGMA-T                   XXXX - TO HUNDREDTHS                    51

SCAN CONDITION            ONE-CHARACTER CODE INDICATING METHOD OF 55

SCANNING DATA - USE CODE 0080

PRESSURE                  XXXXX (DECIBARS TO TENTHS)              56

TEMPERATURE               XXXXX NEGATIVE TEMPERATURES ARE         61

PRECEDED BY A MINUS SIGN ADJACENT TO

TEMPERATURE VALUE - DEG C TO THOUSANDTHS

SALINITY                  XXXXX - PARTS PER THOUSAND TO           66

THOUSANDTHS

SIGMA-T                   XXXX - TO HUNDREDTHS                    71

SCAN CONDITION            ONE-CHARACTER CODE INDICATING METHOD OF 75

SCANNING DATA - USE CODE 0080

PRESSURE                  XXXXX (DECIBARS TO TENTHS)              76

TEMPERATURE               XXXXX NEGATIVE TEMPERATURES ARE         81

PRECEDED BY A MINUS SIGN ADJACENT TO

TEMPERATURE VALUE - DEG C TO THOUSANDTHS

SALINITY                  XXXXX - PARTS PER THOUSAND TO           86

THOUSANDTHS

SIGMA-T                   XXXX - TO HUNDREDTHS                    91

SCAN CONDITION            ONE-CHARACTER CODE INDICATING METHOD OF 95

SCANNING DATA - USE CODE 0080

PRESSURE                  XXXXX (DECIBARS TO TENTHS)              96

TEMPERATURE               XXXXX NEGATIVE TEMPERATURES ARE         101

PRECEDED BY A MINUS SIGN ADJACENT TO

TEMPERATURE VALUE - DEG C TO THOUSANDTHS

SALINITY                  XXXXX - PARTS PER THOUSAND TO           106

THOUSANDTHS

SIGMA-T                   XXXX - TO HUNDREDTHS                    111

SCAN CONDITION            ONE-CHARACTER CODE INDICATING METHOD OF 115

SCANNING DATA - USE CODE 0080

SEQUENCE NUMBER           XXXXX - USED FOR SORTING DATA RECORDS   116

%

022/PG 6

DETAIL RECORD 5                ALWAYS '7'                              10

CAST NUMBER               SEE RECORD '1'                          11

PRESSURE                  XXXXX (DECIBARS TO TENTHS)              16

TEMPERATURE               XXXXX NEGATIVE TEMPERATURES ARE         21

PRECEDED BY A MINUS SIGN ADJACENT TO

TEMPERATURE VALUE - DEG C TO THOUSANDTHS

CONDUCTIVITY              XXXXX (MMHO/CM TO THOUSANDTHS)          26

BLANKS                                                            31

SCAN CONDITION            ONE-CHARACTER CODE INDICATING METHOD OF 35

SCANNING DATA - USE CODE 0080

PRESSURE                  XXXXX (DECIBARS TO TENTHS)              36

TEMPERATURE               XXXXX NEGATIVE TEMPERATURES ARE         41

PRECEDED BY A MINUS SIGN ADJACENT TO

TEMPERATURE VALUE - DEG C TO THOUSANDTHS

CONDUCTIVITY              XXXXX (MMHO/CM TO THOUSANDTHS)          46

BLANKS                                                            51

SCAN CONDITION            ONE-CHARACTER CODE INDICATING METHOD OF 55

SCANNING DATA - USE CODE 0080

PRESSURE                  XXXXX (DECIBARS TO TENTHS)              56

TEMPERATURE               XXXXX NEGATIVE TEMPERATURES ARE         61

PRECEDED BY A MINUS SIGN ADJACENT TO

TEMPERATURE VALUE - DEG C TO THOUSANDTHS

CONDUCTIVITY              XXXXX (MMHO/CM TO THOUSANDTHS)          66

BLANKS                                                            71

SCAN CONDITION            ONE-CHARACTER CODE INDICATING METHOD OF 75

SCANNING DATA - USE CODE 0080

PRESSURE                  XXXXX (DECIBARS TO TENTHS)              76

TEMPERATURE               XXXXX NEGATIVE TEMPERATURES ARE         81

PRECEDED BY A MINUS SIGN ADJACENT TO

TEMPERATURE VALUE - DEG C TO THOUSANDTHS

CONDUCTIVITY              XXXXX (MMHO/CM TO THOUSANDTHS)          86

BLANKS                                                            91

SCAN CONDITION            ONE-CHARACTER CODE INDICATING METHOD OF 95

SCANNING DATA - USE CODE 0080

PRESSURE                  XXXXX (DECIBARS TO TENTHS)              96

TEMPERATURE               XXXXX NEGATIVE TEMPERATURES ARE         101

PRECEDED BY A MINUS SIGN ADJACENT TO

TEMPERATURE VALUE - DEG C TO THOUSANDTHS

CONDUCTIVITY              XXXXX (MMHO/CM TO THOUSANDTHS)          106

BLANKS                                                            111

SCAN CONDITION            ONE-CHARACTER CODE INDICATING METHOD OF 115

SCANNING DATA - USE CODE 0080

SEQUENCE NUMBER           XXXXX - USED FOR SORTING DATA RECORDS   116

%

022/PG 7

DETAIL RECORD 6                ALWAYS '8'                              10

CAST NUMBER               SEE RECORD '1'                          11

PRESSURE                  XXXXX - DB TO TENTHS                    16

TEMPERATURE               XXXXX - DEG C TO THOUSANDTHS            21

SALINITY                  XXXXX - PPT TO THOUSANDTHS              26

DISSOLVED OXYGEN          XXXX - MILLILITERS/LITER                31

SCAN CONDITION CODE       ONE-CHARACTER CODE - USE 0080           35

PRESSURE                  XXXXX - DB TO TENTHS                    36

TEMPERATURE               XXXXX - DEG C TO THOUSANDTHS            41

SALINITY                  XXXXX - PPT TO THOUSANDTHS              46

DISSOLVED OXYGEN          XXXX - MILLILITERS/LITER                51

SCAN CONDITION CODE       ONE-CHARACTER CODE - USE 0080           55

PRESSURE                  XXXXX - DB TO TENTHS                    56

TEMPERATURE               XXXXX - DEG C TO THOUSANDTHS            61

SALINITY                  XXXXX - PPT TO THOUSANDTHS              66

DISSOLVED OXYGEN          XXXX - MILLILITERS/LITER                71

SCAN CONDITION CODE       ONE-CHARACTER CODE - USE 0080           75

PRESSURE                  XXXXX - DB TO TENTHS                    76

TEMPERATURE               XXXXX - DEG C TO THOUSANDTHS            81

SALINITY                  XXXXX - PPT TO THOUSANDTHS              86

DISSOLVED OXYGEN          XXXX - MILLILITERS/LITER                91

SCAN CONDITION CODE       ONE-CHARACTER CODE - USE 0080           95

PRESSURE                  XXXXX - DB TO TENTHS                    96

TEMPERATURE               XXXXX - DEG C TO THOUSANDTHS            101

SALINITY                  XXXXX - PPT TO THOUSANDTHS              106

DISSOLVED OXYGEN          XXXX - MILLILITERS/LITER                111

SCAN CONDITION CODE       ONE-CHARACTER CODE - USE 0080           115

SEQUENCE NUMBER           XXXXX - USED FOR SORTING DATA RECORDS   116

FILE TYPE 015 - CURRENT METER (EULERIAN) - 3/30/79 VERSION

NOTES AND CORRECTIONS

THIS FORMAT IS DESIGNED TO RECORD TIME SERIES MEASUREMENTS

FOR ARCHIVED OR FIXED CURRENT METER ARRAYS FOR ANY WATER DEPTH.

THESE MEASUREMENTS SUPPORT STUDIES TO DETERMINE CIRCULATION AND

TRANSPORT PATTERNS IN OFFSHORE AND NEARSHORE OCEAN REGIMES.

THE FORMAT CONSISTS OF FOUR DATA RECORDS FOR REPORTING

CURRENT COMPONENTS, TEMPERATURE, PRESSURE AND SALINITY OR CONDUCTIVITY

AS WELL AS METER POSITION AND METER DEPTH, DATES OF OPERATION, WATER

DEPTH, METER NUMBER, INSTITUTION AND OTHER SUPPLEMENTARY INFORMATION

INCLUDING A RECORD FOR TEXT.

DATA CAN BE REPORTED OVER ANY ACTUAL OR FILTERED TIME

INTERVAL AND IS EXPRESSED IN HOURS AND MINUTES. DIRECTION AND SPEED ARE

EXPRESSED IN TERMS OF U AND V COMPONENTS IN CM/SEC WITH POSITIVE DIRECTIONS

EAST AND NORTH AND NEGATIVE DIRECTIONS WEST AND SOUTH. CURRENTS ARE

INDICATED AS DIRECTION 'TOWARD'.

ALL RECORDS IN THIS FORMAT ARE 60 COLUMNS IN LENGTH. THIS

FILE IS SORTED BY STATION NUMBER (METER NUMBER), RECORD TYPE AND

SEQUENCE NUMBER TO OBTAIN THE PROPER SEQUENCE OF RECORDS.

THE FIRST NINE COLUMNS FOR ALL RECORDS ARE TO BE USED FOR

FILE TYPE (COLUMNS 1-3) AND FILE IDENTIFIER (COLUMNS 4-9). THE FILE

IDENTIFIER, TO BE ASSIGNED BY THE ORIGNATOR, IS A UNIQUE ORIGINATOR ID FOR EACH

DATA SUBMISSION. AFTER SUBMISSION, THE NODC REASSIGNS TO THIS FIELD A

UNIQUE NODC IDENTIFIER FOR INTERNAL USE.

***** FILETYPE 015 - 3/30/79 - SALINITY FIELD (SC 50) EXTENDED TO 5 BYTES *****

PARAMETER                      DESCRIPTION                             SC

TEXT RECORD

NODC FILE NUMBER               ALWAYS "015"                             1

NODC TRACK NUMBER              6-CHARACTER UNIQUE CRUISE OR DATA SET    4

IDENTIFIER ASSIGNED BY NODC

RECORD NUMBER                  ALWAYS '1'                              10

METER NUMBER                   FIVE-CHARACTER FIELD ASSIGNED BY        11

THE ORIGINATOR - ALSO INCLUDED ON

RECORD TYPES 2 AND 3

TEXT                           THIRTY-EIGHT CHARACTER FIELD FOR        16

COMMENTS OR PERTINENT INFORMATION

BLANK                                                                  54

SEQUENCE NUMBER                XXXXXX - USED FOR SORTING TEXT          55

INFORMATION

MASTER RECORD

NODC FILE NUMBER               ALWAYS "015"                             1

NODC TRACK NUMBER              6-CHARACTER UNIQUE CRUISE OR DATA SET    4

IDENTIFIER ASSIGNED BY NODC

RECORD NUMBER                  ALWAYS '2'                              10

METER NUMBER                   SEE RECORD '1'                          11

LATITUDE                       DDMMXX   PLUS HEMISPHERE 'N' OR 'S' -   16

MINUTES TO HUNDREDTHS

LONGITUDE                      DDDMMXX   PLUS HEMISPHERE 'E' OR 'W' -  23

MINUTES TO HUNDREDTHS

DEPTH OF BOTTOM                XXXXX (WHOLE METERS)                    31

DEPTH OF CURRENT               XXXXX (METERS TO TENTHS)                36

METER

METER USAGE SEQUENCE           XXX - USED FOR INDICATING NUMBER OF     41

NUMBER                        TIMES METER HAS BEEN USED

INSTITUTION                    TWO-CHARACTER NODC INSTITUTION CODE -   44

USE CODE 0218

AXIS ROTATION                  XXX - DEGREES CLOCKWISE FROM TRUE NORTH 46

OF V AXIS - VALUES SHOULD BE 0 WHEN

FINAL PROCESSED TO PROVIDE TRUE

DIRECTION INFORMATION

LOCATION NAME                  SIX-CHARACTER NAME DETERMINED BY        49

ORIGINATOR

NUMBER OF DETAIL               XXXXXX - USED TO INDICATE NUMBER OF     55

RECORDS                       DETAIL RECORDS (3) TO FOLLOW THE MASTER

RECORD (2)

DETAIL RECORD 1

NODC FILE NUMBER               ALWAYS "015"                             1

NODC TRACK NUMBER              6-CHARACTER UNIQUE CRUISE OR DATA SET    4

IDENTIFIER ASSIGNED BY NODC

RECORD NUMBER                  ALWAYS '3'                              10

METER NUMBER                   SEE RECORD '1'                          11

DATE (GMT)                     YYMMDD                                  16

TIME (GMT)                     XXXXXX (HOURS, MINUTES TO HUNDREDTHS)   22

EAST-WEST CURRENT              XXXXXX - CM/SEC TO HUNDREDTHS WITH      28

COMPONENT (U)                 POSITIVE DIRECTIONS (EAST AND NORTH)

INDICATED WITHOUT PLUS SIGN - NEGATIVE

DIRECTIONS (WEST AND SOUTH) PRECEDED

BY MINUS SIGN - DIRECTION TOWARD

NORTH-SOUTH CURRENT            XXXXXX - CM/SEC TO HUNDREDTHS WITH      34

COMPONENT (V)                 POSITIVE DIRECTIONS (EAST AND NORTH)

INDICATED WITHOUT PLUS SIGN - NEGATIVE

DIRECTIONS (WEST AND SOUTH) PRECEDED

BY MINUS SIGN - DIRECTION TOWARD

TEMPERATURE                    XXXXX  WITH NEGATIVE TEMPERATURES       40

PRECEDED BY MINUS SIGN (DEG C TO

THOUSANDTHS)

PRESSURE                       XXXXX (DECIBARS TO TENTHS)              45

CONDUCTIVITY                   XXXX - MMHOS/CM TO HUNDREDTHS           50

BLANK                                                                  54

SEQUENCE NUMBER                XXXXXX - USED FOR SORTING DATA RECORDS  55

DETAIL RECORD 2

NODC FILE NUMBER               ALWAYS "015"                             1

NODC TRACK NUMBER              6-CHARACTER UNIQUE CRUISE OR DATA SET    4

IDENTIFIER ASSIGNED BY NODC

RECORD NUMBER                  ALWAYS '4'                              10

METER NUMBER                   SEE RECORD '1'                          11

DATE (GMT)                     YYMMDD                                  16

TIME (GMT)                     XXXXXX (HOURS, MINUTES TO HUNDREDTHS)   22

EAST-WEST CURRENT              XXXXXX - CM/SEC TO HUNDREDTHS WITH      28

COMPONENT (U)                 POSITIVE DIRECTIONS (EAST AND NORTH)

INDICATED WITHOUT PLUS SIGN - NEGATIVE

DIRECTIONS (WEST AND SOUTH) PRECEDED

BY MINUS SIGN - DIRECTION TOWARD

NORTH-SOUTH CURRENT            XXXXXX - CM/SEC TO HUNDREDTHS WITH      34

COMPONENT (V)                 POSITIVE DIRECTIONS (EAST AND NORTH)

INDICATED WITHOUT PLUS SIGN - NEGATIVE

DIRECTIONS (WEST AND SOUTH) PRECEDED

BY MINUS SIGN

TEMPERATURE                    XXXXX  WITH NEGATIVE TEMPERATURES       40

PRECEDED BY MINUS SIGN (DEG C TO

THOUSANDTHS)

PRESSURE                       XXXXX (DECIBARS TO TENTHS)              45

SALINITY                       XXXXX  PARTS PER THOUSAND TO            50

THOUSANDTHS

SEQUENCE NUMBER                XXXXXX - USED FOR SORTING DATA RECORDS  55

NODC CODES USED WITH THIS FORMAT:

NODC Code Tables for use with F015:

0218   DATA SOURCE

----   --------------------

02 -- WOODS HOLE OCEANOGRAPHIC INST

03 -- OREGON STATE UNIVERSITY

09 -- UNIVERSITY OF WASHINGTON(SEATTLE)

12 -- COLUMBIA UNIV. LAMONT-DOHERTY GEOL OBSERVATORY

(PALISADES N.Y.)

3F -- PMEL-UNIVERSITY OF WASHINGTON(SEATTLE)

BC -- BROWN & CALDWELL (COSTA MESA, CA.)

CI -- UNIV. OF ALASKA

DS -- DOBROCKY SEATECH (ANCHORAGE, ALASKA)

I7 -- UNIVERSITY OF ALASKA-IMS (FAIRBANKS)

TB -- NATIONAL OCEAN SURVEY, PMC (SEATTLE)