Explorer of the Seas METADATA - 2004

Class of Data:  Surface ocean carbon dioxide concentrations

Dataset Identifier:  AOML_Explorer

Two Files:  Explorer_2004_East

Explorer_2004_West

Statement of how to cite dataset:

Explorer website: http://www.aoml.noaa.gov/ocd/gcc/explorer_cruisetracks.php

These data are made freely available to the public and the scientific community

in the belief that their wide dissemination will lead to greater understanding

and new scientific insights.  The availability of these data does not

constitute publication of the data.  We rely on the ethics and integrity of

the user to assure that AOML receives fair credit for our work.  Please send

manuscripts using this data to AOML for review before they are submitted

for publication so we can insure that the quality and limitations of the data

are accurately represented.

Measurement platform identifier:  Explorer of the Seas

Cruise Information:

Weekly cruises through the Caribbean departing from Miami, Florida.

Project Information:

This project represents a collaboration between Royal Caribbean

International, the University of Miami's Rosenstiel School of Marine

and Atmospheric Science, the National Oceanic and Atmospheric

Administration, the National Science Foundation, the Office of

Naval Research, and the National Aeronautics and Space Administration.

Scientist responsible for technical quality of dataset:

Rik Wanninkhof

NOAA/AOML/Ocean Chemistry Division

4301 Rickenbacker Causeway

Miami, Florida  33149

Rik.Wanninkhof@noaa.gov

Contact person for this dataset:

Betty Huss

NOAA/AOML/Ocean Chemistry Division

4301 Rickenbacker Causeway

Miami, Florida  33149

Betty.Huss@noaa.gov

Timestamp for initial submission of dataset: 7/11/07

Timestamp for the most recent update of dataset:  7/11/07

Timestamp period the dataset refers to: 1/02/2004 - 1/02/2005

Geographic area the dataset refers to:

16 N to 27 N

63 W to 88 W

Eastern Cruise Track:

Day 1 - Miami, Florida (25.8 N/80.7 W)

Day 3 - San Juan, Puerto Rico (18.5 N/66.1 W)

Day 4 - Philipsburg, St. Maarten (18.0 N/63.0 W)

Day 5 - Charlotte Amalie, St. Thomas (18.3 N/64.9 W)

Day 7 - Nassau, Bahamas (25.1 N/77.3 W)

Day 8 - Miami, Florida (25.8 N/80.7 W)

Western Cruise Track for EX0401W and EX0403W:

Day 1 - Miami, Florida (25.8 N/80.7 W

Day 3 – Labadee, Hispaniola (19.8 N/72.3 W)

Day 4 – Ocho Rios, Jamaica (18.4 N/77.1 W)

Day 6 – Cozumel, Mexico (20.5 N/87.0 W)

Day 8 - Miami, Florida (25.8 N/80.7 W)

Western Cruise Track for EX0405W, EX0409W, EX0411W,

EX0413W, EX0415W, and EX0417W:

Day 1 - Miami, Florida (25.8 N/80.7 W)

Day 3 – Labadee Hispaniola (19.8 N/72.3 W)

Day 4 - Ocho Rios, Jamaica (18.4 N/77.1 W)

Day 5 - George Town, Grand Cayman (19.3 N/81.4 W)

Day 6 - Cozumel, Mexico (20.5 N/87.0 W)

Day 8 - Miami, Florida (25.8 N/80.7 W)

Western Cruise Track for the remaining Western cruises:

Day 1 - Miami, Florida (25.8 N/80.7 W)

Day 3 - Belize City, Belize (17.8 N/88.3 W)

Day 4 - Costa Maya, Mexico (18.7 N/87.7 W)

Day 5 - Cozumel, Mexico (20.5 N/87.0 W)

Day 6 - George Town, Grand Cayman (19.3 N/81.4 W)

Day 8 - Miami, Florida (25.8 N/80.7 W)

List of variables included in this dataset:

COLUMN   HEADER               EXPLANATION

1.	   Group_Ship:	      AOML_Explorer for all Explorer of the Seas data.

2.       Cruise:			Cruise Name (For example, EX0546W:  EX = Explorer

of the Seas, 05 = 2005, 46 = 46th weekly cruise, and

W = western track).

3.	   JD_GMT:	            Decimal year day.

4.	   Date_DDMMYYYY:	      The date format has been changed to comply with the IOCCP

recommendations.

5.	   TIME_HH:MM:SS:	      GMT time.

NOTE: local time = GMT - 4 hr or GMT - 5 hr.

6.	   Lat_dec_degree:	Latitude in decimal degrees (negative values are in the

southern hemisphere).

7.	   Long_dec_degree:	Longitude in decimal degrees (negative values are in the

western latitudes).

8.	   xCO2eq_ppm:          Mole fraction of CO2 (dry) in the headspace equilibrator

at equilibrator temperature (Teq) in parts per million.

9.	   xCO2a_ppm:           Mole fraction of CO2 in air in parts per million.

This field is not measured on the Explorer of the Seas -

all data initialized to -9.

10.	   Pres_Equil_hPa:      Barometric pressure in the lab in hectopascals (1

hectopascal = 1 millibar).

11.	   Pres_sealevel_hPa:	Barometric pressure from ship's barometer, corrected to

sea level in hectopascals (1 hectopascal = 1 millibar).

12.	   EqTemp_C:            Temperature in equilibrator water in degrees centigade.

Temperature in equilibrator measured with a calibrated

thermistor.

13.	   SST(TSG)_C:          Temperature from the ship's thermosalinograph in degrees

centigrade.

14.	   Sal(TSG)_Permil:     Salinity from the ship's thermosalinograph on the Practical

Salinity Scale.

15.	   Water_flow_l/min:    Water flow through equilibrator in liters per minute.

16.	   Gasflow_IR_ml/min:   Gas flow through the Licor infrared analyzer before the

flow is stopped in milliliters per minute.

17.	   Temp_IR_C:           Temperature of the Licor infrared analyzer sample cell in

degrees centigrade.

18.	   Pres_IR_hPa:         Pressure in the Licor infrared analyzer in hectopascals.

NOTE:  There is no pressure sensor in the Licor but since

it is vented to atmosphere prior to measurement, this

value is the same as the pressure in the lab (number 10

above). (1 hectopascal = 1 millibar).

19.      Ship_heading_true_degree:  Ship's heading from ship's navigation system in

degrees with 0 = North and 90 = East.

20.	   Ship_speed_knot:     Ship's speed from ship's navigation system in knots.

21.	   Wind_dir_rel_degree:	Wind direction relative to the ship from ship's navigation

system in degrees with 0 = from the bow and 90 = from

starboard.

22.	   Wind_speed_rel_m/s:	Wind speed relative to the ship from ship's navigation

system in meters per second.

23.	   fCO2W@SST_uatm:      Fugacity of CO2 in sea water in microatmospheres calculated

as outlined below.

24.	   Qcflag_water:        Quality control flag for sea water xCO2 and fCO2 values

with 2 = good value, 3 = questionable value, 4 = bad value,

and 9 = no measurement taken.

25.	   fCO2a_uatm:          Fugacity of CO2 in air in microatmospheres.  This field is

not measured on the Explorer of the Seas - all data

initialized to -9.

26.	   Qcflag_air:          Quality control flag for air xCO2 and fCO2.  Since no

air values were taken, all values are initialized to 9.

27.	   dfCO2_uatm:          Sea water fCO2 - air fCO2 in microatmospheres.  This uses

the average air value for the current hour.  This field is

not measured on the Explorer of the Seas - all data

initialized to -9.

28.	   Fluoro_ug/l:         Reading from the fluorometer in micrograms per liter.

29.      Wind_speed_true_m/s:    True wind speed in meters per second.

30.	   Wind_dir_true_degree:   True wind direction in degrees were 0 = North and

90 = East.

31.	   Air_Temp_C:          Outside air temperature from ship's computer system

in degrees centigrade.

32.      Oxygen:              Units not known at this time.

The following fields have been QC'ed by the CO2 group:

Group_Ship

Cruise

JD_GMT

DATE_DDMMYYYY

TIME_HH:MM:SS

Lat_dec_degree

Long_dec_degree

xCO2eq_ppm

Pres_Equil_hPa

EqTemp_C

Water_flow_l/min

Gasflow_IR_ml/min

Temp_IR_C

Pres_IR_hPa

fCO2W@SST_uatm

QCflag_water

The following fields are from ship's onboard systems and the quality of this

data cannot be verified:

Pres_sealevel_hPa

SST(TSG)_C

Sal(TSG)_Permil

Ship_heading_true_degree

Ship_speed_knot

Wind_dir_rel_degree

Wind_speed_rel_m/s

Fluoro_ug/l

Wind_speed_true_m/s

Wind_dir_true_degree

Air_Temp_C

Oxygen

Narrative description of system design:

CO2 ANALYTICAL SYSTEM:

The concentration of carbon dioxide (CO2) in surface ocean water is determined by

measuring the concentration of CO2 in gas that is in contact with the water.

Surface water is pumped over 200 m through 7/8" Teflon tubing from an inlet

in the ship's bow to the equilibration chamber.  Water comes from the bow

intake 2 m below the water line and the TSG is located close to the inlet.

Since the sea surface temperature is high and the ship is well air-conditioned,

the Teq is on average about 0.4 °C lower than SST.  The equilibration chamber

has an enclosed volume of gas, or headspace, and a pool of seawater that

continuously overflows to a drain.  As the water flows through the chamber,

the dissolved gases (like CO2) partition between the water and the headspace.

At equilibrium, the ratio of CO2 in the water and in the headspace is

influenced most by temperature, and that relationship is known.  By measuring

the concentration of CO2 in the headspace and the temperature in the chamber,

the partial pressure (or fugacity) of CO2 in the surface water can be calculated.

INSTRUMENT DESCRIPTION

The general principle of instrumental design can be found in Wanninkhof and Thoning

(1993), Ho et al. (1995), and Feely et al. (1998).  The concentration of CO2 in the

headspace gas is measured using the adsorption of infrared (IR) radiation, which

results from changes in the rotational and vibrational energy state of the CO2

molecule.  The LI-COR detector passes IR radiation through two 6" cells.  The

reference cell is flushed with a gas of known CO2 concentration.  The sample cell

is flushed with the headspace gas.  A vacuum-sealed, heated filament is the

broadband IR source.  The IR radiation alternates between the two cells via a

chopping shutter disc. An optical filter selects an adsorption band specific

for CO2 (4.26 micron) to reach the detector.  The solid state (lead selenide)

detector is kept at -12 degrees °C for excellent stability and low signal noise

(less than 0.2 ppm).

Several steps are taken to reduce interferences and to increase the accuracy

of the measurements.  After the equilibration chamber, the headspace travels

through a drying trap to remove water vapor.  During each analysis, the

headspace gas is compared to a reference gas of known concentration.  To

improve the accuracy of the measurements, three different gaseous standards

for CO2 are analyzed once an hour instead of the headspace gas.

Analyzer:  LI-COR 6252 (analog output) infrared (IR) analyzer.

Method of Analysis:  Differential analyses relative to a reference gas which is

close to the CO2 concentration of the middle standard.  Measures dried equilibrator

headspace gas.  Gas flow is stopped prior to IR readings.

Drying Method:  The equilibrator headspace sample gas first goes through an air

filter and a thermoelectric refrigerator (~6-10 °C).  The sample and standard gases

pass through a Perma Pure (Nafion) dryer and a short column of magnesium perchlorate

before reaching the analyzer.  The counter flow in the Perma Pure tube is the

reference gas.

Equilibrator (setup, size, flows):  The equilibrator was fabricated using a filter

housing (ColeParmer, U-010509-00) with ~0.5 L water reservoir and ~0.8 L gaseous

headspace.  Water flow rate is ~1.5 L/min.  Headspace recirculation rate is

~80 ml/min.

Narrative statement identifying measurement method for each required parameter:

CALCULATIONS:

The mixing ratios of ambient air and equilibrated headspace air are calculated

by fitting a second-order polynomial through the hourly averaged millivolt

response of the detector versus mixing ratios of the standards.  Mixing ratios

of dried equilibrated headspace and air are converted to fugacity of CO2 in

surface seawater and water saturated air in order to determine the fCO2.

For ambient air and equilibrator headspace, the fCO2a (or fCO2eq) is calculated

assuming 100% water vapor content:

fCO2eq = xCO2eq(P-pH2O)exp(B11+2d12)P/RT

where fCO2eq is the fugacity in the equilibrator, pH2O is the water vapor

pressure at the sea surface temperature, P is the atmospheric pressure (in atm),

T is the SST or equilibrator temperature (in K) and R is the ideal gas constant

(82.057 cm^3·atm·deg^-1·mol^-1).  The exponential term is the fugacity correction

where B11 is the second virial coefficient of pure CO2

B11 = -1636.75 + 12.0408T - 0.032795T^2 + 3.16528E-5 T^3

and d12 = 57.7 - 0.118 T is the correction for an air-CO2 mixture in units of

cm^3·mol^-1 (Weiss, 1974).

The calculation for the fugacity at SST involves a temperature correction term

for the increase of fCO2 due to heating of the water from passing through the

pump and through 5 cm ID PVC tubing within the ship.  The water in the equilibrator

is typically 0.4 °C cooler than sea surface temperature. The empirical temperature

correction from equilibrator temperature to SST is outlined in Weiss et al. (1982).

dln(fCO2)=(teq-SST)(0.0317-2.7851E-4 teq - 1.839E-3 ln(fCO2eq))

where dln(fCO2) is the difference between the natural logarithm of the fugacity

at Teq and SST, and Teq is the equilibrator temperature in degrees C.

Sampling Cycle:

The system runs on an hourly cycle during which 3 standard gases, a reference gas

and 20 surface water samples (from the equilibrator head space) are analyzed on

the following schedule:

Mins. after hour                   Sample

================			  ======

0:00                               Low Standard

2:10                               Mid Standard

4:25                               High Standard

6:40                               Reference

8:55                               Water

11:28                               Water

14:01                               Water

16:34                               Water

19:07                               Water

21:40                               Water

24:13                               Water

26:46					  Water

29:19					  Water

31:52                               Water

34:25                               Water

36:58                               Water

39:31                               Water

42:04                               Water

44:37                               Water

47:10                               Water

49:43                               Water

52:16                               Water

54:49                               Water

57:22                               Water

NOTES ON DATA:

Columns have a default value of -9 in case of instrument malfunction, erroneous

readings or missing data.  Furthermore, if a suspicious xCO2 value, pressure or

temperature value is encountered, the fCO2 is not calculated.

Analytical Instrument Manufacturer/Model:

The Explorer of the Seas system was built by Dave Chipman in 2000.  The

analyzer is a LI-COR 6252 (analog output) infrared analyzer.

Standard Gases and Reference Gas:  The three standard gases come from CMDL

in Boulder and are directly traceable to the WMO scale.  The reference gas

is a non-calibrated gas from a commercial company.  Any value outside the

range of the standards (192 ppm, 426 ppm, and 514 ppm) should be considered

approximate (5 ppm).  While individual data points above 514 ppm or below

192 ppm may not be accurate, the general trends should be indicative of

the seawater chemistry.

Description of any additional environmental control:

The system is located in the ocean laboratory of the Explorer of the

Seas.  The room is air-conditioned with little temperature fluctuation.

Resolution of measurement:

The resolution of the instrument is better than 0.1 ppm.

Estimated overall uncertainty of measurement:

The xCO2eq measurements are believed accurate to 1 ppm.  The

fCO2@SST measurements are believed to be precise to 2 ppm.

List of calibration gases used:

The standards used on the cruise are:

STANDARD      TANK #       CONCENTRATION        VENDOR

STD1          CA4563          192.34             CMDL

STD2          CA3253          426.96             CMDL

STD3          CA3758          514.24             CMDL

Traceability to an internationally recognized scale (including date/place

of last calibration made):

All standards are obtained from NOAA/CMDL, now called the Global

Monitoring Division of the Earth Research Laboratory and are directly

traceable to WHO scale.

Uncertainty of assigned value of each calibration gas:

The uncertainty based on pre and post cruise calibrations is less

than 0.05 ppm.

Pressure/Temperature/Salinity:

Thermosalinograph information can be found at http://www.rsmas.miami.edu/

rccl/facilities.html.

Units:

All xCO2 values are reported in parts per million (ppm) and fCO2 values

are reported in microatmospheres (uatm) assuming 100% humidity at the

equilibrator temperature.

Bibliography:

DOE (1994). Handbook of methods for the analysis of the various parameters of the

carbon dioxide system in sea water; version 2. DOE.

Feely, R. A., R. Wanninkhof, H. B. Milburn, C. E. Cosca, M. Stapp and P. P. Murphy

(1998). A new automated underway system for making high precision pCO2

measurements onboard research ships. Analytica Chim. Acta 377: 185-191.

Ho, D. T., R. Wanninkhof, J. Masters, R. A. Feely and C. E. Cosca (1997).

Measurement of underway fCO2 in the Eastern Equatorial Pacific on NOAA

ships BALDRIGE and DISCOVERER, NOAA data report ERL AOML-30, 52 pp.,

NTIS Springfield.

Wanninkhof, R. and K. Thoning (1993). Measurement of fugacity of CO2 in surface

water using continuous and discrete sampling methods. Mar. Chem. 44(2-4):

189-205.

Weiss, R. F. (1970). The solubility of nitrogen, oxygen and argon in water and

seawater. Deep-Sea Research 17: 721-735.

Weiss, R. F. (1974). Carbon dioxide in water and seawater: the solubility of

a non-ideal gas. Mar. Chem. 2: 203-215.

Weiss, R. F., R. A. Jahnke and C. D. Keeling (1982). Seasonal effects of

temperature and salinity on the partial pressure of CO2 in seawater.

Nature 300: 511-513.

Comments related to the individual legs:

Explorer of the Seas Naming Convention:  EX04NNT where EX is the ship abbreviation,

04 is the year, NN is the number of the cruise which will be a number between 1

and 52, and T is the cruise track which will be either W for the west track

or E for the east track.

For all legs, due to the slow response time of the system, the first

10 minutes of data for each hour for the entire leg was removed.  This

slow response time is not fully understood.  On certain legs the response

time was worse and up to 40 minutes of data was removed.

For all cruises, the first hour of data after leaving port is removed.

EX0401W:  The system was down on 1/03/04 to 1/04/04 from 23:00 to 1:02.

Merged missing Salinity, SST, Latitude and Longitude into the datafile.

EX0405W:  Merged missing salinity, SST, latitude and longitude into the

Datafile.

EX0407W:  The water flow stopped at 12:03 on 2/18/04 to the end of the

cruise.

EX0408E:  Water meter broken.  Merged water flow data from the VIDS Lab

Feed Flow subdirectory into the data file before reducing the data.

Ex0409W: LI-COR was off from 2/28/04 (19:02) to 3/2/04 (16:02) – data

was removed.

EX0411W:  Incorrect voltages during the water phase on 3/14/2004 from

14:11 to 20:59 - data was removed.

EX0413W:  The TSG data was missing - merged data from MicroTSG into

datafile.  Replaced all zeros with -9 in the Pres_sealevel_hPa field.

Checked the BowWeatherPak directory and there was not data for JD 91.

EX0414E:  The TSG data was missing - merged data from MicroTSG into

datafile.  On 4/06/04 the MicroTSG SST was about 2 degrees higher

than the equilibrator temperature.

EX0415W:  Water flow meter problems.  Merged water flow from the VIDS Lab

Feed Flow subdirectory into the data file before reducing the data.

Voltage in the water phase was zero from 4/11/04 (22:08) to 4/12/04 (11:59) –

changed these voltages to -9.

EX0416E:  Missing salinity values on 4/17/04 from 16:02 to 22:16.

EX0419W:  Missing water flow data, merged water flow from the VIDS

Lab Feed Flow subdirectory into the data file.  Questionable data was

removed on 5/10/04 from 19:39 to 23:24.  Questionable data was removed

from on 5/10/04 to 5/11/04 from 1:36 to 10:40.

EX0423W:  Questionable data was removed on 6/6/04 to 6/7/04 from 18:02 to

13:59.  The equilibrator thermistor went bad.  EQ_TEMP was calculated using

a linear fit of EQ_TEMP – SST (EqTemp = 0.8951 * SST + 2.45) using the

following six cruises:  Ex0417W, Ex0418E, Ex0419W, Ex0420E, Ex0421W, and

Ex0422E.

EX0424E:  Due to the broken equilibrator thermistor, EQ_TEMP was calculated

using a linear fit of EQ_TEMP – SST (EqTemp = 0.8951 * SST + 2.45) using the

following six cruises:  EX0417W, EX0418E, EX0419W, EX0420E, EX0421W and EX0422E.

Problems with the gas flow, removed data from 6/19/04 (11:11) to 6/20/04 (9:08).

EX0425W:  Due to the broken equilibrator thermistor, EQ_TEMP was calculated

Using a linear fit of EQ_TEMP - SST (EqTemp = 0.8951 * SST + 2.45) using the

following six cruises: EX0417W, EX0418E, EX0419W, EX0420E, EX0421W and EX0422E.

Ex0426E:  Due to the broken equilibrator thermistor, EQ_TEMP was calculated

using a linear fit of EQ_TEMP - SST (EqTemp = 0.8951 * SST + 2.45) using the

following six cruises:  EX0417W, EX0418E, EX0419W, EX0420E, EX0421W and EX0422E.

The system was down on 7/03/04 from 23:02 to 23:59.  Replaced the zeros with –9

in the Pres_sealevel_hPa field; checked the BowWeatherPak data in the VIDs

directory and there was not barometric pressure data for this cruise.

EX0427W: Due to the broken equilibrator thermistor, EQ_TEMP was calculated

using a linear fit of EQ_TEMP – SST (EqTemp = 0.8951 * SST + 2.45) using the

following six cruises: EX0417W, EX0418E, EX0419W, EX0420E, EX0421W and EX0422E.

EX0428E: Due to the broken equilibrator thermistor, EQ_TEMP was calculated

using a linear fit of EQ_TEMP – SST (EqTemp = 0.8951 * SST + 2.45) using the

following six cruises: EX0417W, EX0418E, EX0419W, EX0420E, EX0421W and EX0422E.

EX0429W: Due to the broken equilibrator thermistor, EQ_TEMP was calculated

using a linear fit of EQ_TEMP – SST (EqTemp = 0.8951 * SST + 2.45) using the

following six cruises: EX0417W, EX0418E, EX0419W, EX0420E, EX0421W and EX0422E.

EX0430E: Due to the broken equilibrator thermistor, EQ_TEMP was calculated

using a linear fit of EQ_TEMP – SST (EqTemp = 0.8951 * SST + 2.45) using the

following six cruises: EX0417W, EX0418E, EX0419W, EX0420E, EX0421W and EX0422E.

EX0431W: Due to the broken equilibrator thermistor, EQ_TEMP was calculated

using a linear fit of EQ_TEMP – SST (EqTemp = 0.8951 * SST + 2.45) using the

following six cruises: EX0417W, EX0418E, EX0419W, EX0420E, EX0421W and EX0422E.

EX0432E: Due to the broken equilibrator thermistor, EQ_TEMP was calculated

using a linear fit of EQ_TEMP - SST (EqTemp = 0.8951 * SST + 2.45) using the

following six cruises: EX0417W, EX0418E, EX0419W, EX0420E, EX0421W and EX0422E.

EX0433W: Due to the broken equilibrator thermistor, EQ_TEMP was calculated

using a linear fit of EQ_TEMP – SST (EqTemp = 0.8951 * SST + 2.45) using the

following six cruises: EX0417W, EX0418E, EX0419W, EX0420E, EX0421W and EX0422E.

Gas flow problems from 8/21/04 to 8/22/04 – questionable data removed.

EX0434E: Due to the broken equilibrator thermistor, EQ_TEMP was calculated

using a linear fit of EQ_TEMP – SST (EqTemp = 0.8951 * SST + 2.45) using the

following six cruises: EX0417W, EX0418E, EX0419W, EX0420E, EX0421W and EX0422E.

EX0435W: Due to the broken equilibrator thermistor, EQ_TEMP was calculated

using a linear fit of EQ_TEMP – SST (EqTemp = 0.8951 * SST + 2.45) using the

following six cruises: EX0417W, EX0418E, EX0419W, EX0420E, EX0421W and EX0422E.

EX0436E:  The broken equilibrator thermistor was replaced.

EX0438E:  Questionable data removed from 9/24/04 (22:24) to 9/27/04

(7:52).  No water flow data - merged water flow from the VIDS Lab Feed Flow

subdirectory into the data file.  There is also low flow from the shower head.

EX0440E:  Air Temperature not available for this cruise.  There was a plumbing

problem during the beginning of the cruise.  When the system came back on line,

there was no flow coming from the shower head; the 3-way valve for the seawater

connection was clogged.  Once the valve was cleaned and reconnected, flow

returned from the shower head and data acquisition resumed.

EX0441W:  Due to a system leak, the following data was removed:  10/11/04 from

19:47 to 22:16, 10/15/04 from 14:47 to 16:11, and 10/15/04 from 19:49 to 22:59.

EX0442E:  Due to a leak in the system, the following data was removed:

10/20/04 from 1:54 to 5:16 and 10/21/04 from 22:11 to 23:59.

EX0443W:  Problems with the barometric pressure sensor.  Replaced all zeros

and bad data (very low values) with -9 in Pres_sealevel_hPa.

EX0444E:  Due to a leak in the system, the following data was removed:

11/03/04 from 1:26 to 4:21 and 11/04/04 to 11/05/04 from 22:11 to 00:31.

EX0446E:  Due to a water flow problem, the following data was removed:

11/17/04 from 4:24 to 7:36 and 11/18/04 to 11/19/04 from 23:36 to 1:36.

EX0447W:  Low gas flow during the water phase; Chip readjusted the gas flow.

EX0448E:  Gas flow problems in the water phase – removed the data from

11/29/04 (4:21( to 12/1/04 (23:59).

EX0449W:  Gas flow problems in the water phase - removed the data from

12/09/04 (0:02) to 12/11/04 (1:59).

EX0450E:  Air temperature data was not available for this cruise.

Leak in system, removed data from 12/13/04 (0:11) to 12/14/04 (16:59) -

there was an air leak coming out on the side of the fridge where the

tubing is connected.  Don stopped data collection and pushed the tubing

further up the port and anchored it with two tie-wraps.